3GPP TSG-RAN4 Meeting #115	R4-2506726

# Malta, MT, 19th – 23rd May, 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.133 | CR | 5567 | rev | - | Current version: | 18.9.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR on TC maintenance for R18 MIMO uTCI R18 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_MIMO_evo_DL_UL-Perf |  |  |  |  |  | Date: |  |  | 2025-04-21 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | 1. In TC A.6.5.13, based on current SSB configuration, SSB for TRP0 is SSB#0, SSB for TRP1 should be SSB#2 and SSB#3 instead of SSB#1 and SSB2. Thus, the DLorJoint TCI States table Table A.3.16A.2-2 should be corrected accordingly. 2. In TC A.6.5.13, the SSB index should be updated for the same reason as mentioned above. 3. In TC A.6.5.13, the TCI state 0 and TCI state 1 is configured as DLorJoint TCI.State.6 and DLorJoint TCI.State.7, which are wrong. As there is no TRS resource set 3 and set 4 for FR1. It should be corrected to DLorJoint TCI.State.2 and DLorJoint TCI.State.3 4. In A.7.5.13.4, there is only one Cell, thus the timing offset between Cell 2 and Cell 1 is not correct. 5. In A.7.5.13.4, there is only two SSB index configured, but four SSB index are needed for the TC. 6. DLorJoint TCI.State.8 is for TRS resource set 5, but there is no set 5 for FR2. |  |  |  |  |  |  |  |  |
| Summary of change: |  | 1. In TC A.6.5.13, correct the SSB configuration for TRP1 2. corret the QCL source to SSB2 and SSB3 in Table A.3.16A.2-2: DLorJoint TCI States for mTRP FR1 3. In TC A.6.5.13, corect the SSB index in test purpose and environment. 4. In TC A.6.5.13, correc TCI states to DLorJoint TCI.State.2 and DLorJoint TCI.State.3 5. In A.7.5.13.4, correct the timing offset as the offset between TRPs 6. In A.7.5.13.4, configure SSB for TRP0 and TRP1 seperately. 7. Add a new CRI-RS configuration Set 5 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The test can not be performed |  |  |  |  |  |  |  |  |
| Clauses affected: |  | A.3.16A, A.3.17, A.6.5.13 and A.7.5.13.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS 38.533 |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

### <Start of Change 1>

Table A.3.16A.2-2: DLorJoint TCI States for mTRP FR1

| Parameter | DLorJoint TCI.State.1.1 | DLorJoint TCI.State.1.2 | DLorJoint TCI.State.1.3 | DLorJoint TCI.State.1.4 |
| --- | --- | --- | --- | --- |
| tci-StateUnifiedId | Id1 | Id2 | Id3 | Id4 |
| qcl-Type1 | typeA | typeA | typeC | typeC |
| qcl-Type2 | N/A | N/A | N/A | N/A |
| referenceSignal Note2 | Resource #4 in TRS resource set 1 Note2 | Resource #4 in TRS resource set 2 Note3 | SSB2 | SSB3 |
| pathlossReferenceRS | N/A | N/A | N/A | N/A |
| additionalPCI | N/A | N/A | N/A | N/A |
| NOTE 1: referenceSignal configurations towards which the TCI states are configured are defined in a test-specific manner. NOTE 2: Reference TRS resource sets are defined in clause A.3.17, and the applicable TRS resource set(s) are specified in each test case. When a single TRS resource set is configured in a test case, it is considered as resource set 1. The TCI state of the TRS is the DLorJoint TCI.State.1.1. NOTE 3: Reference TRS resource sets are defined in clause A.3.17, and the applicable TRS resource set(s) are specified in each test case. When a single TRS resource set is configured in a test case, it is considered as resource set 1. The TCI state of the TRS is the DLorJoint TCI.State.1.2. |  |  |  |  |

### <End of Change 1>

### <Start of Change 2>

Table A.3.17.2.1-6: CSI-RS for tracking for SCS=120 kHz Set 5

| Parameter | Unit | Value |
| --- | --- | --- |
| Reference channel |  | TRS.2.5 TDD |
| Bandwidth |  | BW of Active BWPNote 1,3 |
| SCS | kHz | 120 |
| First subcarrier index in the PRB used for CSI-RS |  | k0=0 for CSI-RS resource 1,2,3,4 |
| First OFDM symbol in the slot used for CSI-RS |  | l0 = 4 for CSI-RS resource 1 and 3 l0 = 8 for CSI-RS resource 2 and 4 |
| Number of CSI-RS ports (X) |  | 1 for CSI-RS resource 1,2,3,4 |
| CDM Type |  | ‘No CDM’ for CSI-RS resource 1,2,3,4 |
| Density (ρ) |  | 3 for CSI-RS resource 1,2,3,4 |
| CSI-RS periodicity | slots | 80 for CSI-RS resource 1,2,3,4 |
| CSI-RS offset | slots | 40 for CSI-RS resource 1 and 2 41 for CSI-RS resource 3 and 4 |
| EPRE ratio to SSS | dB | 0Note 2 |
| TCI state |  | TCI.State.3 |
| NOTE 1: BW of TRS is configured same as the BW size of UE active BWP in the RRM test cases. NOTE 2: Unless otherwise specified in the test case. NOTE 3: If active BWP is larger than 52RBs, BW of TRS is configured as 52RBs. Otherwise, same as active BWP size. NOTE 4: If unified TCI states are configured, the TCI state ID is defined in each test case and from A.3.16A Unified TCI state Configuration. |  |  |

### <End of Change 2>

### <Start of Change 3>

### A.6.5.13	Active TCI state switch delay

#### A.6.5.13.1	MAC-CE based joint TCI state switch for mDCI with two TA when RTD is larger than CP

##### A.6.5.13.1.1	Test Purpose and Environment

The purpose of this test is to verify the active TCI state switch delay requirement defined in clause 8.22.3. Supported test configuration is shown in table A.6.5.13.1.1-1.

The test scenario comprises of one NR PCell (Cell 1) containing two TRPs (i.e., TRP 0 and TRP 1) belonging to two TAGs as given in table A.6.5.13.1.1-2. Cell-specific parameters of NR PCell are specified in table A.6.5.13.1.1-3 below.

PDCCHs associated with corsetPoolIndex 0 and 1 indicating new transmissions shall be sent continuously on PCell to ensure that the UE would have ACK/NACK sending on PUCCH associated with TRP 0.

Before the test starts,

-	UE is connected to Cell 1 (PCell) on radio channel 1 (PCC).

-	UE is configured with 1 TCI state associated with coresetPoolIndex 0 for PCell, PDCCH TCI state 0 (QCL’d to SSB0), in Cell 1 before starting the test. UE is configured with 2 TCI states associated with coresetPoolIndex 1 for PCell, PDCCH TCI state 0 (QCL’d to SSB2) and TCIstate 1 (QCL’d to SSB3).

-	UE is indicated in TCI state 0 as the active PDCCH TCI state for PDCCH associated with coresetPoolIndex 0, and UE is indicated in TCI state 0 as the active PDCCH TCI state for PDCCH associated with coresetPoolIndex 1.

-	Target TCI state is not in the active TCI state list.

The test consists of two time periods, T1 and T2. During T1 only SSB0 and SSB2 are transmitted. At the beginning of T2, the SSB3 corresponding to TCI state 1 starts transmitting. The UE is configured to provide periodic L1-RSRP reports. In slot n which is within 1280 ms of UE providing L1-RSRP report with results for both SSB2and SSB3, UE receives a MAC-CE command indicating a switch to TCI state 1 for PDCCH associated with coresetPoolIndex 1. tci-PresentInDCI is not configured in the PDSCH configuration, i.e. TCI state for the PDSCH is identical to the PDCCH TCI state.

The test equipment verifies that UE can be scheduled on PCell on TCI state 0 associated with coresetPoolIndex 1  till n+ THARQ +3 ms. The test equipment also verifies the TCI state switch time in PCell by scheduling the UE on TCI state 1 associated with coresetPoolIndex 1  after n+ THARQ +3 ms + (Tfirst-SSB + TSSB-proc).

Table A.6.5.13.1.1-1: Supported test configurations

| Configuration | Description |
| --- | --- |
| 1 | FDD duplex mode, 15 kHz SSB SCS, 10 MHz bandwidth |
| 2 | TDD duplex mode, 15 kHz SSB SCS, 10 MHz bandwidth |
| 3 | TDD duplex mode, 30 kHz SSB SCS, 40 MHz bandwidth |
| NOTE: The UE is only required to pass in one of the supported test configurations in FR1 |  |

Table A.6.5.13.1.1-2: General test parameters for TCI state switch

| Parameter | Unit | Value | Comment |
| --- | --- | --- | --- |
| NR RF Channel Number |  | 1 | One NR radio channel is used for this test |
| Active PCell |  | Cell 1 | PCell on RF channel number 1. |
| CP length |  | Normal |  |
| Timing offset between SSB 0 and SSB1 | s | 25 |  |
| Timing offset between SSB0 and SSB2 | s | 33 |  |
| DRX |  | OFF |  |
| T1 | s | 0.2 |  |
| T2 | s | 0.2 |  |

Table A.6.5.13.1.1-3: NR Cell specific test parameters for TCI state switch

| Parameter |  | Unit | Cell 1 |
| --- | --- | --- | --- |
| Frequency Range |  |  | FR1 |
| Duplex mode | Config 1 |  | FDD |
|  | Config 2,3 |  | TDD |
| TDD configuration | Config 1 |  | Not Applicable |
|  | Config 2 |  | TDDConf.1.1 |
|  | Config 3 |  | TDDConf.1.2 |
| BWchannel | Config 1,2 |  | 10: NPRB,c = 52 |
|  | Config 3 |  | 40: NPRB,c = 106 |
| DL initial BWP configuration | Config 1, 2, 3 |  | DLBWP.0.1 |
| DL dedicated BWP configuration | Config 1, 2, 3 |  | DLBWP.1.1 |
| UL initial BWP configuration | Config 1, 2, 3 |  | ULBWP.0.1 |
| UL dedicated BWP configuration | Config 1, 2, 3 |  | ULBWP.1.1 |
| PDSCH Reference | Config 1 |  | SR.1.1 FDD |
| measurement channel Note 4 | Config 2 |  | SR.1.1 TDD |
|  | Config 3 |  | SR.2.1 TDD |
| RMSI CORESET | Config 1 |  | CR.1.1 FDD |
| parameters | Config 2 |  | CR.1.1 TDD |
|  | Config 3 |  | CR.2.1 TDD |
| Dedicated CORESET | Config 1 |  | CCR.1.2 FDD |
| parameters | Config 2 |  | CCR.1.2 TDD |
|  | Config 3 |  | CCR.2.4 TDD |
| TRS Configuration for TRP 0 | Config 1 |  | TRS.1.1 FDD |
|  | Config 2 |  | TRS.1.1 TDD |
|  | Config 3 |  | TRS.1.2 TDD |
| TRS Configuration for TRP 1 Set 1 | Config 1 |  | TRS.1.5 FDD |
|  | Config 2 |  | TRS.1.5 TDD |
|  | Config 3 |  | TRS.1.7 TDD |
| TRS Configuration for TRP 1 Set 2 | Config 1 |  | TRS.1.6 FDD |
|  | Config 2 |  | TRS.1.6 TDD |
|  | Config 3 |  | TRS.1.8 TDD |
| OCNG Patterns | Config 1,2 |  | OP.1 |
|  | Config 3 |  | OP.1 |
| SSB Configuration for TRP 0 | Config 1,2 |  | SSB.1 FR1 |
|  | Config 3 |  | SSB.2 FR1 |
| SSB Configuration for TRP 1 | Config 1,2 |  | SSB.7 FR1 |
|  | Config 3 |  | SSB.8 FR1 |
| TCI state 0 associated with coresetPoolIndex 0 |  |  | DLorJoint TCI.State.0 |
| TCI state 0 associated with coresetPoolIndex 1 |  |  | DLorJoint TCI.State.2 |
| TCI state 1 associated with coresetPoolIndex 1 |  |  | DLorJoint TCI.State.3 |
| Correlation Matrix and Antenna Configuration |  |  | 1x2 Low |
| EPRE ratio of PSS to SSS |  | dB | 0 |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |
| EPRE ratio of PDSCH to PDSCH DMRS |  |  |  |
| EPRE ratio of OCNG DMRS to SSS(Note 1) |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS (Note 1) |  |  |  |
| NocNote 2 | Config 1,2 | dBm/SCS | -104 |
|  | Config 3 |  | -101 |
| NocNote 2 |  | dBm/15kHz | -104 |
| SS-RSRP Note 3 | Config 1,2 | dBm/SCS | -87 |
|  | Config 3 |  | -84 |
| Ês/Iot |  | dB | 17 |
| Ês/Noc |  | dB | 17 |
| IoNote3 | Config 1,2 | dBm/ 9.36 MHz | -58.96 |
|  | Config 3 | dBm/ 38.16 MHz | -52.86 |
| Propagation Condition |  |  | AWGN |
| NOTE 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. NOTE 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for Noc to be fulfilled within BWoccupied. NOTE 3: SS-RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. NOTE 4: PDSCH RMC for TRP0 and TRP1 are scheduled in non-overlapping PRBs |  |  |  |

##### A.6.5.13.1.2	Test Requirements

During T2, UE shall send L1-RSRP report with results for both SSB0 and SSB1.

After receiving MAC-CE command in slot n, UE shall be able to continue receive on TRP 0, and for TRP 1, UE shall:

-	be able to continue to receive on TCI state 0 till   n+ THARQ +3 ms

-	be able to start receiving on TCI state 1 after n+ THARQ +5 ms + Tfirst-SSB

The rate of correct events observed during repeated tests shall be at least 90 %.

### <End of change 3>

### <Start of Change 4>

#### A.7.5.13.4	sDCI MAC-CE based joint TCI state switching

##### A.7.5.13.4.1	NR PCell FR2 dual downlink and uplink TCI state switch in sDCI for known case

###### A.7.5.13.4.1.1	Test Purpose and Environment

The purpose of this test is to verify both active downlink and uplink TCI state switch delay requirement defined in clause 8.21 and 8.23, respectively, by using joint TCI state of unified TCI state switch framework when tci-JointTCI-UpdateSingleActiveTCI-PerCC-r18 is supported and when tci-SeparateTCI-UpdateSingleActiveTCI-PerCC-r18 is not supported. Supported test configuration is shown in table A.7.5.13.4.1.1-1.

Table A.7.5.13.4.1.1-1: Supported test configurations

| Config | Description |
| --- | --- |
| 1 | NR 120 kHz SSB SCS, 100 MHz bandwidth, TDD duplex mode |

Table A.7.5.13.4.1.1-2: General test parameters for dual TCI state switch

| Parameter | Unit | Value | Comment |
| --- | --- | --- | --- |
| NR RF Channel Number |  | 1 | One NR radio channel is used for this test |
| Active PCell |  | Cell 1 | PCell on RF channel number 1. |
| CP length |  | Normal |  |
| DRX |  | OFF |  |
| L1-RSRP reporting period | slot | 320 | Periodic L1-RSRP reporting configured |
| L1-RSRP measured RS |  | SSB0 and SSB2 of TRP0, SSB1 and SSB3 of TRP1 | L1-RSRP measurements of SSB0, SSB1 SSB2 and SSB3. |
| Number of RS for L1-RSRP reporting |  | 4 | SSB0, SSB1, SSB2 and SSB3 in Joint TCI state 0, 1, 2, and 3. |
| TRP1 timing offset to TRP0 | us | <CP |  |
| T1 | s | 0.2 |  |
| T2 | s | 1 | UE is required to activate dual joint TCI states (TCI state 2 and TCI state 3). |

###### A.7.5.13.4.1.2	Test parameters

The test scenario comprises of one NR PCell (Cell 1) as given in table A.7.5.13.4.1.2-1. Cell-specific parameters of NR PCell are specified in table A.7.5.13.4.1.2-2 below. The OTA related test parameters for FR2 are shown in table A.7.5.13.4.1.2-3.

Before the test starts,

-	UE is connected to Cell 1 (PCell) on radio channel 1 (PCC).

-	PDCCHs indicating new transmissions shall be sent continuously on Pcell to ensure that the UE would have ACK/NACK sending.

-	UE is provided with dl-OrJoint-TCIStateList-r17 and UE’s higher layer signalling unifiedTCI-StateType-r17 in IE MIMOParam-r17 is set to joint.

-	applyIndicatedTCI-State-r18 is set as {first} for TRP0 (CORESET index p associated with Joint TCI state 0 and Joint TCI state 2) and as {second} for TRP1 (CORESET index q associated with Joint TCI state 1 and Joint TCI state 3).

-	tci-SelectionPresentInDCI-r18 is configured in the BWP configuration, i.e. TCI state for the PDSCH is indicated by DCI format 1_1 and PDSCHs on two TRPs are scheduled in TDM manner.

-	UE is configured with two joint TCI states (TCI state 0 and TCI state 2) for TRP0 and two joint TCI states (TCI state 1 and TCI state 3) for TRP1. QCL info to Joint TCI state 0,1,2 and 3 are provided by SSB0, SSB1, SSB2 and SSB3, respectively.

-	UE is indicated in TCI state 0 and TCI state 1 as the active TCI state for TRP0 and TRP1.

The test consists of two time periods, T1 and T2. During T1, SSB0 in joint TCI state 0 and SSB1 in joint TCI state 1 are transmitted.

At the beginning of T2, SSB2 in joint TCI state 2 and SSB3 in joint TCI state 3 start transmitting. The UE is configured to provide periodic L1-RSRP reports. In slot n which is within 1280 ms after the slot in which UE provides L1-RSRP report with results for SSB2 and SSB3 in joint TCI state 2 and joint TCI state 3, UE receives a MAC-CE command indicating a switch to dual joint TCI state 2 and TCI state 3 for two TRPs.

The test equipment verifies that UE can be scheduled by two TRPs on joint TCI state 0 and joint TCI state 1 till slot n+ THARQ +3Nslotsubframe,µ. The test equipment also verifies the TCI state switch time for two TRPs by scheduling the UE on joint TCI state 2 and joint TCI state 3 after slot n+THARQ + 3Nslotsubframe,µ + max{NM1* (Tfirst_target-PL-RS1 + 4*Ttarget_PL-RS1 + 2 ms), NM2* (Tfirst_target-PL-RS2 + 4*Ttarget_PL-RS 2+ 2 ms) }/ NR slot length.

Table A.7.5.13.4.1.2-1: General test parameters for dual TCI state switch

| Parameter | Unit | Value | Comment |
| --- | --- | --- | --- |
| NR RF Channel Number |  | 1 | One NR radio channel is used for this test |
| Active PCell |  | Cell 1 | PCell on RF channel number 1. |
| CP length |  | Normal |  |
| DRX |  | OFF |  |
| L1-RSRP reporting period | slot | 320 | Periodic L1-RSRP reporting configured |
| L1-RSRP measured RS |  | SSB0 and SSB2 of TRP0, SSB1 and SSB3 of TRP1 | L1-RSRP measurements of SSB0, SSB1 SSB2 and SSB3. |
| Number of RS for L1-RSRP reporting |  | 4 | SSB0, SSB1, SSB2 and SSB3 in Joint TCI state 0, 1, 2, and 3. |
| TRP1 timing offset to TRP0 | us | <CP |  |
| T1 | s | 0.2 |  |
| T2 | s | 1 | UE is required to activate dual joint TCI states (TCI state 2 and TCI state 3). |

Table A.7.5.13.4.1.2-2: NR Cell specific test parameters for dual TCI state switch

| Parameter | Unit | Cell 1 |
| --- | --- | --- |
| Frequency Range |  | FR2 |
| Duplex mode |  | TDD |
| TDD configuration |  | TDDConf.3.1 |
| BWchannel |  | 100 MHz: NPRB,c = 66 |
| Data PRBs allocated |  | 66 |
| Initial BWP Configuration |  | DLBWP.0.2 ULBWP.0.2 |
| Dedicated BWP Configuration |  | DLBWP.1.1 ULBWP.1.1 |
| PDSCH Reference measurement channel |  | SR.3. 2 TDD |
| RMSI CORESET parameters |  | CR.3.1 TDD |
| Dedicated CORESET parameters (CORESET index p) |  | CCR.3.1 TDD |
| Dedicated CORESET parameters (CORESET index q) |  | CCR.3.2 TDD |
| OCNG Patterns |  | OP.5 defined in A.3.2.1 |
| SSB Configuration for TRP 0 |  | SSB.1 FR2 |
| SSB Configuration for TRP 1 |  | SSB.2 FR2 |
| SMTC Configuration |  | SMTC.1 |
| Joint TCI State 0 |  | DLorJoint TCI.State.2 |
| Joint TCI State 1 |  | DLorJoint TCI.State.6 |
| Joint TCI State 2 |  | DLorJoint TCI.State.7 |
| Joint TCI State 3 |  | DLorJoint TCI.State.8 |
| TRS Configuration |  | TRS.2.1 TDD for DLorJoint TCI.State.2 TRS.2.4 TDD for DLorJoint TCI.State.6 TRS.2.5 TDD for DLorJoint TCI.State.7 TRS.2.6 TDD for DLorJoint TCI.State.8 |
| Correlation Matrix and Antenna Configuration |  | 1x2 Low |
| EPRE ratio of PSS to SSS | dB | 0 |
| EPRE ratio of PBCH DMRS to SSS |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |
| EPRE ratio of PDSCH to PDSCH |  |  |
| EPRE ratio of OCNG DMRS to SSS(Note 1) |  |  |
| EPRE ratio of OCNG to OCNG DMRS (Note 1) |  |  |
| Propagation Condition |  | AWGN |
| NOTE 1: OCNG shall be used such that a constant total transmitted power spectral density is achieved for all OFDM symbols. |  |  |

Table A.7.5.13.4.1.2-3: OTA related test parameters for dual TCI state switch

| Parameter | Units | TRP0 |  |  |  | TRP1 |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | SSB0 |  | SSB2 |  | SSB1 |  | SSB3 |  |
|  |  | T1 | T2 | T1 | T2 | T1 | T2 | T1 | T2 |
| Angle of arrival configuration |  | Setup [TBD] according to clause A.3.XX |  |  |  |  |  |  |  |
|  |  | AoA1 |  | AoA2 |  | AoA2 |  | AoA3 |  |
| Assumption for UE beams Note 6 |  | Rough |  |  |  |  |  |  |  |
| Ês | dBm/SCS | -80.6 | -80.6 | -Infinity | -80.6 | -80.6 | -80.6 | -Infinity | -80.6 |
| SS B_RP Note 2 | dBm/ SCS | -80.6 | -80.6 | -Infinity | -80.6 | -80.6 | -80.6 | -Infinity | -80.6 |
| BB Note 7 | dB | 8.3 | 8.3 | -Infinity | 8.3 | 8.3 | 8.3 | -Infinity | 8.3 |
| IoNote2 | dBm/95.04 MHz Note4 | -56.0 | -56.0 | - Infinity | -56.0 | -56.0 | -56.0 | - Infinity | -56.0 |
| NOTE 1: SS B_RP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. NOTE 2: Equivalent power received by an antenna with 0 dBi gain at the centre of the quiet zone NOTE 3: As observed with 0 dBi gain antenna at the center of the quiet zone. NOTE 4: Information about types of UE beam is given in clause B.2.1.3 and does not limit UE implementation or test system implementation. NOTE 5: Calculation of Es/IotBB includes the effect of UE internal noise up to the value assumed for the associated Refsens requirement in clause 7.3.2 of TS 38.101-2 [19], and an allowance of 1 dB for UE multi-band relaxation factor ΔMBP from TS 38.101-2 [19] Table 6.2.1.3-4. |  |  |  |  |  |  |  |  |  |

###### A.7.5.13.4.1.3	Test Requirements

During T2, the test verifies that UE can be scheduled by two TRPs on joint TCI state 2 and joint TCI state 3.

After receiving MAC-CE command in slot n, UE shall:

-	be able to continue to receive and transmit with joint TCI state 0 and joint TCI state 1 till slot n+ THARQ +3Nslotsubframe,µ

-	be able to start receiving and transmitting with joint TCI state 2 and joint TCI state 3 after slot n+THARQ + 3Nslotsubframe,µ + max{NM1* (Tfirst_target-PL-RS1 + 4*Ttarget_PL-RS1 + 2 ms), NM2* (Tfirst_target-PL-RS2 + 4*Ttarget_PL-RS 2+ 2 ms) }/ NR slot length.

-	where NM1=1, NM2=1.

The rate of correct events observed during repeated tests shall be at least 90 %.

### <End of change 4>