3GPP TSG-RAN WG4 Meeting # 115	R4-2506731

Malta, MT, 19th – 23rd May, 2025

Title: 	Discussion on CSI-RS based L1 measurement

Source: 	Huawei, HiSilicon

Agenda item:	7.29.3.2

Document for:	Discussion

Introduction

The revised R19 mobility WI was agreed in [1]. The objectives are duplicated as below.

### 4.1 Objective of SI or Core part WI or Testing part WI - Specify support for inter-CU Layer1/Layer 2 Triggered Mobility (LTM) [RAN2, RAN3] - Prioritize the case when CU is acting as MN when DC is not configured - When DC is configured, inter-CU LTM can be configured either in MN or in SN but not both at the same time. For such cases: - As secondary priority, support the case where CU is acting as SN and MN is unchanged - As secondary priority, support the case where CU is acting as MN and SN is unchanged or SN is released - Specify support for subsequent LTM mobility procedures aiming to avoid RRC configuration between cell switches as per Rel-18 LTM - Coordination with SA3 needed with respect to security key handling - Note: Rel. 18 intra-CU LTM procedure is considered as baseline for adding inter-CU support - Measurements related enhancements for purpose of supporting LTM: [RAN2, RAN1] - Measurement related enhancements are applicable to Intra-CU MCG/SCG LTM and Inter-CU MCG/SCG LTM - Specify necessary components to support event triggered L1 measurement reporting [RAN2, RAN1] - Specify support for CSI-RS measurements for LTM procedures and enable CSI-RS based beam management [RAN1] - Specify CSI acquisition on candidate cell(s) based on CSI-RS before or during LTM cell switch [RAN1] NOTE: RAN1 WG to decide on whether to support CSI report before or during the LTM cell switch, as part of the CSI acquisition procedure.

This contribution provides analysis on CSI-RS based L1 measurement on neighbor cells.

Discussion

Measurement procedure

As per RAN4#114bis agreement, the baseline of CSI-RS based L1 measurement on neighbour cell is 3-step.

- Step 1 (gNB TX wide SSB beam, UE RX wide SSB beam): UE performs SSB based L3 measurement and acquires the SSB index information. UE reports the L3 SSB measurement results with SSB index to network.

- Step 2 (gNB TX wide SSB beam, UE RX fine SSB beam): UE performs SSB based L1-RSRP measurement on the candidate cell to refine the SSB based RX beam. Step 2 is what we specified for R18 LTM where UE sweeps RX beam based on SSB.

- Step 3 (gNB TX narrow CSI-RS beam, UE RX fine SSB beam): UE performs measurements on the configured CSI-RS resources of candidate cell where each CSI-RS resource is QCL-typeD with SSB for L1-RSRP measurement in step 2. Herein UE performs L1 CSI-RS based measurement without RX beam sweeping. Step 3 is similar as R15 CSI-RS based L1-RSRP measurement, the difference is step3 is on candidate cell and R15 CSI-RS based L1-RSRP measurement is on serving cell.

As a compromise, one optional UE capability is agreed to introduced if UE is able to skip step 2 for FR2. However whether step 2 can be skipped for serving cell is still under discussion.

- Issue 4-2-1: Whether to keep or remove SSB based L1 measurement for FR2-1 in step 2 Agreement: - Whether step 2 can be skipped for neighboring cell or not based on new UE capability reporting. - Further discuss whether step 2 can be skipped for serving cell or not.

In our understanding if step 2 is removed, the CSI-RS based L1 measurement procedure is: CSI-RS resources within a resource set are transmitted with different downlink spatial domain transmission filter, and UE sweeps Rx beam with finer CSI-RS level beam. The schematic diagram is shown in figure 1. The measurement procedure on neighbor cell is quite different with legacy CSI-RS L1 measurement in serving cell. Then from UE implementation perspective, UE who reports the capability shall support two schemes for CSI-RS L1 measurements. Furthermore when we define CSI-RS L1 measurement requirements for step-2 skipped scheme, if the CSI-RS resources in serving cell and neighbour cell are colliding in time domain, sharing factor shall be introduced.

Proposal 1: For step-2 skipped CSI-RS L1 measurement requirement, if the CSI-RS resources in serving cell and neighbour cell are colliding in time domain, sharing factor shall be introduced.

Figure 1. CSI-RS L1 measurement on neighbour cell with skipping step2

In addition, a special case needs to be considered. As we know, a SCell can also be configured as a candidate cell in R18. So there would be two different CSI-RS measurement procedures for the same cell: one is 2-step based candidate cell measurement procedure, the other is serving cell L1 measurement procedure. From UE implementation perspective, UE can perform two measurement procedures as per different measurement configuration (LTM configuration and serving cell configuration), however the operation is complicated, duplicated and weird from UE side. Especially for the same cell, the measurement results may be various, as there is different RX gain in different measurement scheme. So we slightly prefer that serving cell still follows the legacy way (i.e., 3-step based procedure) if the serving cell is configured as a candidate cell.

Proposal 2: If a SCell cell is configured as a candidate cell, we prefer the serving cell follows the legacy way (i.e., step 2 is not skipped).

Intra-frequency CSI-RS based L1 RSRP measurement definition

Candidate options: - Option 1: (Apple, ZTE, MTK, CMCC, Huawei, Ericsson) A measurement is defined as a CSI-RS based intra-frequency measurement provided that: - the SCS of the CSI-RS resource of the neighbor cell configured for L1 measurement is the same as the SCS of active DL BWP, and - the CP type of the CSI-RS resource of neighbor cell configured for L1 measurement is the same as the CP type of active DL BWP, and - It is applied for SCS = 60KHz - the center frequency of the CSI-RS resource of the neighbour cell configured for L1 measurement is the same as the centre frequency of any of the CSI-RS resources within DL BWP (if configured). - Option 2: (Qualcomm, MTK, Nokia, E///, vivo, China Telecom, HW, ZTE, CATT) A measurement is defined as a CSI-RS based intra-frequency L1 measurement provided that: - the SCS of the CSI-RS resource of the neighbour cell configured for L1 measurement is the same as the SCS of active DL BWP, and - the CP type of the CSI-RS resource of neighbour cell configured for L1 measurement is the same as the CP type of active DL BWP, and - the CSI-RS resource of the neighbour cell configured for L1 measurement is included within the active DL BWP. - at least 48 RBs of the CSI-RS resource, needs to be confined within the active DL BWP.

We understand the motivation of both options. Option 1 follows the legacy way of intra-frequency/inter-frequency measurement definition. This kind of definition keeps the consistency of the definition L3 SSB measurement, L3 CSI-RS measurement and L1 SSB based measurement. This can facilitate the readability and comprehensibility of the specification.

Option 2 is a different way of the legacy intra-frequency/inter-frequency measurement definition. It cares about whether measurement gap is needed. The CSI-RS resources within active BWP are called as intra-frequency and measured without gap, even for CSI-RS resource in neighbour cell which has different center frequency as that of any resources in the serving cells.

It is agreed in RAN4#114bis meeting that not to define requirements for both L1 measurement with gap and for RTD>CP for CSI-RS based L1 measurement on neighbour cell. RAN4 also agreed that no need to introduce definition of frequency layer. In this way, either option of intra-frequency CSI-RS based L1 RSRP measurement definition have no big difference. The concept of intra-frequency is just a terminology. By comparison, option 2 is more straight forward. Moreover the definition of “inside/outside active BWP” also have reached consensus.

- Issue 4-6: measurement GAP and RTD assumption Agreement: - From RAN4 perspective, - Not to define RAN4 requirements for L1 measurement with gap for CSI-RS based L1 measurement on neighbor cell. - Not to define RAN4 requirements for RTD > CP.

- Issue 4-3: Definition of “inside/outside active BWP” Agreement: - RRM requirements (delay and accuracy) apply under the following condition: - At least 48 RBs of the CSI-RS configured for measurement is within the active BWP.

Proposal 3: A measurement is defined as a CSI-RS based intra-frequency L1 measurement provided that:

- the SCS of the CSI-RS resource of the neighbour cell configured for L1 measurement is the same as the SCS of active DL BWP, and

- the CP type of the CSI-RS resource of neighbour cell configured for L1 measurement is the same as the CP type of active DL BWP, and

- the CSI-RS resource of the neighbour cell configured for L1 measurement is included within the active DL BWP.

- at least 48 RBs of the CSI-RS resource, needs to be confined within the active DL BWP.

Restriction on CSI-RS resource configuration

In RAN4#114 meeting, it is agreed that at most two separate windows for CSI-RS resources.

- In FR1, when CSI-RS based L1 measurement does not cause scheduling restriction, no need to consider restriction on measurement timing configuration. - In FR2, or in FR1 when CSI-RS based L1 measurement would cause scheduling restriction: - All CSI-RS resources on [one intra-frequency layer] are configured within up to two separate windows where each window is up to [5 ms]

In RAN4#114 bis meeting, one further proposal is provided. One restriction of “within a 40ms window” is added on top of the previous agreement.

- Issue 4-8: Restriction on CSI-RS resource configuration Candidate options: - Option 1: Ericsson - In FR2, or in FR1 when CSI-RS based L1 measurement would cause scheduling restriction, all CSI-RS resources within a 40 ms window on one intra-frequency layer should be configured within up to two separate windows, each lasting up to 5 ms.

As we know, the periodicity of CSI-RS resources for L1-RSRP measurement on candidate cells can be up to 640ms. Adding the condition of “40ms” seems meaningless for large CSI-RS resource periodicity, as there are no resources within a 40ms window.

Early CSI acquisition on candidate cell

Early CSI acquisition is supported in RAN1. The baseline capability is that CSI reporting is performed after LTM CSC MAC CE. At this moment, the connection of serving cell is broken and the SW processing unit/resource for serving cell is released. The CSI acquisition of target cell can be performed during the LTM cell switching procedure. From RRM requirements perspective, we never specify RRM requirements for CSI related procedure.

The optional capability is that CSI measurement can start before reception of LTM CSC MAC CE. To some extent, the behaviour of CSI acquisition on neighbour cell is alike that of CSI-RS based L1-RSRP measurement on candidate cell. However, there are also some differences, e.g., CSI measurement can be semi-persistent and triggered by MAC CE, whether or how to select a subset of CSI-RS resources to measure. More input for early CSI acquisition on candidate cell from RAN1 is needed. Moreover considering limited time slots for completing the core requirement of the WI, we suggest not to define requirements for both baseline and optional early CSI acquisition on candidate cells.

Agreement in RAN1#120 meeting For target cell CSI acquisition, - A UE is provided with RRC configurations for periodic CSI-RS resource(s) and CSI report(s) for one or more candidate cell - For a candidate cell, - down-select from the following alternatives: - Alt 1: A single CSI report configuration is configured - Alt 2: Multiple CSI report configurations can be configured - down-select from the following alternatives: - Alt X: A single CSI-RS resource for CMR is associated with a CSI report configuration - Alt Y: Multiple CSI-RS resources for CMR can be associated with a CSI report configuration - FFS: Semi-persistent CSI-RS resource - After the RRC configuration and before the reception of CSC, the UE may measure CSI based on the configured CSI-RS resource(s), which is subject to UE capability - FFS: whether or how to select a subset of CSI-RS resources to measure - FFS: when the UE may start measuring the configured CSI-RS resources - UE determines the CSI report configuration based on the CSC - After the reception of cell switch command, the UE may measure (depending on the timeline) CSI-RS resource(s) associated with determined CSI report configuration - The latest available measured CSI on target cell resource(s) is conveyed at least by a single report, and the report is sent to the target cell - Option 1: to use UCI - Option 2: to use MAC CE Note: with this agreement, the working assumption made in RAN1#119 is automatically confirmed.

Working Assumption in RAN1#119 meeting As baseline, CSI-RS measurement and CSI reporting operations are performed after reception of LTM CSC MAC CE. - The report is sent directly to target cell. - Introduce UE capability for CSI-RS measurement can start before reception of LTM CSC MAC CE. - Other than UE capability, there is no additional spec impact compared to the baseline (only one triggering mechanism will be specified).

- Issue 4-12: Early CSI acquisition on a candidate cell Candidate options: - Option 1: Qualcomm - For the baseline CSI acquisition (DL CSI acquisition after LTM CSC MAC-CE reception), RAN4 to not define requirements. - For the optional CSI acquisition (DL CSI acquisition before LTM CSC MAC-CE reception), RAN4 to discuss whether and what requirements to define after the following aspects for CSI-RS based L1-RSRP measurement from the candidate cell are concluded: - Pre-requisite conditions, maximum RTD value, gap-less vs. gap-based, etc. - Option 2: Nokia - Discuss what is the point in time during the cell switch delay UE starts CSI-acquisition.

Proposal 4: Not to define requirements for both baseline and optional early CSI acquisition on candidate cells.

Conclusion

The contribution provides analysis on CSI-RS based L1 measurement. The following proposals are proposed:

Proposal 1: For step-2 skipped CSI-RS L1 measurement requirement, if the CSI-RS resources in serving cell and neighbour cell are colliding in time domain, sharing factor shall be introduced.

Proposal 2: If a SCell cell is configured as a candidate cell, we prefer the serving cell follows the legacy way (i.e., step 2 is not skipped).

Proposal 3: A measurement is defined as a CSI-RS based intra-frequency L1 measurement provided that:

- the SCS of the CSI-RS resource of the neighbour cell configured for L1 measurement is the same as the SCS of active DL BWP, and

- the CP type of the CSI-RS resource of neighbour cell configured for L1 measurement is the same as the CP type of active DL BWP, and

- the CSI-RS resource of the neighbour cell configured for L1 measurement is included within the active DL BWP.

- at least 48 RBs of the CSI-RS resource, needs to be confined within the active DL BWP.

Proposal 4: Not to define requirements for both baseline and optional early CSI acquisition on candidate cells.

Reference

- RP-242356, Revised Work Item: NR mobility enhancements Phase 4

- R4-2416862, WF on RRM requirements for NR_Mob_Ph4_Part1, Apple

- R4-2502601, WF on RRM requirements for NR_Mob_Ph4_Part1, Apple