3GPP TSG-RAN WG4 Meeting # 115	R4-2506733

Malta, MT, 19th – 23rd May, 2025

Title: 	Discussion on conditional Intra-CU LTM

Source: 	Huawei, HiSilicon

Agenda item:	7.29.3.3

Document for:	Discussion

Introduction

The revised R19 mobility WI was agreed in [1]. The objectives are duplicated as below.

- 4.1 Objective of SI or Core part WI or Testing part WI - Specify support for inter-CU Layer1/Layer 2 Triggered Mobility (LTM) [RAN2, RAN3] - Prioritize the case when CU is acting as MN when DC is not configured - When DC is configured, inter-CU LTM can be configured either in MN or in SN but not both at the same time. For such cases: - As secondary priority, support the case where CU is acting as SN and MN is unchanged - As secondary priority, support the case where CU is acting as MN and SN is unchanged or SN is released - Specify support for subsequent LTM mobility procedures aiming to avoid RRC configuration between cell switches as per Rel-18 LTM - Coordination with SA3 needed with respect to security key handling - Note: Rel. 18 intra-CU LTM procedure is considered as baseline for adding inter-CU support - Measurements related enhancements for purpose of supporting LTM: [RAN2, RAN1] - Measurement related enhancements are applicable to Intra-CU MCG/SCG LTM and Inter-CU MCG/SCG LTM - Specify necessary components to support event triggered L1 measurement reporting [RAN2, RAN1] - Specify support for CSI-RS measurements for LTM procedures and enable CSI-RS based beam management [RAN1] - Specify CSI acquisition on candidate cell(s) based on CSI-RS before or during LTM cell switch [RAN1] NOTE: RAN1 WG to decide on whether to support CSI report before or during the LTM cell switch, as part of the CSI acquisition procedure. - Specify support of conditional Intra-CU LTM [RAN2, RAN3, RAN1] - Specify UE evaluated conditions for triggering LTM - Aim to support conditional LTM including subsequent LTM - Limit specifying the conditional LTM to the scenario where the UE is in non-DC - Checkpoint at RAN#107 to review the objective on whether Intra-CU conditional LTM can be specified to DC scenarios and if so, to which cases. RAN WG work to not start before this checkpoint - Specify RRM requirements related to the above objectives as necessary [RAN4]

This contribution provides analysis on intra-CU LTM from RAN4 RRM requirement perspective.

Discussion

Regarding the third objective conditional intra-CU LTM, according to the latest revised WID, both conditional LTM and subsequent conditional LTM are to be supported. The analysis on the RRM requirements for conditional LTM and subsequent conditional LTM are provided in the followings.

General aspects on conditional LTM

Issue 1-1-1: L1 measurement on deactivated Scell and UE based TA measurement < Way forward > FFS the following options: Conclude this issue in the next meeting. - Option 1: Define requirements for L1 measurement on deactivated Scell. - Option 2: Define requirements for UE based TA measurement. - Option 3: Not define requirement for any of them.

As per R18 RAN2 signalling framework, UE can execute L1-RSRP measurement on the deactivated SCell/neighbour cell on the same frequency when the candidate cell is configured as a LTM candidate PCell. However so far there are no RRM requirements in RAN4. From defining requirements perspective, some aspects need to be considered. On the one hand, as the SCell is deactivated, to save power, UE may normally power off the SCell and turn on the RF chain when measurement is to be performed. In this case interruptions on activated serving cells are expected. Moreover whether to consider extended L1 measurement period for deactivated SCell (e.g., measCycleSCell in L3 RSRP measurement) may also need to be discussed in RAN4. On the other hand, since the cell on deactivated SCC is configured as candidate PCell, UE can always open the RF chain. In other words, UE regards the deactivated SCell as an activated PCell. In this case, there may be no interruptions due to measurement on the target cell.

We are open to whether to define L1 measurement requirements on deactivated SCell and the cells with same frequency as deactivated SCell. If companies would like to define the corresponding requirements, the above aspects are to be discussed.

Proposal 1: If companies would like to define L1 measurement requirements on deactivated SCell and the cells with same frequency as deactivated SCell, the following aspects need to be further discussed:

- Whether interruption is allowed on activated serving cells, and what’s the interruption percentage

- Whether to extend L1 measurement period of deactivated SCC (e.g., based on measCycleSCell).

Conditional LTM

As per the latest progress of RAN4#114bis meeting, the basic framework of CLTM delay is settled down. In the following, we provide our view on some undecided components.

Issue 1-2-5: TRRC < Way forward > FFS the following proposals: - Proposal 1: TRRC is the RRC procedure delay.

In RAN4#114 meeting, for the issue of starting point of C-LTM delay, one bracket is remained, because some company spotted that RAN2 is discussing when UE starts to execute condition evaluation, e.g., upon it receives the conditional LTM command or an activation indication. As per the latest RAN2 agreement: the UE starts execution condition evaluation once it receives CLTM configuration including C-LTM execution condition. Therefore the starting point of legacy CHO delay is from UE receives a RRC message implying conditional handover.

Proposal 2: TRRC is the RRC procedure delay.

Issue 1-2-4: Tinterrupt < Agreement > - Keep either TCLTM_execution or TCLTM-RRC-processing as one component for the conditional LTM delay. - Note: The legacy value(s) in previous releases will be reused. - TCLTM_execution or TCLTM-RRC-processing is not included in Tinterrupt. - Not include the components for TCI state activation and/or PDCCH order PRACH in C-LTM delay. - TEvent_DU can be updated based on RAN2 progress if needed. - Tmeasure is measurements time. It is defined from the end of TEvent_DU until UE executes LTM to a target cell. - If SSB based L1-RSRP measurement is used in the event, Tmeasure is SSB based L1-RSRP measurement period in R18 LTM plus SSB index acquisition time if needed. - Measurement latency for both serving and candidate cells need to be considered. - If SSB based L3-RSRP measurement is used in the event, reuse Tmeasure in CHO. - FFS: If CSI-RS based L1-RSRP measurement is used in the event, Tmeasure is CSI-RS based L1-RSRP measurement period which is to be introduced in R19. < Way forward > FFS - Tfirst-RS and TRS-proc are included in Tinterrupt - Reuse the values in Rel-18 LTM, and further discuss whether the conditions for Tfirst-RS = 0 and TRS-proc = 0 from Rel-18 LTM can be reused or not.

In legacy conditional handover, Tmeasure is the time duration when the condition event is really met at UE side (i.e., UE realize the condition of CHO is met).

The measurement time delay is defined from the end of TEvent_DU until UE executes a handover to a target cell and interruption time starts. For conditional intra-frequency handover, the measurement time delay measured without Time To Trigger (TTT) and L3 filtering shall be less than Tidentify intra with index or Tidentify_intra_without_index defined in clause 9.2.5.1 or clause 9.2.6.2.

As both L1 measurement and L3 measurement can be used to trigger conditions of C-LTM, Tmeas can be the L3 measurement period or L1 measurement period. Regarding CSI-RS based L1 RSRP measurement, to correctly receive configured CSI-RS resource, UE shall perform PSS/SSS detection, acquire the SFN information and perform CSI-RS based measurement. Therefore if CSI-RS based L1-RSRP measurement is used in the event, Tmeasure shall include TPSS/SSS, TCSI-RS_SFN_intra and CSI-RS based L1-RSRP measurement period.

Proposal 3: In C-LTM delay, Tmeasure is measurement time. It is defined from the end of TEvent_DU until UE executes LTM to a target cell.

    - If SSB based L1-RSRP measurement is used in the event, Tmeasure is SSB based L1-RSRP measurement period in R18 LTM plus SSB index acquisition time if needed.

      - Measurement latency for both serving and candidate cells need to be considered.

    - If SSB based L3-RSRP measurement is used in the event, reuse Tmeasure in CHO.

    - If CSI-RS based L1-RSRP measurement is used in the event, Tmeasure shall include TPSS/SSS, the time period used to acquire the SFN information (TCSI-RS_SFN_intra) and CSI-RS based L1-RSRP measurement period.

In RAN4#114bis meeting, it is agreed that TCLTM_execution or TCLTM-RRC-processing is one component for the conditional LTM delay and but it is not included in Tinterrupt. The controversial part is that the component is TCLTM_execution or TCLTM-RRC-processing. Regarding TCHO_execution, in legacy CHO delay, TCHO_execution is the UE execution preparation time for conditional handover. This time duration originally called TRRC_2 for RRC processing time of the target cell. TCHO_execution is equivalent to the term TLTM-RRC-processing in LTM delay (which is the time for ASN.1 decoding and validity/compliance check). However, there is a slight difference between two terms. In legacy CHO, TCHO_execution is fixed to 10ms.

TCHO_execution is the UE execution preparation time for conditional handover. For conditional handover, it starts after UE realizes the condition of CHO is fulfilled and identity of the target cell is determined. For NES-based conditional handover, - It starts after UE realizes the condition of CHO is met and identity of the target cell is determined provided that UE has already successfully decoded DCI 2-9. - Otherwise, it starts after UE successfully decodes DCI 2-9.And TCHO_execution can be up to 10ms.

In R18 LTM delay, TLTM-RRC-processing is 0ms or 10ms according to ltm-FastProcessingConfig-r18 capability. It shall be noted that in R18 there are some further optimizations: when the number of the configured serving cells and candidate cells does not exceed maxNumberStoredConfigCells-r18, TLTM-RRC-processing is 0ms; otherwise, if the activated TCI states and/or received PDCCH order does not exceed maxNumberConfigs-r18, TLTM-RRC-processing can be 0ms as well. In the existing description of optimization, TCI states and received PDCCH orders shall be triggered in advanced for certain time before CSC. However in C-LTM, no CSC is indicated from network. Therefore if companies would like to reuse TLTM-RRC-processing in CLTM, the wording” before the LTM cell switch command” shall be updated.

TLTM-RRC-processing = 0, if the UE supports ltm-FastProcessingConfig-r18 capability, and: - The number of LTM candidate cell configurations does not exceed maxNumberConfigs-r18 and the number of the configured serving cells and the cells in the LTM configuration does not exceed maxNumberStoredConfigCells-r18, or - The following conditions are fulfilled for the target cell: - UE has received LTM candidate cell TCI state activation command for the target cell at least THARQ + 13 ms before the LTM cell switch command, and/or - UE has received PDCCH order for early RACH for the target cell at least NT,2+10 ms before the LTM cell switch command, where NT,2 is defined in section 8.1 of TS 38.213 [3], and - The total number of LTM candidate cells for which TCI state(s) were activated or PDCCH order was received before the cell switch command does not exceed maxNumberConfigs-r18, and the total number of serving cells and the cells in the LTM configuration for which TCI state(s) were activated and/or PDCCH order was received before the cell switch does not exceed maxNumberStoredConfigCells-r18. Otherwise TLTM-RRC-processing = 10 ms.

Proposal 4: If TCLTM_execution is used in C-LTM delay, TCLTM_execution =10ms; if TCLTM-RRC-processing is used in C-LTM delay, the legacy description of TLTM-RRC-processing (i.e., ” before the LTM cell switch command”) shall be updated.

In C-LTM, upon condition is met, after UE finishes preparation operations and leaves the source cell, the steps which are defined in R18 LTM cell switch delay, i.e., TLTM-processing, Tfirst-RS, TRS-proc , TLTM-IU, shall be followed.

In R18 LTM cell switch delay:

- TLTM-processing is the time for UE processing, consisting of applying the target cell parameters and L1/L2 change.

- Tfirst-RS is the time for fine time tracking and acquiring full timing information of the target cell.

- TRS-proc is the time for SSB processing.

- TLTM-IU is the uncertain time in transmit the first UL transmission.

Issue 1-2-10: TLTM-processing < Way forward > FFS the following proposal: - Proposal 1: TLTM-processing is the time for UE processing, consisting of applying the target cell parameters and L1/L2 change. - Proposal 1a: TLTM-processing can be up to 20ms.

In R18 LTM cell switch delay, TLTM-processing is the time for UE processing, consisting of applying the target cell parameters and L1/L2 change.

If the UE supports ltm-FastUE-Processing-r18 capability, the value of TLTM-processing equals to - fr1-r18 for FR1 to FR1 LTM cell switch. - fr2-r18 for FR2 to FR2 LTM cell switch. - fr1-AndFR2-r18 for FR1 to FR2 and FR2 to FR1 LTM cell switch. Otherwise, the value of TLTM-processing equals to - 20 ms for FR1 to FR1 and FR2 to FR2 LTM cell switch. - 40 ms for FR1 to FR2 and FR2 to FR1 LTM cell switch.

Where

Proposal 5: TLTM-processing in R18 LTM cell switch delay can be reused for TCLTM-processing.

Issue 1-2-14: Twait_RS < Way forward > FFS the following proposal: - Proposal 1: Twait_RS is the uncertainty in acquiring the first SSB for T/F tracking on the new cell.

In RAM4#114bis meeting, some company proposed CLTM delay and interruption as [R4-2503605]:

DCLTM = TRRC + TEvent_DU + Tmeasure + TCLTM_interrupt + TLTM-RRC-processing + Twait_RS and TCLTM_interrupt = TRS + TRS-proc + TLTM-processing + TLTM-IU,

- Twait_RS is the uncertainty in acquiring the first SSB for T/F tracking on the new cell.

- TRS is time for fine time tracking and acquiring full timing information of the target cell. TRS is the time needed to get ready to receive SSB on target cell plus the SSB duration. Here we propose TRS as 2ms

We understand the motivation of the proposal is that UE can receive/transmit data on serving cell during the time duration of waiting for SSB which is used for fine time tracking on the target cell. Interruption happens from the moment when SSB is received. For conditional LTM procedure, as network has no information of the exact instance when UE is to switch to another cell, network would keep scheduling the UE during the duration of waiting for the first SSB. So we think the proposal in [R4-2503605] is reasonable and can reduce interruption length during CLTM procedure. Furthermore we think the value of TRS shall be large than (SSB duration+2ms). We also understand the proposal of taking Twait_RS as a component of CLTM delay and taking TRS as a component in Tinterruption is an optimization. However due to limited time, using the legacy Tfirst-RS in R18 LTM is also fine.

Proposal 6: The legacy Tfirst-RS which is the time for fine time tracking and acquiring full timing information of the target cell can be reused, and the component shall be included in TCLTM-interruption.

Issue 1-2-13: TLTM-IU < Way forward > FFS the following proposal: - Proposal 1: TLTM-IU is the interruption uncertainty during LTM cell switch.

It is straight forward that TLTM-IU is the uncertain time in transmit the first UL transmission, and it shall be included in TCLTM-interruption.

Proposal 7: Legacy TLTM-IU which is the uncertain time in transmit the first UL transmission can be reused, and the component shall be included in TCLTM-interruption.

Based on the above discussion, C-LTM delay can be depicted as

DCLTM = TCLTM-RRC + TCLTM-Event_DU + T CLTM-measure + TCLTM-RRC-processing + TCLTM_interruption

Where TCLTM_interruption = T CLTM-LTM-processing + Tfirst-RS + TRS-proc + T CLTM-LTM-IU

Proposal 8: C-LTM delay can be depicted as

DCLTM = TCLTM-RRC + TCLTM-Event_DU + T CLTM-measure + TCLTM-RRC-processing + TCLTM-interruption

Where TCLTM-interruption = T CLTM-LTM-processing + Tfirst-RS + TRS-proc + T CLTM-LTM-IU

Subsequent conditional LTM

The requirements of subsequent CPA and subsequent CPC have been specified in R18. The delay for subsequent CPA/CPC is alike CHO delay in some extent. The difference is the starting point. In subsequent CPA/CPAC the starting point is from the time UE transmits RRC Reconfigurationcomplete message for the previous PSCell addition or change. In last meeting, the start point has reached conclusion for subsequent conditional LTM

Issue 1-4-2: The starting point of Subsequent conditional LTM delay < Agreement > ad hoc - The starting point of the subsequent conditional LTM delay is when UE transmits RRCReconfigurationcomplete message for the previous conditional LTM, [under assumption that UE will continue evaluating the CLTM event after the completion of previous CLTM]. - Revisit if RAN2 has more progress.

Apart from the starting point, the subsequent conditional LTM cell switch delay can be discussed based on the conclusion of conditional LTM delay, as most components in conditional LTM delay can be reused for subsequent C-LTM.

Proposal 9: Apart from the starting point, the subsequent conditional LTM cell switch delay can be discussed based on the conclusion of conditional LTM delay, as most components in conditional LTM delay can be reused for subsequent C-LTM.

Conclusion

The contribution provides analysis on intra-CU LTM from RAN4 RRM requirement perspective. The following proposals are proposed:

Proposal 1: If companies would like to define L1 measurement requirements on deactivated SCell and the cells with same frequency as deactivated SCell, the following aspects need to be further discussed:

- Whether interruption is allowed on activated serving cells, and what’s the interruption percentage

- Whether to extend L1 measurement period of deactivated SCC (e.g., based on measCycleSCell).

Proposal 2: TRRC is the RRC procedure delay.

Proposal 3: In C-LTM delay, Tmeasure is measurement time. It is defined from the end of TEvent_DU until UE executes LTM to a target cell.

    - If SSB based L1-RSRP measurement is used in the event, Tmeasure is SSB based L1-RSRP measurement period in R18 LTM plus SSB index acquisition time if needed.

      - Measurement latency for both serving and candidate cells need to be considered.

    - If SSB based L3-RSRP measurement is used in the event, reuse Tmeasure in CHO.

    - If CSI-RS based L1-RSRP measurement is used in the event, Tmeasure shall include TPSS/SSS, the time period used to acquire the SFN information (TCSI-RS_SFN_intra) and CSI-RS based L1-RSRP measurement period.

Proposal 4: If TCLTM_execution is used in C-LTM delay, TCLTM_execution =10ms; if TCLTM-RRC-processing is used in C-LTM delay, the legacy description of TLTM-RRC-processing (i.e., ” before the LTM cell switch command”) shall be updated.

Proposal 5: TLTM-processing in R18 LTM cell switch delay can be reused for TCLTM-processing.

Proposal 6: The legacy Tfirst-RS which is the time for fine time tracking and acquiring full timing information of the target cell can be reused, and the component shall be included in TCLTM-interruption.

Proposal 7: Legacy TLTM-IU which is the uncertain time in transmit the first UL transmission can be reused, and the component shall be included in TCLTM-interruption.

Proposal 8: C-LTM delay can be depicted as

DCLTM = TCLTM-RRC + TCLTM-Event_DU + T CLTM-measure + TCLTM-RRC-processing + TCLTM-interruption

Where TCLTM-interruption = T CLTM-LTM-processing + Tfirst-RS + TRS-proc + T CLTM-LTM-IU

Proposal 9: Apart from the starting point, the subsequent conditional LTM cell switch delay can be discussed based on the conclusion of conditional LTM delay, as most components in conditional LTM delay can be reused for subsequent C-LTM.

Reference

- RP-242356, Revised Work Item: NR mobility enhancements Phase 4

- R4-2504910, WF on RRM requirements for NR_Mob_Ph4_Part2, China Telecom