3GPP TSG-RAN WG4 Meeting # 115	R4-2506739

Malta, MT, 19th – 23rd May, 2025

Title: 	Discussion on low band carrier aggregation via switching

Source: 	Huawei, HiSilicon

Agenda item:	7.7.4

Document for:	Discussion

Introduction

WI on low band carrier aggregation via switching is approved in [1].

- 4.1 Objective of SI or Core part WI or Testing part WI Introduce physical layer procedures and requirements to enable low band carrier aggregation via switching according to the following objectives: - Specify UE requirements, including at least switching gap (if needed), and corresponding physical layer procedures to allow switching between {case 1, case 2} [RAN4, RAN1] - Case 1: Tx/Rx on FDD carrier 1 and no Rx on SDL carrier 2 - Case 2: Rx on SDL carrier 2 and no Tx/Rx on FDD carrier 1 - RAN1 to specify only a semi-static switching pattern based on RRC configuration, liaising with RAN2 and RAN4 as necessary - Specify the switching delay and time mask for carrier switching [RAN4] - Specify necessary RRM requirements [RAN4] - Define the corresponding UE capabilities [RAN4, RAN2, RAN1] - Consider the following deployment constraints: - The carrier frequency for all cases is <1 GHz - Co-located and synchronized network deployment for both carriers - Both carriers are in a single TAG - SCS 15KHz on both carriers Note 1: Specify requirements for the feature with the following example band combination: CA_n5A-n29A in this WI, with additional band combinations to be handled via the basket work item approach Note 2: Strive to minimize the RAN1 impact

The contribution provides further analysis on low band carrier aggregation via switching from RRM perspective.

Discussion

General Aspects

Issue 1-1-1: SSB assumptions for R19 LB CA via switching Continue discussion in next meeting. Options of this meeting are captured in topic summary R4-2503614.

In previous meeting, SSB-less operation and SSB operation for LB CA switching were discussed. It shall be noted that SSB-less operation just leverages the concept that some measurement results from PCC can be used to SDL and it does not mean that no SSB is transmitted on SDL SCell. As SSB (rather than ATRS or P-TRS) is always used as a baseline when RAN4 specify RRM requirements, SSB based requirements can be used as baseline for Rel-19 low band CA via switching.

Proposal 1: SSB based (rather than TRS/A-TRS) requirements can be used as baseline for Rel-19 low band CA via switching.

Issue 1-1-2: MRTD assumptions for R19 LB CA via switching - FFS: - Option 1 (OPPO, vivo, QC, ZTE, Nokia, Xiaomi, CATT, Samsung, HW, MTK, LGE): The maximum receive timing difference for collocated FDD PCell and SDL SCell CA via switching is 3us. - Option 3 (Ericsson): MRTD = CP for in LB-LB CA with switching.

As per TS38.104, for inter-band carrier aggregation, TAE shall not exceed 3µs. As the two switching carriers are co-located low bands and synchronized, RTD between both carriers at UE side shall be less than or equal to 3us.

Proposal 2: The maximum receive timing difference for collocated FDD PCell and SDL SCell CA via switching is 3us.

Switching delay or interruption requirement

Operators and interested companies have provided sufficient information of the background, motivation and possible solutions of low band CA [2]. The agreed WI aims to specify requirements with the example band combination CA_n5A-n29A where n29 is SDL band, and n5 is FDD band.  The two carriers are collocated and synchronized in a single TAG.

Figure 1. Example scenario (referenced from [RP-242532])

If a single antenna is implemented to aggregate the two bands, it is challenged for simultaneous Tx/Rx between the PCell and the SCell. Therefore switching between two carriers is executed. A semi-static switching pattern is configured by RRC configuration. UE is allowed to switch between case 1 and case 2.

-	Case 1: Tx/Rx on FDD carrier and no Rx on SDL carrier

-	Case 2: Rx on SDL carrier and no Tx/Rx on FDD carrier

Issue 1-2-1A: Necessity of interruption requirement in RRM between FDD CC and SDL CC Continue discussion in next meeting. Options of this meeting are captured in topic summary R4-2503614.

In RAN1#120bis meeting, the switching gap for switching from SDL to FDD carrier had reached consensus, where the switching gap ends at the end of slot on SDL and it includes at least “the switching period (i.e., 35us, 140us and FFS on other additional value)” and “UL TA”.  It means that during switching gap, UE is not expected to receive any DL signals on SDL carrier, and the UL transmission on FDD carrier can be scheduled normally. In other words, for switching from FDD carrier to SDL carrier, the scheduling restriction for SDL carrier during switch gap would be specified in RAN1, and the switch gap has no impact on FDD UL. Therefore interruption requirements are not needed for this case.

For switching from FDD carrier to SDL carrier, three solutions are under discussion in RAN1. RAN4 can wait the conclusion from RAN1.

Agreement For RRC configuration of semi-static switching pattern in Rel-19 low NR band carrier aggregation via switching, regarding the location of the switching gap, - For switching from SDL carrier to FDD carrier, the switching gap is always assumed at the “switch from” carrier, i.e., SDL carrier. - For switching from FDD carrier to SDL carrier, downselect one of the following solutions - Alt 1: Switching gap is always assumed at the “switch from” carrier, i.e., FDD carrier - Alt 2: NW configures whether the switching gap is at the FDD carrier or SDL carrier - Alt 3: Switching gap is always assumed at the SDL carrier Agreement For RRC configuration of semi-static switching pattern in Rel-19 low NR band carrier aggregation via switching, the duration of the switching gap is RRC configured by the NW, - Switching gap duration can be different for FDD to SDL switch, and SDL to FDD switch. - Note: NW ensures that the switching gap is enough to cover at least the switching period as in RAN4 LS (R1-2501702) and UL TA (if needed) - For the UL TA, - Switching gap for SDL to FDD switch ends at the end of slot on SDL - Note: RAN1 assumes that switching gap for SDL to FDD switch includes UL TA - FFS: whether it has specification impact

SDL to FDD switch

Reference from R1-2502637

Proposal 3:

- For switching from SDL carrier to FDD carrier, during switching gap, UE is not expected to receive any DL signals on SDL carrier, and UL transmission on FDD carrier can be scheduled normally. RAN4 would not specify interruption requirements as RAN1 would define switching gap.

- For switching from FDD carrier to SDL carrier, RAN4 wait RAN1 conclusion.

Issue 1-2-1B: Necessity of interruption requirement in RRM when more carriers are considered Continue discussion in next meeting. Options of this meeting are captured in topic summary R4-2503614.

RF session agreed that DL switching happens between two low bands, while the band combination can be more than two bands. Under this scenario, the switching between FDD and SDL would cause interruption to other combined active SCell.

Issue 2-1: Switching delay Agreement: - Specify the switching period(s) based on the scenario with the switching between two low bands in Rel-19 WI - It is not precluded that such two low bands are further combined with the additional band - The combination of those two low bands with other band can be further discussed in the future release

In RF, the switching period for low-low band switching is concluded as 35us, 140us and FFS on other additional value(s) [4].

- Define switching periods between FDD low band and SDL low band with the following values as a UE capability to accommodate different UE implementations - 35us, 140us Additional value(s) are under discussion. If any additional value(s) are confirmed, RAN4 will provide them in a future LS. RAN4 respectfully requests RAN1 to take this information into account in their work.

As we known, the interruption length in RRM considers not only switching period but also the time misalignment between aggressive band and victim band. Therefore the interruption can be TswitchingSlot-length-victim+1.

Proposal 4: When more carriers are considered, switching between FDD and SDL would interrupt other combined active SCell, the interruption length is TswitchingSlot-length-victim+1.

SDL SCell related requirements

L3 measurement on activated SDL SCell

Issue 1-3-1: Activated SDL SCell L3 measurement and corresponding interruption Continue discussion in next meeting. Options of this meeting are captured in topic summary R4-2503614.

Regarding L3 measurement, in theory, as the two carriers are in low bands and co-located, and the transmit power configuration for each CC is supposed to be similar, the L3 RSRP results on SDL SCell and PCell is supposed to be within a limited range. Regarding neighbor cell measurement, as a SDL carrier is always co-located with a PCC, NW can determine the mobility based on the measurement on the target cell on PCC. Therefore the L3 RSRP measurement on SDL carrier seems unnecessary. Network can leverage the L3 measurement results on PCC and not configure MO on SDL carrier. However it is up to network implementation whether to configure SSB based L3 measurement. If network configures SSB based L3 measurement on SDL carrier, UE would follow network configuration and perform corresponding measurements.

Proposal 5: If network configures SSB based L3 measurement on SDL carrier, UE would perform corresponding measurements.

Regarding the L3 measurement requirements on SDL carrier, besides SMTC periodicity and DRX, the switching pattern as another factor needs to be considered. The switching pattern related agreements in RAN1 are duplicated as below.

Agreement - For RRC configuration of semi-static switching pattern in Rel-19 low NR band carrier aggregation via switching, support bitmap design - NW configures a periodicity, denoted as P slots, and a bitmap of P bits. Each bit in the bitmap indicates which carrier is configured for the corresponding slot, ‘0’ indicates FDD carrier 1, and ‘1’ indicates SDL carrier 2 - The pattern starts from the beginning of SFN 0 of PCell - Restriction of maximum X switch(es) within Y slot(s), - FFS: detailed value and relation of X and Y. - FFS: other restriction Agreement For RRC configuration of semi-static switching pattern in Rel-19 low NR band carrier aggregation via switching, semi-static switching pattern is periodic with periodicity of P slots (ms), support P= 40 ms

UE only performs SDL measurement on the overlapped occasions between SMTC window and SDL duration in switch pattern.

Proposal 6: The SSB based L3 measurement on SDL is performed on the overlapped occasions between SMTC window and SDL duration in switch pattern.

L3 measurement on deactivated SDL SCell

Issue 1-3-2: Deactivated SDL SCell measurement and corresponding interruption Continue discussion in next meeting. Options of this meeting are captured in topic summary R4-2503614.

As we know, the switch pattern specified in RAN1 is only applicable to activated carriers. As per the agreement in last RAN4 meeting (see below), our original thinking is that as the measurement on deactivated SDL would result in interruption on PCell, UE can skip the L3 measurement and network can leverage the measurement result of the co-collocated PCC.

- For the SCell which is deactivated, - The switching pattern for SDL SCell is not applied. - RAN4 will discuss and decide whether/how UE to perform the deactivated SDL SCell measurement

However if network configures MO on the deactivated SDL carrier, UE will still follow the network configuration to perform L3 measurement on SDL. Since the periodicity of switching pattern P is assumed as 40ms in RAN1 so far, there is at least one available SSB within measCycleSCell. So the legacy measurement period on deactivated SDL would not be impacted by switching pattern.

Proposal 7: If network configures SSB based L3 measurement on the deactivated SDL carrier, the legacy measurement period on deactivated SCC is reused for measurement on deactivated SDL carrier.

In the following, the interruption during measurement on deactivated SDL carrier is to be discussed. As we know the legacy interruption due to measurement on deactivated SCC requires has two cases:

-when measCycleSCell is smaller than 640ms, no interruption is allowed. It means that UE shall always open the SCC RF chain after SCell addition.

-when measCycleSCell is 640 ms or longer, up to 0.5% interruption is allowed. It means UE can close SCC RF chain after SCell addition, and open the RF chain again when measurement on deactivated SCC is performed.

Interruptions on PCell or activated SCell(s) due to measurements when an SCell is deactivated are allowed with up to 0.5 % probability of missed ACK/NACK when the configured measCycleSCell [2] is 640 ms or longer.

In R19 low band switching scenario, UE is not able to simultaneously receive on both FDD DL and SDL. In other words, UE shall swich back to PCC after measurement on deactivated SDL carrier, even if measCycleSCell is smaller than 640m. Therefore the missed ACK/NACK percentage shall be allowed in low band switching scenario.

Proposal 8: Interruptions due to measurements on deactivated SDL are allowed with up to 0.5 % probability of missed ACK/NACK, regardless of the length of measCycleSCell.

SDL SCell activation

Issue 1-3-3: SDL SCell activation/deactivation Continue discussion in next meeting. Options of this meeting are captured in topic summary R4-2503614.

In Rel-19 low NR band carrier aggregation via switching, we don’t think the “unknown” SCell activation procedure is supposed to be considered. Even if UE didn’t report L3 measurement of deactivated SDL SCell before SCell activation MAC CE is indicated, the channel quality information of the SDL SCell can still be known to network and UE can still acquire the coarse timing of the SDL SCell:

- As the two carriers are in low bands (<1GHz) and co-located, and the transmit power configuration for each CC is similar, the L3 RSRP results on SDL SCell and PCell shall be almost identical. So the channel quality on PCell can represents that on SDL SCell.

- As per TS38.104, for inter-band carrier aggregation, TAE shall not exceed 3µs. According to the deployment constraint in WID, the SCS for both carrier is 15kHz, then CP length is 4.67us. Furthermore, as the two switching carriers are co-located and synchronized, RTD between both carriers at UE side shall be less than CP. It means that the coarse timing on SDL SCell can refer to that of PCell.

Proposal 9: RAN4 not to consider unknown SDL SCell activation procedure in R19 low NR band carrier aggregation via switching.

In R18 inter-band SSB-less WI, the SSB-Less SCell activation delay is shorter than normal SCell activation procedure when the side conditions (duplicated as below) are satisfied.

- The RTD between the target SCell and the collocated reference serving cell is within CP where CP is corresponding to the SCS of SSB-less SCell, and - The EPRE difference at the UE is smaller than or equal to [30] dB, where EPRE difference is the power difference between TRS/A-TRS symbol on the SSB-less SCell and SSB symbol on the reference serving cell normalized by SCSs of SSB of reference cell and A-TRS/P-TRS of SSB-less SCell, and - The RS(s) of the SSB-less SCell being activated is (are) QCL-TypeA with TRS(s) of the SSB-less SCell being activated, and the TRS(s) of the SSB-less SCell being activated is (are) further QCL-TypeC with SSB(s) of an inter-band active serving cell, and the inter-band active serving cell shall be same as the reference serving cell.

Tactivation_time is - Tfirst_TRS + TTRS +5 ms, if aperiodic CSI-RS resources are not configured for SCell activation or UE do not support aperiodicCSI-RS-FastScellActivation-r17, when the the EPRE difference is smaller than or equal to 12 dB - Tfirst_ATRS + Tgap + TATRS + 5 ms if aperiodic CSI-RS resources are configured for SCell activation for UE supporting aperiodicCSI-RS-FastScellActivation-r17, when the the EPRE difference is smaller than or equal to 12 dB - Tfirst_TRS + 2*TTRS +5 ms, when the EPRE difference is larger than 12 dB but smaller than or equal to [30] dB

For low band CA via switching, as discussed above, the first RTD condition can be satisfied. In R19 low band carrier switching scenario, as the frequency separation is very small, EPRE difference between the two switching carriers can definitely be smaller than 12dB.  Therefore in general the SDL SCell activation delay can take R18 SSB-less SCell activation delay (where the EPRE difference is smaller than or equal to 12 dB) as a baseline. In other words, two RS resources are needed during SDL SCell activation procedure, i.e.., one for AGC, and the other is for fining time tracking.

Proposal 10: In SDL SCell activation procedure, two RS resources are needed for Tactivation_time with the side conditions:

  - RTD between the FDD PCell and SDL SCell <=3us

  - Reception power difference between the FDD PCell and SDL SCell is smaller than or equal to 12dB

If the above RS resource is SSB, the question is whether UE would stay on SDL carrier until the SCell activation procedure is completed.

- Option 1: UE stays on SDL carrier until the SCell activation procedure is completed. The whole SCell activation delay would be specified as interruption or scheduling restriction. It shall be noted that UE shall also wait for one CSI-RS resource for CQI. And a valid CQI will be transmitted on PCC.

- Option 2: As switching pattern agreed in RAN1 is not applicable to SDL SCell activation procedure, UE can switch to SDL carrier according to SSB occasions. The interruption or scheduling restriction would happen on measured SSB symbols and 1 symbol before each consecutive SSB symbols to be measured and 1 symbol after each consecutive SSB symbols to be measured. It shall be noted that UE shall also measure one CSI-RS resource for CQI. And a valid CQI will be transmitted on PCC.

To reduce throughput loss, we more prefer option 2.

If the to-be-used RS can be A-TRS or P-TRS, more discussion is needed.

Proposal 11: During SDL SCell activation,

If the to-be-used RS resource is SSB:

- Option 1: UE stays on SDL carrier until the SCell activation procedure is completed. The whole SCell activation delay would be specified as interruption or scheduling restriction. It shall be noted that UE shall also wait for one CSI-RS resource for CQI. And a valid CQI will be transmitted on PCC.

- Option 2: UE switches to SDL carrier according to SSB occasions. The interruption or scheduling restriction would happen on SSB symbols and 1 symbol before each consecutive SSB symbols to be measured and 1 symbol after each consecutive SSB symbols to be measured. It shall be noted that UE shall also measure one CSI-RS resource for CQI. And a valid CQI will be transmitted on PCC.

FFS if the to-be-used RS is A-TRS or P-TRS.

BFD/CBD, TCI state switch delay, PL-RS switch delay, L1-RSRP/SINR requirements for SDL SCell

Issue 1-3-4: BFD/CBD, TCI state switch delay, PL-RS switch delay, L1-RSRP/SINR requirements for SDL SCell Continue discussion in next meeting. Options of this meeting are captured in topic summary R4-2503614.

As there is no uplink on SDL carrier, no PL-RS switch delay requirements are to be considered.

Proposal 12: There is no PL-RS switch delay requirements for SDL SCells in LB CA via switching.

For R18 SSB-less SCell, if CSI-RS based L1 measurement is configured, the legacy requirements shall apply.

For R19 Low band CA switching, if SSB/CSI-RS based L1 measurement is configured on SDL SCell, UE shall perform related measurement. The SSB/CSI-RS based L1 measurement on SDL shall be performed on the overlapped occasions between SSB/CSI-RS and SDL duration in switch pattern.

Proposal 13: The SSB/CSI-RS based L1 measurement on SDL is performed on the overlapped occasions between SSB/CSI-RS and SDL duration in switch pattern.

BFD/CBD/TCI switching can also be performed on SDL. The similar principle as L1-RSRP measurement is to be applied.

For TCI switching, we don’t observe strong necessity of using SSB on PCell as the QCL source for TCI state SDL SCell. UE can perform TCI state switching on SDL carrier based on SSB which is overlapped with SDL duration in switch pattern.

Proposal 14: The BFD/CBD/TCI switching on SDL carrier is performed on the overlapped occasions between SSB/CSI-RS and SDL duration in switch pattern.

PCC related RRM requirements

For low band switching, UE semi-static switches between PCell and SDL SCell. The SSB based L3/L1 measurement on PCC is supposed to be performed on the overlapped occasions between SMTC window and PCC duration in switch pattern.

Proposal 15: For L1/L3 measurement on PCC, the SSB based L3/L1 measurement is performed on the overlapped occasions between SMTC window and PCC duration in switch pattern.

In last meeting, RAN4 identified UL transmit timing issue on PCell and sent LS to RAN1[5].

- In TS38.133, UL transmit timing requirements are applicable provided that at least one SSB is available at the UE during the last 160 ms.

As per the latest RAN1 agreement, “semi-static switching pattern is periodic with periodicity of P slots (ms), support P= 40 ms”, switching pattern has no impact on uplink transmission requirements. If additional P value which is larger than 160ms is introduced in RAN1, RAN4 can revisit the conclusion

Proposal 16: The existing timing requirement for PCell can be reused assuming P=40ms.

Conclusions

This contribution provides further analysis on low band carrier aggregation via switching from RRM perspective. The following proposals are provided:

Proposal 1: SSB based (rather than TRS/A-TRS) requirements can be used as baseline for Rel-19 low band CA via switching.

Proposal 2: The maximum receive timing difference for collocated FDD PCell and SDL SCell CA via switching is 3us.

Proposal 3:

- For switching from SDL carrier to FDD carrier, during switching gap, UE is not expected to receive any DL signals on SDL carrier, and UL transmission on FDD carrier can be scheduled normally. RAN4 would not specify interruption requirements as RAN1 would define switching gap.

- For switching from FDD carrier to SDL carrier, RAN4 wait RAN1 conclusion.

Proposal 4: When more carriers are considered, switching between FDD and SDL would interrupt other combined active SCell, the interruption length is TswitchingSlot-length-victim+1.

Proposal 5: If network configures SSB based L3 measurement on SDL carrier, UE would perform corresponding measurements.

Proposal 6: The SSB based L3 measurement on SDL is performed on the overlapped occasions between SMTC window and SDL duration in switch pattern.

Proposal 7: If network configures SSB based L3 measurement on the deactivated SDL carrier, the legacy measurement period on deactivated SCC is reused for measurement on deactivated SDL carrier.

Proposal 8: Interruptions due to measurements on deactivated SDL are allowed with up to 0.5 % probability of missed ACK/NACK, regardless of the length of measCycleSCell.

Proposal 9: RAN4 not to consider unknown SDL SCell activation procedure in R19 low NR band carrier aggregation via switching.

Proposal 10: In SDL SCell activation procedure, two RS resources are needed for Tactivation_time with the side conditions:

  - RTD between the FDD PCell and SDL SCell <=3us

  - Reception power difference between the FDD PCell and SDL SCell is smaller than or equal to 12dB

Proposal 11: During SDL SCell activation,

If the to-be-used RS resource is SSB:

- Option 1: UE stays on SDL carrier until the SCell activation procedure is completed. The whole SCell activation delay would be specified as interruption or scheduling restriction. It shall be noted that UE shall also wait for one CSI-RS resource for CQI. And a valid CQI will be transmitted on PCC.

- Option 2: UE switches to SDL carrier according to SSB occasions. The interruption or scheduling restriction would happen on SSB symbols and 1 symbol before each consecutive SSB symbols to be measured and 1 symbol after each consecutive SSB symbols to be measured. It shall be noted that UE shall also measure one CSI-RS resource for CQI. And a valid CQI will be transmitted on PCC.

FFS if the to-be-used RS is A-TRS or P-TRS.

Proposal 12: There is no PL-RS switch delay requirements for SDL SCells in LB CA via switching.

Proposal 13: The SSB/CSI-RS based L1 measurement on SDL is performed on the overlapped occasions between SSB/CSI-RS and SDL duration in switch pattern.

Proposal 14: The BFD/CBD/TCI switching on SDL carrier is performed on the overlapped occasions between SSB/CSI-RS and SDL duration in switch pattern.

Proposal 15: For L1/L3 measurement on PCC, the SSB based L3/L1 measurement is performed on the overlapped occasions between SMTC window and PCC duration in switch pattern.

Proposal 16: The existing timing requirement for PCell can be reused assuming P=40ms.

Reference

- RP-243317, New WID on low band carrier aggregation via switching

- RP-242532, Motivation for a new work item on low band carrier aggregation via switching in Rel-19

- R4-2502588, WF on RRM requirements for NR_LBCA_Sw, Apple

- R4-2502877, LS on switching periods for low-low band switching, ATT

- R4-2502697, LS of RAN4 RRM agreements on LB CA via switching, Apple

- R4-2504895, WF on RRM requirements for NR_LBCA_Sw, Apple