3GPP TSG-RAN4 Meeting #115	R4-2506740

# Malta, MT, 19th – 23rd May, 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.133 | CR | 5569 | rev | - | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR on introducing antenna connections for 6Rx capable UEs |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_ENDC_RF_Ph4-Perf |  |  |  |  |  | Date: |  |  | 2025-04-27 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | The antenna connection for 6Rx capable UEs are to be specified. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Specify the antenna connection for 6Rx capable UEs. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | For RLM and BFD testing, the SNR levels for 6Rx capable UE are not clearly defined. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | A.3.6.1.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS38.533 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Start of Change 1>

A.3.6.1.3	Antenna connection for 6 Rx capable UEs

A.3.6.1.3.1	Introduction

All tests in clause A.4 and A.6 are specified for UEs supporting 2RX. In this clause, the antenna connection method for applying 2RX tests or 4RX tests to UEs supporting 6Rx antenna ports is specified. No tests are currently specified in clause A.4 or A.6 which are applicable only to 6Rx antenna ports, so 6Rx capable UEs are always tested by reusing tests which were originally specified for 2RX UEs or 4Rx UEs.

A.3.6.1.3.2	Principle of testing

A.3.6.1.3.2.1	Single carrier tests

For 6Rx capable UEs supporting at least one band where 2RX is supported and either 4RX or 6Rx is not supported, all single carrier tests specified in clauses A.4 and A.6 except those in clauses A.4.7 and A.6.7 shall be tested on any band where 2RX is supported and 6Rx is not supported with the antenna connection specified in clause A.3.6.1.3.2.4. For single carrier tests specified in clauses A.4.7 or A.6.7, all tests shall be tested with the antenna connection specified in clause A.3.6.1.3.2.4 for bands where 2RX is supported and 6Rx is not supported, and the antenna connection specified in clause A.3.6.1.3.2.6 for bands where 6Rx is supported.

For 6Rx capable UEs supporting at least one band where 4RX is supported and 6Rx is not supported but without supporting any band where only 2RX is supported, all single carrier tests specified in clauses A.4 and A.6 except those in clauses A.4.7 and A.6.7 shall be tested on any band where 4RX is supported and 6Rx is not supported with the antenna connection specified in clause A.3.6.1.3.2.5. For single carrier tests specified in clauses A.4.7 or A.6.7, all tests shall be tested with the antenna connection specified in clause A.3.6.1.3.2.5 for bands where 4RX is supported and 6Rx is not supported, and the antenna connection specified in clause A.3.6.1.3.2.6 for bands where 6Rx is supported. For radio link monitoring tests, the SNR levels are modified according to table A.3.6.1.1.2.1-1 and table A.3.6.1.1.2.1-2. For beam failure detection and link recovery tests, the SNR levels are modified according to table A.3.6.1.1.2.1-3.

For 6Rx capable UEs which do not support any band where 6Rx is not supported, all tests specified in clauses A.4 and A.6 shall be tested using the antenna connection specified in clause A.3.6.1.3.2.6. For radio link monitoring tests, the SNR levels are modified according to table A.3.6.1.1.2.1-1 and table A.3.6.1.1.2.1-2. For beam failure detection and link recovery tests, the SNR levels are modified according to table A.3.6.1.1.2.1-3.

A.3.6.1.3.2.2	Carrier aggregation tests

All carrier aggregation tests are performed using the antenna connection in clause A.3.6.1.3.2.4 for the PCell antenna connection if the PCell is on a band where 2RX is supported and either 4RX or 6Rx is not supported, or using the antenna connection in clause A.3.6.1.3.2.5 for the PCell antenna connection if the PCell is on a band where 4RX is supported and 6Rx is not supported, or using the antenna connection in clause A.3.6.1.3.2.6 for the PCell antenna connection if the PCell is on a band where 6Rx is supported.

All carrier aggregation tests are performed using the antenna connection in clause A.3.6.1.3.2.4 for the SCell antenna connection if an SCell is on band where 2RX is supported and either 4RX or 6Rx is not supported, or using the antenna connection in clause A.3.6.1.3.2.5 for the SCell antenna connection if an SCell is on a band where 4RX is supported and 6Rx is not supported, or using the antenna connection in clause A.3.6.1.3.2.6 for the SCell antenna connection if an SCell is on a band where 6Rx is supported.

A.3.6.1.3.2.3	EN-DC tests

All EN-DC tests are performed using the antenna connection in clause A.3.6.1.3.2.7 for the PCell antenna connection if the PCell is on a band where 2RX is supported and either 4RX or 6Rx is not supported, or using the antenna connection in clause A.3.6.1.3.2.8 for the PCell antenna connection if the PCell is on a band where 4RX is supported and 6Rx is not supported, or using the antenna connection in clause A.3.6.1.3.2.9 for the PCell antenna connection if the PCell is on a band where 6Rx is supported.

All EN-DC tests are performed using the antenna connection in clause A.3.6.1.3.2.4 for the PSCell or SCell antenna connection if an SCell or PSCell is on band where 2RX is supported and either 4RX or 6Rx is not supported, or using antenna connection in clause A.3.6.1.3.2.5 for the PSCell or SCell antenna connection if an SCell or PSCell is on a band where 4RX is supported and 6Rx is not supported, or using antenna connection in clause A.3.6.1.3.2.6 for the PSCell or SCell antenna connection if an SCell or PSCell is on a band where 6Rx is supported.

A.3.6.1.3.2.4	Antenna connection for bands where 2RX is supported

For bands where 2RX is supported and either 4RX or 6Rx is not supported, it is left to the UE declaration and antenna port configuration to decide which 2 of the 6 Rx ports are connected with data source from system simulator. The remaining 6 RX ports shall be connected with zero input. No test parameters or requirements are modified.

A.3.6.1.3.2.5	Antenna connection for bands where 4RX is supported

For bands where 4RX is supported and 6Rx is not supported, it is left to the UE declaration and antenna port configuration to decide which 4 of the 6 Rx ports are connected with data source from system simulator. The remaining 4 RX ports shall be connected with zero input. Except for the modifications to radio link monitoring thresholds and beam failure detection thresholds described in clause A.3.6.1.1.2.1, no test parameters or requirements are modified.

A.3.6.1.3.2.6	Antenna connection for bands where 6Rx is supported

For bands where 6Rx is supported, all 6 RX antennas are connected with data source from system simulator. The system simulator shall provide independent noise and fading (low correlation) for each antenna port. Except for the modifications to radio link monitoring thresholds described in clause A.3.6.1.1.2.1, no test parameters or requirements are modified.

A.3.6.1.3.2.7	EN-DC LTE Antenna connection for bands where 2RX is supported

For E-UTRAN bands where 2RX is supported and 4RX or 6Rx is not supported, it is left to the UE declaration and antenna port configuration to decide which 2 of the 6 Rx ports are connected with data source from system simulator. The remaining 6 RX ports shall be connected with zero input. No test parameters or requirements are modified.

A.3.6.1.3.2.8	EN-DC LTE Antenna connection for bands where 4RX is supported

For bands E-UTRAN where 4RX is supported and 6Rx is not supported, it is left to the UE declaration and antenna port configuration to decide which 4 of the 6 Rx ports are connected with data source from system simulator. The remaining 4 RX ports shall be connected with zero input. Except for the modifications to radio link monitoring thresholds described in clauses A.3.8.1.2.1 and A.3.8.1.2.2 of TS 36.133 [15], no test parameters or requirements are modified.

A.3.6.1.3.2.9	EN-DC LTE Antenna connection for bands where 6Rx is supported

For bands E-UTRAN where 6Rx is supported, all 6 RX antennas are connected with data source from system simulator. The system simulator shall provide independent noise and fading (low correlation) for each antenna port. Except for the modifications to radio link monitoring thresholds described in clauses A.3.8.1.2.1 and A.3.8.1.2.2 of TS 36.133 [15], no test parameters or requirements are modified.

<End of Change 1>