3GPP TSG-RAN WG4 Meeting #115	R4-2506774

Malta, MT, 19 – 23 May, 2025

Title: 	(NR_DualTxRx_MUSIM-Core) Discussion on MUSIM gaps without priority

Source: 	Huawei, HiSilicon

Agenda item:	5.25.3

Document for:	Discussion

Introduction

RRM core requirements for MUSIM gaps have been defined in R18. RAN2#129-bis sent an LS [1] with following action.

On the collision between the MUSIM Gap and Measurement Gap (for the UE supporting musim-GapPriorityPreference capability) in R18, RAN2 identified that the network can configure the MUSIM gap without priority and the measurement Gap with priority together and vice versa. In addition, it is also supported from RAN2 specification that the network can configure the MUSIM gap without priority and the measurement Gap without priority together. RAN2 respectfully ask RAN4 on whether UE requirement is needed for the above collision cases.

In this paper, we will provide our views on handling of MUSIM gaps without priority.

Discussion

Based on following statements in clause 9.1.10.2, it is clear that RAN4 requirements are defined assuming all periodic MUSIM gaps will be configured with priorities.

Priorities are applied for each periodic MUSIM gap. A UE shall request priorities for all requested periodic MUSIM gaps when the UE requests MUSIM gaps [2]. The UE shall request different priorities for each periodic MUSIM gap. Priorities of the configured MUSIM gaps will be configured. Theconfigured priorities may differ from the priorities requested by the UE. The MUSIM requirements apply if the configured MUSIM gap priorities retain the same relative priorities among MUSIM gaps as requested by the UE.

This is reflected in the handling of collision between MUSIM gaps in clause 9.1.10.4.

When “keep solution” in clause 9.1.10.3 is not configured: - the periodic MUSIM gap occasion colliding with an aperiodic MUSIM gap are dropped, and - collisions between periodic MUSIM gap occasions are resolved based on the assigned MUSIM gap priorities. Collisions are resolved sequentially in order of decreasing priority, starting with the gap that has the highest priority. For each collision, the occasion of the MUSIM gap with higher priority shall be kept and the occasion of the MUSIM gap with lower priority shall be dropped.

This is also reflected in the handling of collision between MUSIM gaps and measurement gaps in clause 9.1.10.5.

Collisions between MUSIM gaps and measurement gaps configured via GapConfig-r17 with assigned priority but without preConfigInd-r17 or ncsgInd-r17 are resolved based on their assigned priorities: …… Collisions between MUSIM gaps and measurement gaps configured via GapConfig or configured via GapConfig-r17 without assigned priority are handled based on MGRP of the colliding gaps: ……

On the other hand, based on [1] it is possible that NW can configure the MUSIM gap without priority no matter if priority is configured for measurement gaps or not. Table 1 lists all 4 cases regarding priority configuration for MUSIM gaps and measurement gaps.

Table 1: Cases regarding priority configuration for MUSIM gaps and measurement gaps

| Case No. | Priority for MUSIM gap | Priority for measurement gap |
| --- | --- | --- |
| 1 | Y | Y |
| 2 | Y | N |
| 3 | N | Y |
| 4 | N | N |

Based on clause 9.1.10.5, case 1 and 2 are supported by existing RAN4 requirements, while case 3 and 4 are not. In addition, RAN4 has not considered handling of collision between MUSIM gaps without priority in clause 9.1.10.4.

We see 2 options for RAN4 to address RAN2 LS:

- Option 1: Define requirements for cases where MUSIM gaps are not configured with priority. The most straightforward way is to re-use the MGRP based approach in clause 9.1.10.5.

- Option 2: Clarify that collision handling for case where MUSIM gaps are not configured with priority is left to UE implementation, and requirements in clause 9.1.10 do not apply when such MUSIM gaps collide with other gaps.

We do not have strong view, but considering R18 has been closed for long time, we slightly prefer option 2, i.e. not to define new requirement for UE.

Proposal: Clarify that collision handling for case where MUSIM gaps are not configured with priority is left to UE implementation, and requirements in clause 9.1.10 do not apply when such MUSIM gaps collide with other gaps.

Conclusions

In this paper we provided our views on handling of MUSIM gaps without priority.

Proposal: Clarify that collision handling for case where MUSIM gaps are not configured with priority is left to UE implementation, and requirements in clause 9.1.10 do not apply when such MUSIM gaps collide with other gaps.

Reference

- R2-2503163, LS on Collision between MUSIM Gap and Measurement Gap, RAN2