3GPP TSG-RAN WG4 Meeting #115	R4-2506781

Malta, MT, 19 – 23 May, 2025

Title: 	Discussion on RRM test cases for less than 5MHz for NTN

Source: 	Huawei, HiSilicon

Agenda item:	7.10.4.5

Document for:	Discussion

Introduction

RRM performance requirements for less than 5MHz for NTN are discussed in RAN4#114-bis, and the outcomes are captured in [1]. Based on [1] the following issues need to be further discussed.

- Test case list

In this paper, we will provide our views on RRM test cases for less than 5MHz for NTN.

Discussion

Issue 2-2-2: Test cases to verify the core requirements reusing NTN Agreement: - Not define the new test cases for the following requirements RRM requirements; NTN requirements in TS38.133 v18.5.0. RRC connection mobility control; 6.2C RRC Connection Mobility Control for Satellite Access. UE Tx timing, MTTD/ MRTD, timer accuracy, TA accuracy; 7.1C, 7.2C, 7.3C. - Further discuss whether to define the new test case for the following requirement IDLE/inactive mode mobility; 4.2C Cell Re-selection for NR UE for Satellite Access.

We support to define test case for cell reselection. Similar to R18 WI to support less than 5MHz in TN, there is no core requirement change in cell reselection, but this is because the exact delay for reading MIB of the target cell is not defined in core part but in test cases, i.e. captured as part of TSI.

For this reason, UE is not supposed to pass the existing test case for cell reselection with the target cell has 12 RB SSB. Instead, in R18 a new cell reselection test case with increased TSI is defined for less than 5MHz in TN. It is straightforward to follow the same principle for NTN.

Proposal 1: Define test case for cell reselection for less than 5MHz in NTN.

Issue 2-2-3 Test cases to verify the new core requirements Agreement: - Define the new test cases for the core requirements for NTN less than 5MHz introduced in Rel19.e.g. - HO-1: Intra-frequency HO, [unknown cell] - CHO-1: [Inter-frequency CHO], time based with L3 measurement - SatSwitch-1: Hard switch - RLM-1: RLM OOS for 15RB - RLM-2: RLM IS for 15RB - BFR-1: BFR - MR-1: Intra-frequency with SSB index reading - [MR-2: Inter-frequency without SSB index reading] - FFS the detailed configuration for each test case

We support to define all test cases in Issue 2-2-3 and the related setup, except for the last one MR-2: Inter-frequency without SSB index reading. In RAN4#114-bis, it is agreed to define different extended delay for SSB index reading for intra- and inter-frequency measurement based on different Es/Iot conditions. Then it is meaningful to test SSB index reading for both intra- and inter-frequency, so test case MR-2 should be with SSB index reading rather than without.

Proposal 2: Define the following test cases for less than 5MHz in NTN:

- HO-1: Intra-frequency HO, unknown cell

- CHO-1: Inter-frequency CHO, time based with L3 measurement

- SatSwitch-1: Hard switch

- RLM-1: RLM OOS for 15RB

- RLM-2: RLM IS for 15RB

- BFR-1: BFR

- MR-1: Intra-frequency with SSB index reading

- MR-2: Inter-frequency with SSB index reading

Conclusions

In this paper we provided our views on RRM test cases for less than 5MHz for NTN.

Proposal 1: Define test case for cell reselection for less than 5MHz in NTN.

Proposal 2: Define the following test cases for less than 5MHz in NTN:

- HO-1: Intra-frequency HO, unknown cell

- CHO-1: Inter-frequency CHO, time based with L3 measurement

- SatSwitch-1: Hard switch

- RLM-1: RLM OOS for 15RB

- RLM-2: RLM IS for 15RB

- BFR-1: BFR

- MR-1: Intra-frequency with SSB index reading

- MR-2: Inter-frequency with SSB index reading

Reference

- R4-2504896, WF on RRM requirements for NR_IoT_NTN_req_test_enh, Moderator (Xiaomi)