3GPP TSG-RAN WG4 Meeting #115	R4-2506790

Malta, MT, 19 – 23 May, 2025

Title: 	Discussion on RRM requirements for LP-WUR

Source: 	Huawei, HiSilicon

Agenda item:	7.28.5.2

Document for:	Approval

Introduction

RRM requirements related to LP-WUR are discussed in RAN4#114-bis, and the outcomes are captured in WF [1]. Based on [1] the following issues need further discussion.

- General aspects

- LP-WUR measurement

- MR measurement

- LP-WUR at CONNECTED mode

In this paper, we will provide our views on RRM requirements related to LP-WUR.

Discussion

General aspects

Issue 1-1-9: LP-WUR status at legacy case (not at LP-WUS monitoring case/fully offloading(case 1) case/RRM relaxation (case 3) case)

In RAN4#114 it is agreed that at legacy state, when and how to turn on LR for serving cell measurement is up to UE implementation, but it is FFS whether to define requirements for LR measurement.

We do not support to define LR measurement requirements at legacy state. We understand this is a transient period where LR measurement is needed only for evaluation of the entry condition, and this is quite different from Case 1 or Case 3 where LR measurement is for serving cell evaluation that is important for mobility. In other words, we do not see much significance to define the requirements. Functional wise, UE must evaluate the LR based criterion if configured by NW for the entry condition. This evaluation is not time critical, and it can be up to UE to decide how fast it would like to complete the evaluation as it only impacts when UE can get the power saving gain.

In addition, at legacy state MR measurement is ongoing without relaxation, and it may be difficult to define LR measurement requirements considering UE is not assumed to simultaneously operate LR and MR.

Finally, it would be difficult to verify this requirement via test case since when and how to turn on LR for serving cell measurement is up to UE implementation.

Proposal 1: RAN4 not to define LR measurement requirements before entering or after exiting LP-WUS monitoring or LR measurement offloading.

Issue 1-1-10: Considerations on higher priority frequency layer

In RAN4#114-bis, it is agreed to support relaxed higher priority frequency layer search at Case 1. The details for the requirements and related conditions were also discussed but without conclusion. One issue identified during the discussion is the applicability of existing threshold for inter-frequency and inter-RAT neighbour cell measurement (SnonIntraSearchP and SnonIntraSearchQ).

In legacy requirements,

- When serving cell is below SnonIntraSearchP and SnonIntraSearchQ, UE would measure all configured inter-frequency and inter-RAT carriers. The requirements are same for all carriers regardless of their priority, and are based on DRX cycle.

- When serving cell is above SnonIntraSearchP and SnonIntraSearchQ, UE would measure only measure carriers with higher priority, and the requirements are based on 60s.

Considering Case 1, there can be 2 cases

- Case A: existing threshold for inter-frequency and inter-RAT neighbour cell measurement are same or lower than the threshold for entry condition of Case 1

- Case B: existing threshold for inter-frequency and inter-RAT neighbour cell measurement are higher than the threshold for entry condition of Case 1

The 2 cases and corresponding UE measurement requirements are shown in Figure 1.

Figure 1: UE measurement requirements for Case A and Case B

For Case A UE measurement requirements are clear. However, for Case B there are conflicts when serving cell is below existing threshold but above threshold for entry condition of Case 1, i.e. when UE is within the red cycle but outside the green cycle. In particular, following the existing threshold, UE would not measure serving cell but still measure neighbour cell with equal or lower priority, which is unreasonable. Besides, as UE is not measuring serving cell after in red cycle, it cannot determine whether it is within or outside the green cycle. Therefore, Case B is not a reasonable scenario and should not be considered in the requirements.

With only Case A considered for Case 1, the existing relaxed requirements 60*Thigher_priority_search can be re-used for MR measurement on higher priority carriers.

Proposal 2: For Case 1, existing relaxed requirements 60*Thigher_priority_search are re-used for MR measurement on higher priority carriers.

There are at least 2 ways to exclude Case B in the requirements:

- Option 1: define the relation between SnonIntraSearchP/SnonIntraSearchQ and threshold for entry condition of Case 1, e.g. in R16 power saving WI, RAN2 has defined the relation between the threshold for ‘not at cell edge’ and SIntraSearchP/SnonIntraSearchP and SIntraSearchQ/SnonIntraSearchQ.

- Option 2: clarify that SnonIntraSearchP and SnonIntraSearchQ are not applicable in Case 1.

In our view, it should be up to RAN2 to decide whether to define relation between existing threshold and threshold for entry condition of Case 1, or to clarify the applicability of existing threshold in Case 1, since either option will have impact in RAN2 spec.

It is noted that Case A and B in Figure 1 are defined for existing threshold for inter-frequency and inter-RAT measurement, but existing threshold for intra-frequency measurement (SIntraSearchP and SIntraSearchQ) also need to be discussed, as there is conflict on whether UE should perform intra-frequency neighbour cell measurement in Case 1.

Besides, we believe RAN2 also needs to discuss the applicability of existing threshold in Case 3, or whether there is any relation between threshold for entry condition of Case 3 and existing threshold. This will impact RAN4 requirements, i.e. whether to define different requirements based on existing threshold in Case 3.

Proposal 3: RAN4 to ask RAN2 to decide whether to define relation between existing threshold for neighbour cell measurement and threshold for entry condition of Case 1 and 3, or to clarify the applicability of existing threshold in Case 1 and 3.

A draft for the LS is provided in the Annex.

Issue 1-1-15 RedCap UE capability

In RAN4#114-bis some companies proposed to define LP-WUR related requirements for RedCap UE. We support this proposal, as RedCap UE is also an important use case for LP-WUR. RedCap UE is already considered during SI and in RAN1 agreement for WI, e.g. the following agreement from RAN1#120-bis.

Agreement For RRC idle/inactive, support Alt 1 for both Redcap and non-redcap UEs: - Alt 2: LP-WUS/LP-SS frequency resource can be out of initial DL BWP where UE receives paging but within the same carrier. - FFS whether to define the maximum frequency gap between LP-WUS/LP-SS and SSB

As RedCap and non-RedCap UE differ in MR capability, the LR requirements should be same for RedCap and non-RedCap UE. The MR requirements should be based on R17 RedCap requirements, and be adapted for LP-WUR. We understand the conclusions so far, e.g. the relaxation factor and the evaluation of entry condition, are directly applicable for RedCap UEs.

Proposal 4: RAN4 to define LP-WUR related requirements for RedCap UE.

LP-WUR measurement

Issue 1-2-1-1: Detail on LR accuracy and side conditions requirements

In RAN4#114-bis, the target accuracy for LR measurement is agreed with the RF margin as [2 or 2.5] dB. We suggest to take 2.5 dB because it is the assumption for MR. For example, in [2] it is clearly captured that 2.5 dB is the RF margin to define L1-RSRP accuracy in FR1.

Proposal 5: Take [2.5]dB as the RF impairment margin for LR.

Issue 1-2-2-1: Lower bound on LP-SS measurement periodicity

We support to define lower limit for LP-SS measurement periodicity. RAN4#112-bis agreed that LP-SS measurement periodicity shall be defined based on LP-SS periodicity. In our view, the agreement was made based on the assumption of LP-SS periodicity equalling to 320ms. However, RAN1#120-bis has agreed to support LP-SS periodicity of 160ms, and 80ms is still under discussion.

In our view, smaller LP-SS periodicity like 80ms or 160ms is targeting for sync for LP-WUS decoding, but the LR measurement does not have to be so frequent. As discussed below for SSB measurement periodicity, even the measurement periodicity is LO periodicity, the impact to user experience is still acceptable. On the other hand, too frequent measurement will compromise the power saving gain of measurement offloading.

Proposal 6: Define lower bound for LP-SS measurement periodicity as 320ms.

Issue 1-2-2-2: Upper bound on SSB-based LP-WUR measurement periodicity

In RAN4#113, it was agreed that the periodicity for SSB based LP-WUR measurement delay requirements is based on LO periodicity, no matter if LP-SS is configured or not. One remaining issue is whether an upper bound is needed for the measurement periodicity. It is essentially a trade-off between power saving gain and how fast UE can exit LR when the condition for LR is not met.

Assuming the measurement period for LR measurement is n samples, and m measurements are needed for determining the exit condition is met (m=1 or m>1 is discussed in Issue 1-2-3), the total time between the time point when the channel condition gets insufficient for LR to the time point when UE determines to exit LR would be

Texit = n * Tsample + (m-1) * Tsample

where Tsample is the measurement periodicity. Assuming no upper limit is defined, UE may miss (n+m-1) LP-WUS since UE may not be able to decode LP-WUS under the channel condition. We understand n and m will be in the range of {1,2,3,4}, and it should not be big issue to user experience considering the probability of actual paging is low. Therefore, we suggest not to define upper limit for the measurement periodicity.

Proposal 7: Measurement periodicity for SSB is the LO periodicity, i.e. no upper limit is defined.

Besides, in RAN1#120-bis an LS [3] is sent to RAN4 with the following action.

ACTION: RAN1 respectfully asks RAN4 to confirm the RAN1 agreement that SMTC window is not applicable for LP-SSS-RSRP/RSSI measurement performed by OFDM-based LP-WUR for the serving cell.

In our view, RAN1 agreement can be confirmed as

- SMTC is configured for cell reselection measurement which is performed by MR

- LR is measuring serving cell only and the SSB location is known to the UE

- The LR requirements are based on LO periodicity which is independent of SMTC

Proposal 8: RAN4 to confirm that SMTC window is not applicable for measurement performed by OFDM-based LP-WUR for the serving cell.

Issue 1-2-4: On LR measurement/evaluation requirements

Issue 1-2-4-0: On LR measurement requirement

In RAN4#114 following is agreed regarding the LR evaluation requirements.

Issue 1-2-4-2-3: On how to define LR evaluation requirements Agreement: LR evaluation duration is [x1 samples]*LP-SS (for OOK LR) or [y1 samples] *LO (for SSB LR), assuming x or y samples are used to satisfy accuracy requirement and x1 > x and y1>y. Number x and y are defined as measurement requirement as [x samples]*LP-SS (for OOK LR) or [y samples] *LO (for SSB LR)

The common understanding behind the agreement was to define LR requirements similar to existing MR requirements for neighbour cell. It is noted that in existing requirements for MR neighbour cell (as copied below), both Tevaluate and Tmeasure are defined, so we support to define Tmeasure.

The UE shall measure SS-RSRP and SS-RSRQ at least every Tmeasure,NR_Intra (see table 4.2.2.3-1, table 4.2.2.3-2, table 4.2.2.3-3, table 4.2.2.3-4 or table 4.2.2.3-5) for intra-frequency cells that are identified and measured according to the measurement rules. The UE shall filter SS-RSRP and SS-RSRQ measurements of each measured intra-frequency cell using at least 2 measurements. Within the set of measurements used for the filtering, at least two measurements shall be spaced by at least Tmeasure,NR_Intra/2.

On the two options to define LR measurement requirements, we support option 1, i.e. define the minimum interval where one LR measurement shall be executed. This is same as existing requirements, and it defines how frequent LR should perform serving cell measurement. It would ensure that UE would not evaluate LR threshold based on a single measurement which is unreliable due to channel fading.

As to option 2, i.e. define the minimum duration to meet accuracy requirement, we do not see much value because it effectively defines with how many samples UE must be able to get an accurate measurement result. It is a consideration factor in defining Tevaluate, but does not need to be defined explicitly. It is noted that it is not defined in existing requirements either. It is clear that UE is not expected to meet the accuracy (defined as reselection margin) with single measurement occasion, but exactly how many samples are needed is not specified.

Proposal 9: Define LR measurement requirement as the minimum duration where one LR measurement shall be executed.

Issue 1-2-4-1: On LR measurement filtering requirement

In RAN4#114-bis, filtering requirements are agreed for LR measuring SSB. It is open whether similar filtering requirements are needed for LP-SS measurement. In our view, this is not needed based on the RAN1 agreement so far.

RAN1 has agreed to support 160ms and 320ms LP-SS periodicities, so the minimum interval with which UE can measure LP-SS is 160ms. It is either equal to the assumed measurement interval (in case no lower bound is defined) or half of the assumed measurement interval (in case lower bound is defined as 320ms). In either case, filtering requirements are not needed. If RAN1 introduces 80ms LP-SS periodicity and RAN4 agrees to define a lower bound, this issue can be revisited.

Proposal 10: Do not define filtering requirement for LR measuring LP-SS. This can be revisited if RAN1 introduces LP-SS periodicity lower than 160ms.

Issue 1-2-4-2-3: On how to define LR evaluation requirements

In RAN4#114 the framework of the LR evaluation requirements has been agreed, and the remaining issue is to decide on the exact number of x1 and y1 based on x and y which are determined via simulation.

According to our simulation results, x = y = 2 should be sufficient to meet the target BB accuracy of 3.5dB for both LP-SS (including all {M,L} combinations) and SSB, and we suggest to define x1 = y1 = 4 which allows UE to use multiple measurements for the evaluation.

Proposal 11: Define LR evaluation requirement based on x1 = y1 = [4].

MR measurement

Issue 1-2-6: On MR wake up delay related requirements for LP-WUR and RAN1 LS R1-2501624

In RAN4#114-bis, the number of SSB samples for MR wake up delay are discussed but without conclusion. We suggest to define the number as [3 3 3] based on side condition of -3dB for LR.

- At least one SSB sample is needed for AGC. It is noted that the Rx dynamic range may be quite different for LR and MR, so MR may not be able to provide valid AGC information for MR.

- At least one SSB sample is needed for coarse sync. It is noted that the time and frequency information from LR may not be enough for MR to perform fine sync. In particular, the frequency error of LR is 5 or 10 ppm, which is much larger than the 0.1 ppm assumed for MR.

- At least one SSB sample is needed for fine sync. After obtaining the coarse sync, fine sync is needed to get ready for PDCCH/PDSCH reception. This is same as existing RRM procedure e.g. HO.

Use of 1 sample for each of coarse and fine sync needs to be based on certain side conditions. If the Es/Iot is too low, more SSB samples may be needed. In RAN4#114-bis it is agreed to define LR requirements based on -3dB Es/Iot. We think this condition can also be re-used for the MR wake up. Based on RF conclusion, the NF of MR will be at least 4.5dB better than LR, so the Es/Iot observed by MR would be at least 1.5dB. 1 sample should be enough for coarse and fine sync at this condition.

It is noted that the SSB sample number in RAN1 WA is [3 5 5], i.e. more samples are needed for ultra-deep sleep mode. Based on our analysis above, we do not see strong reason for more samples with ultra-deep sleep assuming LR can provide time and frequency sync to MR.

Proposal 12: RAN4 confirms the number of SSB samples for sync is [3 3 3].

Issue 1-2-8-1: Accuracy for normal or relaxed MR serving cell measurement

Issue 1-2-8-2: Accuracy for relaxed MR neighbour cell measurement

We support to re-use the legacy MR accuracy requirements for MR measurement with relaxation, for both serving and neighbour cell. Accuracy requirements do not depend on the measurement interval, i.e. same accuracy requirements apply regardless of whether the measurement is performed once per DRX cycle or per N DRX cycles. Here, by legacy MR accuracy requirements we mean the accuracy in clause 10 for CA/DC Idle Mode Measurements. We are also open to discuss other existing accuracy numbers in RRC_IDLE or RRC_INACTIVE.

Proposal 13: Legacy MR accuracy requirements in RRC_IDLE or RRC_INACTIVE are re-used for MR measurement with relaxation, for both serving and neighbour cell.

LP-WUR at CONNECTED mode

Issue 1-4-1: LP-WUR at CONNECTED mode

In RAN4#114-bis, the potential RRM requirements for LP-WUS monitoring in CONNECTED mode are discussed. One particular issue raised by some companies is the interruption to LP-WUS monitoring due to MR operation e.g. BWP switching or measurement within MG.

It is noted that RAN2#129-bis has sent LS [4] to RAN1 with following question. We suggest RAN4 to wait for RAN1 conclusion before discussing the potential RRM impacts.

RAN2 discussed whether there are any scenarios on when the UE would not be able to monitor LP-WUS and made the following working assumption: Working assumption for the case of potential collision (if any): In Option 1-1, when the UE is not able to monitor the LP-WUS occasion(s) the UE should start the drx-OnDurationTimer (as if LP-WUS was detected). FFS for Option 1-2. Based on this working assumption, RAN2 would like to ask RAN1: 1. Whether there are any cases/scenarios on when the UE is not able to monitor LP-WUS? - RAN2 would like to note that for DCP, such cases/scenarios included collision with Active Time, measurement gap, interruption caused by BWP switching, and RAR window monitoring for BFR. 2. Is there any case when the UE is not able to monitor LR and MR simultaneously?

Proposal 14: RAN4 to postpone the discussion on RRM impacts of LP-WUR at CONNECTED mode until more RAN1/2 conclusions are available.

Conclusions

In this paper we provided our views on RRM requirements related to LP-WUR.

Proposal 1: RAN4 not to define LR measurement requirements before entering or after exiting LP-WUS monitoring or LR measurement offloading.

Proposal 2: For Case 1, existing relaxed requirements 60*Thigher_priority_search are re-used for MR measurement on higher priority carriers.

Proposal 3: RAN4 to ask RAN2 to decide whether to define relation between existing threshold for neighbour cell measurement and threshold for entry condition of Case 1 and 3, or to clarify the applicability of existing threshold in Case 1 and 3.

Proposal 4: RAN4 to define LP-WUR related requirements for RedCap UE.

Proposal 5: Take [2.5]dB as the RF impairment margin for LR.

Proposal 6: Define lower bound for LP-SS measurement periodicity as 320ms.

Proposal 7: Measurement periodicity for SSB is the LO periodicity, i.e. no upper limit is defined.

Proposal 8: RAN4 to confirm that SMTC window is not applicable for measurement performed by OFDM-based LP-WUR for the serving cell.

Proposal 9: Define LR measurement requirement as the minimum duration where one LR measurement shall be executed.

Proposal 10: Do not define filtering requirement for LR measuring LP-SS. This can be revisited if RAN1 introduces LP-SS periodicity lower than 160ms.

Proposal 11: Define LR evaluation requirement based on x1 = y1 = [4].

Proposal 12: RAN4 confirms the number of SSB samples for sync is [3 3 3].

Proposal 13: Legacy MR accuracy requirements in RRC_IDLE or RRC_INACTIVE are re-used for MR measurement with relaxation, for both serving and neighbour cell.

Proposal 14: RAN4 to postpone the discussion on RRM impacts of LP-WUR at CONNECTED mode until more RAN1/2 conclusions are available.

Reference

- R4-2504908, WF on RRM requirements for NR_LPWUS, Moderator (vivo)

- R4-1904820, Way forward on L1-RSRP accuracy requirements, Ericsson, Nokia, CMCC

- R1-2503103, LS on the RRM measurement metrics for OFDM-based LP-WUR, RAN1

- R2-2503187, Draft LS on LP-WUS in RRC_CONNECTED, RAN2

Annex: draft LS on existing threshold for neighbour cell measurement

3GPP TSG-RAN WG4 Meeting #115 R4-250xxxx Malta, MT, 19 – 23 May, 2025 Title: LS on existing threshold for neighbour cell measurement Response to: Release: Release 19 Work Item: NR_LPWUS-Core Source: RAN4 To: RAN2 Cc: Contact Person: #### Name: Li Zhang - E-mail Address: zhangli164@huawei.com Attachments: - 1. Overall Description: RAN4 is discussing requirements for MR measurement higher priority carriers when UE is in RRM relaxation (Case 3) or RRM offloading mode (Case 1). RAN4 finds it unreasonable to apply existing threshold for intra-frequency neighbour cell measurement (SIntraSearchP and SIntraSearchQ) and inter-frequency and inter-RAT neighbour cell measurement (SnonIntraSearchP and SnonIntraSearchQ) in Case 1, if the existing thresholds are higher than the threshold for entry condition of Case 1. Besides, RAN4 requirements for Case 3 are depending on relation between existing thresholds for neighbour cell measurement and threshold for entry condition of Case 3, or whether existing thresholds for neighbour cell measurement apply in Case 3. RAN4 kindly asks RAN2 to decide whether to define relation between existing thresholds for neighbour cell measurement and thresholds for entry condition of Case 1 and 3, or to clarify the applicability of existing thresholds for neighbour cell measurement in Case 1 and 3. 2. Actions: To RAN2: RAN4 kindly asks RAN2 to decide whether to define relation between existing thresholds for neighbour cell measurement and thresholds for entry condition of Case 1 and 3, or to clarify the applicability of existing thresholds for neighbour cell measurement in Case 1 and 3. 3. Date of Next TSG-RAN4 Meetings: RAN WG4 Meeting #116 August 25 – August 29, 2025 India (TBC), IN RAN WG4 Meeting #116-bis October 13 – October 17, 2025 Prague, CZ