3GPP TSG-RAN WG4 Meeting #115	R4-2506792

Malta, MT, 19 – 23 May, 2025

Title: 	Discussion on test case for LP-WUR

Source: 	Huawei, HiSilicon

Agenda item:	7.28.6

Document for:	Discussion

Introduction

RRM requirements related to LP-WUR are discussed in RAN4#114-bis, and the outcomes are captured in WF [1]. Based on the work plan, RAN4 should start the performance part of the WI in this meeting.

In this paper, we will provide our views on RRM test cases LP-WUR.

Discussion

The RRM core requirements for the WI mainly include

- LR requirements for evaluation of entry conditions for LP-WUS monitoring, Case 1 and 3

- LR requirements for evaluation of exit conditions for LP-WUS monitoring, Case 1 and [3]

- MR requirements for evaluation of entry conditions for LP-WUS monitoring, Case 1 and 3

- [MR requirements for evaluation of exit conditions for 3]

- MR requirements for evaluation of serving cell against S criterion with relaxation

- MR requirements for evaluation of neighbor cell for cell reselection with relaxation

- MR wake up delay

First, we suggest RAN4 not to define dedicated test cases for evaluation of entry conditions, either with MR or LR. Based on RAN1/2 agreements, UE is only allowed to enter LP-WUS monitoring, to relax the MR RRM measurement or to offload the RRM measurement to LR when the corresponding entry conditions are met, but not required to perform those actions. It is thus unnecessary to verify whether and when UE is performing those actions when the entry conditions are met.

Proposal 1: Do not define dedicated test cases for evaluation of entry conditions.

On the other hand, it is necessary to verify whether UE exits LP-WUS monitoring, Case 1 and 3 timely and fall back to PO monitoring and legacy MR measurement, based on LR measurement. We suggest to define 3 test cases for evaluation of exit condition:

- TC1: UE exist Case 1 to legacy

- TC2: UE exist Case 3 to legacy

- TC3: UE exist Case 1 to Case 3

In TC1, LP-WUS monitoring is configured with same entry and exit condition as Case 1. In TC2 and 3, LP-WUS monitoring is configured with same entry and exit condition as Case 3.

Proposal 2: Define test cases for evaluation of exit conditions for the following cases.

- Case 1: UE exist Case 1 to legacy

- Case 2: UE exist Case 3 to legacy

- Case 3: UE exist Case 1 to Case 3

LR measurement requirements are defined for LP-SS (applicable for OOK based LR and OFDM based LR) and SSB (applicable for OFDM based LR). It is meaningful to define separate sets of test cases for the 2 RS, one set with LP-SS and the other without LP-SS. In order to make sure UE is not using SSB in the test cases with LP-SS, TE should only configure LP-RSRP/RSRQ based threshold. It is noted that RAN2#129-bis agreed that “Network is allowed to provide either OOK based threshold or OFDM based WUR measuring SSB threshold or both”.

Proposal 3: Separate test cases for evaluation of exit conditions into LP-SS based and SSB based.

Besides TCs for evaluation of exit conditions, we suggest to define one test case to verify the MR wake up delay when UE receives LP-WUS. In TCs for evaluation of exit conditions, the RSRP of the serving cell would change to enable UE to enter and then exit the LR mode, but in the TC for MR wake up delay, the RSRP of the serving cell is not changed, rather UE is woken up by LP-WUS. The test will verify UE can receive paging after the wake up delay from the LP-WUS.

Proposal 4: Define one test case to verify the MR wake up delay when UE receives LP-WUS.

Finally, it is also meaningful to define test cases to verify the cell reselection performance when UE is in Case 3. This should be similar to test cases for R16 power saving. We suggest to test intra-frequency, inter-frequency and inter-RAT neighbor cell measurement. In addition, one test case can be defined to verify higher priority measurement performance in Case 1.

Proposal 5: Define 3 test cases for intra-frequency, inter-frequency and inter-RAT cell reselection when UE is in Case 3. Define 1 test case for higher priority cell reselection in Case 1.

Conclusions

In this paper we provided our views on RRM test cases LP-WUR.

Proposal 1: Do not define dedicated test cases for evaluation of entry conditions.

Proposal 2: Define test cases for evaluation of exit conditions for the following cases.

- Case 1: UE exist Case 1 to legacy

- Case 2: UE exist Case 3 to legacy

- Case 3: UE exist Case 1 to Case 3

Proposal 3: Separate test cases for evaluation of exit conditions into LP-SS based and SSB based.

Proposal 4: Define one test case to verify the MR wake up delay when UE receives LP-WUS.

Proposal 5: Define 3 test cases for intra-frequency, inter-frequency and inter-RAT cell reselection when UE is in Case 3. Define 1 test case for higher priority cell reselection in Case 1.

Reference

- R4-2504908, WF on RRM requirements for NR_LPWUS, Moderator (vivo)