3GPP TSG-RAN WG4 Meeting #115	R4-2506793

Malta, MT, 19 – 23 May, 2025

Title: 	Discussion on RRM requirements for RedCap UE in NTN

Source: 	Huawei, HiSilicon

Agenda item:	7.31.5.1

Document for:	Discussion

Introduction

RRM requirements for RedCap UE in NTN are discussed in RAN4#114-bis, and the outcomes are captured in WF [1]. Based on the discussions, RAN4 needs to further discuss the following issues.

- RRM requirements

- Impact of HD-FDD

In this paper, we will provide our views on RRM requirements for RedCap UE in NTN.

Discussion

RRM requirements

Issue 1-2-1-1a: The proximity distance between two SMTCs

<Way Forward>: - When (e)RedCap UE is configured with more than one SMTC on a SAN carrier, two SMTC occasions outside measurement gap are considered colliding if at least one of the following conditions is met: - the two SMTC occasions are fully or partially overlapping in time domain, or - the magnitude of the distance between two SMTC occasions in time domain is less than or equals to Y ms. - Y=5 ms

We support to consider relaxation for proximity condition for parallel measurement of multiple SMTCs for (e)RedCap UE. Due to low complexity, RedCap UE may have lower processing capability compared to normal UE, and it may take longer time to switch from one measurement of SMTC1 to another for SMTC2. We are open to discuss the exact relaxation x.

Proposal 1: Consider relaxation for proximity condition for parallel measurement of multiple SMTCs on a SAN carrier for (e)RedCap UE, e.g. Y=5.

Impact of HD-FDD

Issue 3-1-1: General HD-FDD related impact

Agreement: - The legacy requirements and applicable conditions can be reused as baseline: - Paging reception requirements in RRC_IDLE/ RRC_INACTIVE state - Handover interruption time - Random access - SA: RRC Connection Release with Redirection - [Minimum requirement for L1 indication for RLM and LR] - Scheduling availability of UE performing intra/inter measurements RAN4 to further discuss: - The legacy requirements and applicable conditions can be reused as baseline for defining requirements of RedCap UE in NTN scenario, and discuss whether there is spec impact for the following requirements by HD-FDD: - FFS: Scheduling availability of UE during L1-RSRP measurement - Other requirement is not precluded - Candidate options for the potential spec impact for each of the above requirements (if needed): - Option a: the requirement is extended, e.g., the requirement does not count resources, channels, SSB, etc if they overlap with the other direction as specified a valid case in RAN1 specification. - Option b: Clarify that the requirements for CSI-RS and PRS apply provided that no CSI-RS and PRS resource is dropped due to collision with UL transmission during the measurement period. - Option c: The requirement applies when there is at least one RS within each measuremen interval. - FFS the same or different option for DRX and non-DRX

Issue 3-2-2: The impact on other requirements due to HD-FDD

<Way Forward>: - Proposal 1: Clarify that the measurement requirements apply provided that no RS is overrode by UL transmission during the measurement period. Otherwise, longer delays can be expected. - Proposal 2: For scheduling availability of UE performing intra/inter measurements, the existing principle can be reused, that UE is not expected to transmit PUCCH/PUSCH/SRS on SSB symbols/SMTCs to be measured. - Proposal 3: For scheduling availability of UE during L1-RSRP measurement, update the principle as scheduling restrictions apply for transmission on PUCCH/PUSCH/SRS during the CBD evaluation period, when CBD evaluation is prioritized over UL transmission.

In our view, Issue 3-1-1 and 3-2-2 are addressing same set of requirements.

We support to re-use legacy requirements and applicable conditions for the minimum requirement for L1 indication for RLM and LR, as we do not see any difference between TN and NTN.

Proposal 2: The legacy requirements and applicable conditions for minimum requirement for L1 indication for RLM and LR are re-used.

We do not support to re-use legacy requirements and applicable conditions for scheduling availability of UE during L1-RSRP measurement due to new collision handling rule defined by RAN1 for NTN.

Agreements that are relevant to RRM requirements in our view are copied below.

Conclusion The different use cases (e.g. the collision happening in different channel types or different scheduling cases) from RAN1#118 agreement for collision case 3 (Semi-statically configured DL reception collides with semi-statically configured UL transmission) consist of pairs of one semi-statically configured DL reception and one semi-statically configured UL transmission, among: - Semi-statically configured DL reception includes Type-0/0A/0B/1/2-PDCCH CSS and dedicated semi-statically configured PDCCH USS/PDSCH SPS/CSI-RS/PRS/[Type 3 PDCCH CSS] - Semi-statically configured UL transmission includes semi-statically configured PUSCH/PUCCH/SRS Agreement For Rel-19 HD-FDD (e)Redcap UE in RRC connected mode, for collision case 3, - Handling of collision with Type-0/0A/1/2-PDCCH CSS in RRC-Connected mode is left to UE implementation whether to prioritize UL or prioritize DL with the constraint in the following note. - For other use cases, default priority rule for collision case 3 in RRC-Connected mode is that DL is prioritized. - Network is allowed to indicate UL overriding DL for all the other use cases above - This is signaled by RRC configuration Note: UE shall comply to the following existing procedure in 38.331: UEs in RRC_CONNECTED shall monitor for SI change indication in any paging occasion at least once per modification period if the UE is provided with common search space, including pagingSearchSpace, searchSpaceSIB1 and searchSpaceOtherSystemInformation, on the active BWP to monitor paging, as specified in TS 38.213 [13], clause 13.

It can be seen that there are possibilities that semi-static DL reception of CSI-RS and PRS is de-prioritized compared to UL transmission, e.g. when NW indicates UL overriding DL.

In existing requirements for scheduling availability of UE during L1-RSRP measurement, it is specified that CBD evaluation is prioritized over UL transmission. This may not be always true based on the latest RAN1 agreement. Our suggestion is to remove those 2 paragraphs because the UL/DL prioritization is anyway defined in RAN1 spec.

9.5B.6.1 Scheduling availability of UE performing L1-RSRP measurement with a same subcarrier spacing as PDSCH/PDCCH on FR1 For HD-FDD RedCap UE, scheduling restrictions apply for transmission on PUCCH/PUSCH/SRS during the CBD evaluation period, as CBD evaluation is prioritized over UL transmission. The CBD evaluation period equals the measurement period of TL1-RSRP_Measurement_Period_SSB_RedCap in case L1-RSRP measurement is performed on SSB as defined in clause 9.5B.4.1, or TL1-RSRP_Measurement_Period_CSI-RS_RedCap in case L1-RSRP measurement is performed on CSI-RS and/or SSB as defined in clause 9.5B.4.1. #### 9.5B.6.2 Scheduling availability of UE performing L1-RSRP measurement with a different subcarrier spacing than PDSCH/PDCCH on FR1 - For HD-FDD RedCap UE, scheduling restrictions apply for transmission on PUCCH/PUSCH/SRS during the CBD evaluation period, as CBD evaluation is prioritized over UL transmission. The CBD evaluation period equals the measurement period of TL1-RSRP_Measurement_Period_SSB_RedCap in case L1-RSRP measurement is performed on SSB as defined in clause 9.5B.4.1, or TL1-RSRP_Measurement_Period_CSI-RS_RedCap in case L1-RSRP measurement is performed on CSI-RS and/or SSB as defined in clause 9.5B.4.1.

Proposal 3: Do not re-use legacy requirements and applicable conditions in 9.5B.6.1 and 9.5B.6.2 for L1-RSRP scheduling restriction.

Based on above RAN1 agreement, the measurement period of CSI-RS and PRS based measurement may be impacted if the CSI-RS and PRS resources collide with UL transmission based on actual TA used by the UE because of the punctured measurement occasions. We suggest to go with option b.

For option a, considering that such cases are anyway “semi-error” cases that NW would try to avoid, we do not prefer to spend much efforts to define exact requirements for them. For option c, in non-DRX case, the measurement interval is same as CSI-RS/PRS periodicity, so it is same as option b; for DRX case, it may be possible for UE to find an CSI-RS/PRS occasion not colliding with UL during the DRX cycle when there are multiple CSI-RS/PRS occasions within the DRX cycle, but this would require new UE implementation, e.g. UE needs to re-schedule the measurements based on the availability of CSI-RS/PRS occasions which can be dynamic. Again, as we are addressing a “semi-error” cases that NW would try to avoid, we do not prefer to define requirements that requires new UE implementation.

Proposal 4: Clarify that the requirements for CSI-RS and PRS apply provided that no CSI-RS and PRS resource is dropped due to collision with UL transmission during the measurement period, when NW indicates UL overriding DL for collision handling.

Issue 3-2-1: The impact on NR Handover due to HD-FDD

Agreement: - Option 1: agree on the principle below and further discuss the wording in the CR: - For RACH-based HO, RACH-less HO, Time/location-based CHO without L3 measurement and Satellite switching with re-synchronization, following applicability rules to adapt HD-FDD case should be added: - For RedCap NTN UE with HD-FDD, the requirements are met provided that - SSB is available at the UE once every SMTC period during Tsearch - One SSB is available during T∆ - One SSB is available during TIU - For Time/location-based CHO with L3 measurement, following applicability rules to adapt HD-FDD case should be added: - For RedCap NTN UE with HD-FDD, the requirements are met provided that - One SSB is available during T∆ - One SSB is available during TIU - Option 2: No spec impact

We support option 2. Technically, SSB is always prioritized over UL following RAN1 agreement (Case 5), and during those mobility procedures, we do not see any reason for UE to prioritize UL over SSB. If we define those applicability rules, it may be mis-interpreted as SSB may be dropped due to UL, which is not the intention. The requirements should thus be same as full duplex UE without those applicability rules.

Conclusion For collision cases 1, 2, 5 and 6, the existing priority rules can be reused for a HD-FDD (e)RedCap UE in NTN.

Proposal 5: Adopt option 2 (no spec impact) for HO, CHO and satellite switch.

Conclusions

In this paper we provided our views on RRM requirements for RedCap UE in NTN.

Proposal 1: Consider relaxation for proximity condition for parallel measurement of multiple SMTCs on a SAN carrier for (e)RedCap UE, e.g. Y=5.

Proposal 2: The legacy requirements and applicable conditions for minimum requirement for L1 indication for RLM and LR are re-used.

Proposal 3: Do not re-use legacy requirements and applicable conditions in 9.5B.6.1 and 9.5B.6.2 for L1-RSRP scheduling restriction for HD-FDD UE.

Proposal 4: Clarify that the requirements for CSI-RS and PRS apply provided that no CSI-RS and PRS resource is dropped due to collision with UL transmission during the measurement period, when NW indicates UL overriding DL for collision handling.

Proposal 5: Adopt option 2 (no spec impact) for HO, CHO and satellite switch for HD-FDD UE.

Reference

- R4-2504912, WF on RRM requirements for NR_NTN_Ph3_Part1, Moderator (CATT)