3GPP TSG-RAN WG4 Meeting #115	R4-2506796

Malta, MT, 19 – 23 May, 2025

Title: 	Discussion on performance requirements for Rel-19 NTN

Source: 	Huawei, HiSilicon

Agenda item:	7.31.6

Document for:	Discussion

Introduction

RRM requirements related to Rel-19 NTN are discussed in RAN4#114-bis, and the outcomes are captured in WF [1-2]. Based on the work plan, RAN4 should start the performance part of the WI in this meeting.

In this paper, we will provide our views on RRM performance requirements for Rel-19 NTN.

Discussion

RedCap

One work for the performance part is to define accuracy requirements for measurements that RedCap UE would perform under NTN. Based on accuracy requirements defined in R17 for RedCap and for NTN, it is straightforward to define those listed in Proposal 1.

Proposal 1: RAN4 to define accuracy requirements for following measurements for RedCap in NTN.

- Intra- and inter-frequency RSRP

- Intra- and inter-frequency RSRQ

- Intra- and inter-frequency SINR

- L1-RSRP

- Rx-Tx for NW verified location

During core part discussion, RAN4 has made agreements for accuracy requirements for all measurements listed above, so there is no need for further discussion.

For the test cases, our view is to use the current test cases for NTN in A.14 as the starting point. For 2Rx RedCap UE supporting NTN, the core requirements are almost same as normal UE in NTN, so such a UE should pass the existing test case for NTN. Note this does not mean the existing test cases can be directly re-used for RedCap UE, as the test configuration, some RMC and Io values need to be updated for reduced BW.

On the other hand, RAN4 can check what additional test cases specific to RedCap UE need to be defined based on TN RedCap test cases in A.16.

- One additional aspect is 1Rx UE. In A.16, test cases for 1Rx and 2Rx UEs are defined in separate clauses. RAN4 can discuss whether to re-use this approach or to define a single test case for both 1Rx and 2Rx UE. The latter can help to avoid duplication in spec.

- Positioning test cases are defined for RedCap in A.16. in R18, there was a discussion on whether to define test cases for UE Rx-Tx measurement for NW verified location, and in RAN4#111 it was agreed not to define. In our view, this principle should also apply for RedCap UE.

- CG-SDT test cases are defined for RedCap in A.16. As the test case are mainly verifying UE performs measurement for RSRP change based TA validation which is not applicable for NTN, there is no need to define this test case.

- Inter-RAT LTE measurement and mobility test cases are defined for RedCap in A.16. In NTN only cell reselection requirements are defined for LTE, and in R18 it was agreed not to define test case cell reselection to TN. In our view, this principle should also apply for RedCap UE.

- Test cases for NCD-SSB are defined for RedCap in A.16. For NTN it was agreed not to define specific requirements for NCD-SSB, so those test cases should not be defined.

Based on above discussions, we have the following proposal for RRM test cases for RedCap in NTN.

Proposal 2: Re-use existing test cases for NTN in A.14 for RedCap in NTN

- Adapt the test case for reduced BW, e.g. test configuration, some RMC and Io values

- Further discuss whether to define separate test cases for 1Rx and 2Rx RedCap UEs

DL coverage enhancement

For SSB periodicity extension, RAN4 has defined new RRC re-establishment requirements:

- When SMTC is not provided for an inter-frequency carrier, UE assumes the SMTC period is 160ms rather than 20ms in legacy.

- When serving cell Es/Iot is < -8dB, delay requirements are defined for blind search based on SMTC with period up to 160ms.

In RAN4#114-bis, there was some discussion on whether to define test case for the new requirements. In R15, RAN4 has defined test case in clause A.6.3.2.1.3	for RRC Re-establishment without serving cell timing. In R17, this test case (without serving cell timing) was not defined for NTN. In our understanding, the reason is that RAN4 did not repeat all TN test cases for NTN. As the R19 requirements are dedicated for this case, we suggest RAN4 to define one test case for RRC Re-establishment with 160ms SSB.

Proposal 3: Define test case for RRC Re-establishment with 160ms SSB periodicity and without serving cell based on existing test case in A.6.3.2.1.3.

Besides, RAN4 is discussing potential SMTC enhancement based on RAN2 LS. Depending on the outcome, there may be some updates to the cell reselection and L3 measurement requirements in Connected mode. As this part is still pending, RAN4 should wait for the conclusion from core part discussion.

Proposal 4: Wait for conclusion on potential SMTC enhancement before discussing whether to define new test cases for cell reselection and L3 measurement.

Conclusions

In this paper we provided our views on RRM performance requirements for Rel-19 NTN.

Proposal 1: RAN4 to define accuracy requirements for following measurements for RedCap in NTN.

- Intra- and inter-frequency RSRP

- Intra- and inter-frequency RSRQ

- Intra- and inter-frequency SINR

- L1-RSRP

- Rx-Tx for NW verified location

Proposal 2: Re-use existing test cases for NTN in A.14 for RedCap in NTN

- Adapt the test case for reduced BW, e.g. test configuration, some RMC and Io values

- Further discuss whether to define separate test cases for 1Rx and 2Rx RedCap UEs

Proposal 3: Define test case for RRC Re-establishment with 160ms SSB periodicity and without serving cell based on existing test case in A.6.3.2.1.3.

Proposal 4: Wait for conclusion on potential SMTC enhancement before discussing whether to define new test cases for cell reselection and L3 measurement.

Reference

- R4-2504912, WF on RRM requirements for NR_NTN_Ph3_Part1, Moderator (CATT)

- R4-2504913, WF on RRM requirements for NR_NTN_Ph3_Part2, Moderator (Qualcomm Incorporated)