3GPP TSG-RAN WG4 Meeting # 115	R4-2506799

Malta, MT, 19th - 23rd, May, 2025

Agenda item:	7.24.4.2

Source: 	Huawei, HiSilicon

Title: 	Discussion on RRM impact for Rel-19 MIMO on other aspects

Document for:	Discussion

- Introduction

The Rel-19 MIMO WI is approved in [1]. In this contribution, we further provide our views regarding the RAN4 impacts for the WI.

- Discussion

UE reporting enhancement for CJT calibration

The simulation assumption has been updated, the content is captured from WF [2] as below:

Issue 1-1: On the simulation assumption for delay and frequency offset for CJT calibration Agreement: - Update the report resolution to (MD = 256, MFO = 256). Moderator’s note: simulation results are collected in R4-2504955, companies can use the table structure in the excel doc to update their results.

We updated our simulation results based on this new simulation assumption. The simulation results can be found in the R4-2506800.

In the last meeting, CJTC reporting has a conclusion that defines the DO and FO core requirements. The updated info captured from WF [2] is shown below:

Issue 1-2: How to define RRM core requirements for delay and frequency offset reporting based on the simulation results for CJT calibration? Agreement: - Define core requirements for delay and frequency offset reporting for - Measurement period (FFS number of samples) - Scheduling restriction (if identified)

In this paper, we provide our analysis of how many samples are needed for the Frequency offset and Delay offset. Since RAN4 #114 meeting, companies have provided simulation results on CJTC reporting. Based on our simulations, we analysed UE throughput loss and the estimation error levels for FO/DO. Our findings indicate that the larger sample size fails to fully estimate the error between two TRPs, leading to increased UE throughput loss.

Observation 1: The larger sample size fails to fully estimate the error between two TRPs, leading to increased UE throughput loss.

On the basis of simulation results in R4-2501274 and R4-2503877, the CJTC reporting measurement sample is able to be set as 1 or 3. We are open to discussing both these types of values.

Proposal 1: On the basis of R4-2501274 and R4-2503877 simulation results, the CJTC reporting measurement sample is able to be set as 1 or 3.

For the performance requirement of CJTC reporting, based on our simulation results in R4-2501274 and R4-2503877, the measurement reference SNR of CJTC reporting should be set as 0 dB for 1 shot. The reason why is, the CJT reporting estimation level is over the AFO in SNR = -3, which means the estimation is not valid. Therefore, following this rationale, we would like to propose to set the measurement reference of CJTC reporting SNR as 0 dB.

Proposal 2: Setting the measurement reference SNR of the CJTC reporting to 0 dB.

Enhancement for asymmetric DL sTRP/UL mTRP scenarios

At the recent RAN4 meeting, the requirements for DL sTRP/UL mTRP Scenarios 1 and 2 were discussed. In this paper, we will provide our analysis of the requirements in detail.

Issue 2-1: For Scenario#2, how to update the RRM requirements for 2TA enhancement for asymmetric DL sTRP/ UL mTRP? Agreement: - Wait for RAN1 reply to the LS on whether the existing agreement for a single downlink timing reference is extended to Scenario#2.

In response to scenario #2, RAN1 LS has provided its official reply in document R1-2502956. Here is the RAN1 LS content:

Answer: The RAN1 agreements in RAN1#118bis “One downlink reference timing is supported and applied to both TAGs” are NOT applied for the scenario that UE is not configured with PL offset in joint/UL TCI state(s) and UE may expect to receive SSB from UL TRP(s). And RAN1 made the following new agreement for further clarification: Agreement: - For the scenario that the UE is configured with PL offset in joint/UL TCI state(s), when the UE is configured with rel-19 2 TAs, the UE maintains one single downlink reference timing - For the scenario that the UE is not configured with PL offset in joint/UL TCI state(s) and UE may expect to receive SSB from UL TRP(s), when the UE is configured with rel-19 2 TAs: - The UE maintains two downlink reference timings; - Baseline assumption for this feature is that Rx timing difference between two DL reference timings is no larger than one CP length, while it is subject to optional UE capability that the Rx timing difference between two DL reference timings can be assumed to be larger than CP length. - The reference point for PRACH transmission is indicated as follows: - if “PRACH association indicator” in DCI format 1_0 is 0, the reference timing is the first detected path (in time) of one of the corresponding downlink reference signal(s) of DL TCI state(s) of the reference cell associated with the first TAG. - if “PRACH association indicator” in DCI format 1_0 is 1, the reference timing is the first detected path (in time) of one of the corresponding downlink reference signal(s) of DL TCI state(s) of the reference cell associated with the second TAG - Above applies for the case UE is configured with SSB-MTC-additionalPCI.

In terms of RAN1 LS content, scenario #2 maintains 2 DL reference timings and is multi-DCI based multi-TRP operation with two TAs. This indicated that scenario #2 can reuse the legacy timing requirements that Rel-18 multi-DCI two TA.

Proposal 3: In terms of the RAN1 LS, scenario #2 is able to reuse the legacy multi-DCI with two TA requirements without update.

For the objective for asymmetric DL sTRP/UL mTRP, the open issues are summarised as follows [2]:

Issue 2-2: For Scenario#1, how to update gradual timing requirements for 2TA enhancement for asymmetric DL sTRP/ UL mTRP? Way forward: - Discuss whether gradual timing adjustment applies to both TAGs or not. - Discuss whether gradual timing adjustment applies to DL/UL TRP or also to UL TRP.

According to the two TA requirement for scenario 1, the gradual timing adjustment requirement also needs to be updated. Here is one method is able to be used based on our perspective:

For scenario 1, for both TAGs, the gradual timing adjustment defined in the legacy applies to each TAG and shall be met for each TAG.

Proposal 4: The gradual timing adjustment requirement can be updated as:

For scenario 1, for both TAGs, the gradual timing adjustment defined in the legacy applies to each TAG and shall be met for each TAG.

Another issue is whether there are TCI state requirements that impact for TCI state requirement for Scenario #2. The options from the last meeting are captured from WF [2] as below:

Issue 2-6: Whether to define UL TCI state activation requirement for Scenario#2? Way forward: discuss the following options in the next meeting for Scenario#2 - Option 1: No RRM impacts on TCI state switching requirement. - Option 2: Legacy SSB based UL TCI state activation requirement can be re-used.

For the TCI state activation requirement for Scenario #2, our understanding is that RAN4 do not define requirements when the QCL source is the SRS. There are two technical reasons for the explanation shown below.

Firstly, this is matched with the legacy spec description, which is no requirement when the UE is requested to switch to a TCI state with the higher layer parameter TCI-UL-State associated to SRS. Secondly, this issue is related to the RAN1 LS reply for scenario 2 as well. Regarding the RAN1 clarification in LS, scenario #2 seems to be one of the traditional cases. Therefore, there is no RRM impacts on the TCI state switching requirement for Scenario #2.

Proposal 5: No RRM impacts on TCI state switching requirement for Scenario #2.

- Conclusions

Observation 1: The larger sample size fails to fully estimate the error between two TRPs, leading to increased UE throughput loss.

Proposal 1: On the basis of R4-2501274 and R4-2503877 simulation results, the CJTC reporting measurement sample is able to be set as 1 or 3.

Proposal 2: Setting the measurement reference SNR of the CJTC reporting to 0 dB.

Proposal 3: In terms of the RAN1 LS, scenario #2 is able to reuse the legacy multi-DCI with two TA requirements without update.

Proposal 4: The gradual timing adjustment requirement can be updated as:

For scenario 1, for both TAGs, the gradual timing adjustment defined in the legacy applies to each TAG and shall be met for each TAG.

Proposal 5: No RRM impacts on TCI state switching requirement for Scenario #2.

References

[1] RP-242394 WID: NR MIMO Phase 5

[2] R4-2504903 WF on RRM requirements for NR_MIMO_Ph5_Part2