3GPP TSG-RAN WG4 Meeting # 115	R4-2506801

Malta, MT, 19th - 23rd, May, 2025

Agenda item:	7.24.5

Source: 	Huawei, HiSilicon

Title: 	Discussion on RRM impact for Rel-19 MIMO on performance part

Document for:	Discussion

- Introduction

The Rel-19 MIMO WI is approved in [1]. In this contribution, we provide our views regarding the RAN4 impacts on the performance part.

- Discussion

UE-initiated/event-driven beam management

For UEIBR, considering the progress on core requirements discussion so far, test cases to verify UE behaviour and requirements shall be introduced. In details, following aspects shall be considered for test cases design.

- timeRestrictionForChannelMeasurement: Based on discussion in last meeting, as last the UE behaviour when M=1 is clear. Whether to define TC for M=3 (i.e. timeRestrictionForChannelMeasurement needs FFS)

- SSB-based/CSI-RS-based: Both SSB-based and CSI-RS based UEIBR is supported. The remaining issue is the applicable rule of CSI-RS based UEIBR for inter-cell. As discussed for many meetings, it is the historical problem. Thus, at least test case for intra-cell shall be defined for both SSB-based and CSI-RS based UE IBR.

- Event 1/2/7: Based on RAN1 discussion, Event 1/2/7 are both supported. For RRM requirements and TC, RAN4 shall discuss whether we should introduce TC for both Event 1/2/7. One way is to define TC for all events and define applicability rules that UE is not required to test all three events.

- Time window based: RAN4 is still discussing whether to define requirements for time window-based approach. The TC discussion should be aligned with core requirements conclusion.

Proposal 1: For UEIBR, define TC as follows:

- SSB-based UEIBR for Event-2.

- CSI-RS based UEIBR for Event-2.

Note 1: At least define TC when timeRestrictionForChannelMeasurement is configured.

Note 2: At least define TC for intra-cell.

Proposal 2: RAN4 to discuss whether to define TC for inter-cell UEIBR.

Proposal 3: RAN4 to discuss whether to define TC for other Events (i.e. Event 1 and Event 7)

UE reporting enhancement for CJT calibration

The discussion regarding the core aspects of CJT calibration is ongoing, and the performance-related discussions must wait until the core requirements are finalised. Specifically, the RAN4 needs to agree on the number of samples required for CJT calibration estimation before addressing performance issues. While there was consensus on defining the core requirements for DO and FO in the last meeting, the lack of agreement on sample size. Therefore, the priority is to finalise the core aspects, including sample size and requirement definition.

Proposal 4: Wait to finalise the core part requirement, including sample size and requirement definition.

Enhancement for asymmetric DL sTRP/UL mTRP scenarios

At the recent RAN4 meeting, the core part discussion for DL sTRP/UL mTRP included the Scenario issue and whether or not to update the legacy requirement for Scenario #1. In this paper, we will provide our views on the performance part of asymmetric DL sTRP/UL mTRP based on the core part agreements.

According to the core part agreement on the updated 2TA requirement for Scenario #1, the new version shows that the two TAGs applied for the corresponding uplink transmission(s), which is captured from WF [3] as below.

Issue 2-2: How to update the RRM requirements for 2TA enhancement for asymmetric DL sTRP/ UL mTRP? Agreement: - For scenario 1, for both TAGs, the uplink transmission(s) timing for PUCCH/PUSCH/SRS take place (NTA + NTA offset) x Tc before the reception of the first detected path in time of one of the corresponding downlink reference signal(s) of the reference cell associated with DL or Joint TCI state. - The above agreement can be revisited if not aligned with RAN1 further agreement.

The legacy timing accuracy test case can be reused and does not need to be defined as a new test case.

Proposal 5: For updated 2AT requirements for Scenario #1, the legacy timing accuracy test case can be reused without any new updates.

- Conclusions

Proposal 1: For UEIBR, define TC as follows:

- SSB-based UEIBR for Event-2.

- CSI-RS based UEIBR for Event-2.

Note 1: At least define TC when timeRestrictionForChannelMeasurement is configured.

Note 2: At least define TC for intra-cell.

Proposal 2: RAN4 to discuss whether to define TC for inter-cell UEIBR.

Proposal 3: RAN4 to discuss whether to define TC for other Events (i.e. Event 1 and Event 7)

Proposal 4: Wait to finalise the core part requirement, including sample size and requirement definition.

Proposal 5: For updated 2AT requirements for Scenario #1, the legacy timing accuracy test case can be reused without any new updates.

References

[1] RP-242394 WID: NR MIMO Phase 5

[2] R4-2504903 WF on RRM requirements for NR_MIMO_Ph5_Part2

[3] R4-2502596 WF on RRM requirements for NR_MIMO_Ph5_Part2