3GPP TSG-RAN WG4 #115	R4-2506802

Malta, MT, 19th - 23rd, May, 2025

Source: 	Huawei, HiSilicon

Title:  	Discussions on RRM core requirements for Rel-19 ATG

Agenda Item:	7.12.5

Document for:	Discussion

Introduction

In the last meeting, we discussed the Rel-19 ATG CA enhancement. The points and views of companies are detailed in the WF [1]. This paper presents our observations and views on RRM impacts on ATG CA.

RRM core requirement analysis

Compared with the requirements defined in Rel-18 ATG for single-cell, we provide our analysis on the potential RRM impacts to support CA in case-by-case manner in the following part.

The Report granularity of UE capability

In the last meeting, we discussed the ATG UE capability in a common Rx beam case, the details are captured and shown below:

Issue 1-1-2-1: The description of capability definition Agreement: - Indicates whether the UE supports one common Rx beam when UE capable of antennaArrayType-r18 on both PCC and SCC. If the field is absent, UE supports two simultaneous separate Rx beams when UE capable of antennaArrayType-r18 on both PCC and SCC. - The UE indicating support of this feature shall also indicate support of antennaArrayType-r18 on each band of the supported Band combination. Issue 1-1-2-2: The Report granularity of UE capability FFS: - Per BC

In this paper, we present our perspective on the details of this new capability. The agreement reached in the last meeting defined the capability as an indicator of whether the UE supports one common Rx beam across the band. In our view, this capability should only apply to UEs that support a common Rx beam in certain band combinations. The rationale is that the inter-band MRDT has been defined as 33 µs. This implies that the UE is expected to support both separate Rx beams and common Rx beams simultaneously by default. If a UE can meet the 33 µs MRDT requirement, it should also be capable of meeting the more undemanding 3 µs MRDT requirement. Based on this understanding, we believe a note is needed to clarify the intent of this capability:

Note: When UE only support the common Rx beam in certain band combinations, UE needs to report this capability.

Proposal 1: A note is needed for this capability:

Note: When UE only support the common Rx beam in certain band combinations, UE needs to report this capability.

In terms of the spec 38.300 [2], the UE capabilities in NR are generally defined per UE, per duplex mode (FDD/TDD), per frequency range (FR1/FR2), per band, and per band combinations. In our view, the CBM beam case is inter-band CA, the capability for this case will be a per BC capability obviously.

Proposal 2: This capability is able to be defined as per BC.

Interruption requirement

In the last meeting, there was one proposal that triggered the discussion on whether or not to define the interruption requirement separately for CBM or not, which is shown below:

Issue 1-2-2: Interruption requirements - Proposals: - Proposal 1: - For the interruptions caused by SCell addition/release and any types of SCell activation/deactivation, distinguish the interruption requirements for Category 1 and 2. - Category 1: antenna array assumption in intra-band CA and Case 4 in inter-band CA - Category 2: other cases For other interruptions, define unified interruption requirements for all UE antenna types.

In our view, this is not needed if companies can provide the exact cases that may need different interruption requirements.

Proposal 3: No need to define interruption requirements for Case 4 and other cases separately.

Sharing factor between L1 measurement and RRM measurement

In the last meeting, companies proposed to discuss the SCell addition/activation mechanism.

Issue 1-3-2: Sharing factor between L1 measurement and RRM measurement - Proposal 1: - Consider impact on Klayer1_measurement due to 33usec MRTD for Case 4 antenna type, e.g., - if all of the reference signal configured for RLM, BFD, CBD or L1-RSRP for beam reporting outside measurement gap and fully-overlapped by intra-frequency SMTC occasions are not overlapped with any of the SSB symbols and the RSSI symbols, and 2 symbol before each consecutive SSB symbols and the RSSI symbols, and 2 symbol after each consecutive SSB symbols and the RSSI symbols of serving cells when inter-band carrier aggregation within FR1 is configured and UE supports [capability of case 4], given that SSB-ToMeasure and SS-RSSI-Measurement are configured, where SSB symbols are indicated by the union set of SSB-ToMeasure from all the configured measurement objects on the same serving carrier which can be merged. and RSSI symbols are indicated by SS-RSSI-Measurement

In our view, for inter-band CA when ATG UE with common Rx beam the PCC and SCC will use the same L1 and L3 Rx beam. Meanwhile, network can configure the L1 measurement and L3 measurement for PCell and SCell on the same measurement occasion.

Proposal 4: RAN4 assumed that when ATG UE uses the common Rx beam, the UE uses the same L1 beam for PCC and SCC L1 measurement and uses the same L3 beam for PCC and SCC L3 measurement.

When a collision occurs during the L1 and L3 measurements in the inter-band CA for UE supporting case #4, the UE must decide whether to perform the L1 measurement using the L1 common Rx beam for both PCC and SCC, or to perform the L3 measurement using the L3 common Rx beam for both PCC and SCC. So, this Klayer1_measurement for ATG UE with common Rx beam in inter-band CA could be defined as:

Klayer1_measurement=1,

-	if all of the reference signals configured for RLM, BFD, CBD or L1-RSRP for beam reporting on any FR2 serving frequency in the same band outside measurement gap are not fully overlapped by intra-frequency SMTC occasions, or

-	if all of the reference signals configured for RLM, BFD, CBD or L1-RSRP for beam reporting on serving frequency of PCC or SCC that [UE support the case 4 capability] outside measurement gap are not fully overlapped by intra-frequency SMTC occasions of PCC or SCC, or

-	if all of the reference signal configured for RLM, BFD, CBD or L1-RSRP for beam reporting on any FR2 serving frequency in the same band outside measurement gap and fully-overlapped by intra-frequency SMTC occasions are not overlapped with any of the SSB symbols and the RSSI symbols, and 1 symbol before each consecutive SSB symbols and the RSSI symbols, and 1 symbol after each consecutive SSB symbols and the RSSI symbols, given that SSB-ToMeasure and SS-RSSI-Measurement are configured, where SSB symbols are indicated by the union set of SSB-ToMeasure from all the configured measurement objects on the same serving carrier which can be merged. and RSSI symbols are indicated by SS-RSSI-Measurement;

-	if all of the reference signal configured for RLM, BFD, CBD or L1-RSRP for beam reporting on serving frequency of PCC or SCC that [UE support the case 4 capability] outside measurement gap and fully-overlapped by intra-frequency SMTC occasions of PCC or SCC are not overlapped with any of the SSB symbols and the RSSI symbols, and 1 symbol before each consecutive SSB symbols and the RSSI symbols, and 1 symbol after each consecutive SSB symbols and the RSSI symbols, given that SSB-ToMeasure and SS-RSSI-Measurement are configured, where SSB symbols are indicated by the union set of SSB-ToMeasure from all the configured measurement objects on the same serving carrier which can be merged. and RSSI symbols are indicated by SS-RSSI-Measurement;

Klayer1_measurement=1.5, otherwise.

Proposal 5: Klayer1_measurement for ATG UE with common Rx beam in inter-band CA could be defined as:

Klayer1_measurement=1,

-	if all of the reference signals configured for RLM, BFD, CBD or L1-RSRP for beam reporting on any FR2 serving frequency in the same band outside measurement gap are not fully overlapped by intra-frequency SMTC occasions, or

-	if all of the reference signals configured for RLM, BFD, CBD or L1-RSRP for beam reporting on serving frequency of PCC or SCC that [UE support the case 4 capability] outside measurement gap are not fully overlapped by intra-frequency SMTC occasions of PCC or SCC, or

-	if all of the reference signal configured for RLM, BFD, CBD or L1-RSRP for beam reporting on any FR2 serving frequency in the same band outside measurement gap and fully-overlapped by intra-frequency SMTC occasions are not overlapped with any of the SSB symbols and the RSSI symbols, and 1 symbol before each consecutive SSB symbols and the RSSI symbols, and 1 symbol after each consecutive SSB symbols and the RSSI symbols, given that SSB-ToMeasure and SS-RSSI-Measurement are configured, where SSB symbols are indicated by the union set of SSB-ToMeasure from all the configured measurement objects on the same serving carrier which can be merged. and RSSI symbols are indicated by SS-RSSI-Measurement;

-	if all of the reference signal configured for RLM, BFD, CBD or L1-RSRP for beam reporting on serving frequency of PCC or SCC that [UE support the case 4 capability] outside measurement gap and fully-overlapped by intra-frequency SMTC occasions of PCC or SCC are not overlapped with any of the SSB symbols and the RSSI symbols, and 1 symbol before each consecutive SSB symbols and the RSSI symbols, and 1 symbol after each consecutive SSB symbols and the RSSI symbols, given that SSB-ToMeasure and SS-RSSI-Measurement are configured, where SSB symbols are indicated by the union set of SSB-ToMeasure from all the configured measurement objects on the same serving carrier which can be merged. and RSSI symbols are indicated by SS-RSSI-Measurement;

Klayer1_measurement=1.5, otherwise.

Measurement enhancement for ATG UE with common Rx beam

In the last meeting, several measurement enhancements were discussed and finally focused on the UE skip the neighbour cell measurement of SCC.

Issue 1-3-5: Measurement enhancement for ATG UE with antenna array Candidate options: - UE could skip the neighbour cell measurement of {SCC and/or CC with deactivated SCell} by network control. - Option 1: this can already be achieved by the existing configuration. - Option 2: Consider enhancement for Rel-19 ATG CA

In our view, this can be achieved by the NW configuration. For instance, in intra-frequency measurement, the neighbour cell will be measured while simultaneously measuring the serving cell when the serving cell is configured in MO. This is because of measuring the serving cell's coverage area while also measuring the neighbour cell. This situation with no effect on serving cell throughput.

On the other hand, for neighbour cell inter-frequency measurement, the network can only configure the serving cell MOs to skip the neighbour cell measurement of SCC. In summary, UE could skip the neighbour cell measurement of {SCC and/or CC with deactivated SCell} can already be achieved by the existing configuration.

Proposal 6: The UE can measure the serving cell of {SCC or CC with deactivated SCell} only, which is already achievable with the legacy network configuration.

Conclusion

Proposal 1: A note is needed for this capability:

Note: When UE only support the common Rx beam in certain band combinations, UE needs to report this capability.

Proposal 2: This capability is able to be defined as per BC.

Proposal 3: No need to define interruption requirements for Case 4 and other cases separately.

Proposal 4: RAN4 assumed that when ATG UE uses the common Rx beam, the UE uses the same L1 beam for PCC and SCC L1 measurement and uses the same L3 beam for PCC and SCC L3 measurement.

Proposal 5: Klayer1_measurement for ATG UE with common Rx beam in inter-band CA could be defined as:

Klayer1_measurement=1,

-	if all of the reference signals configured for RLM, BFD, CBD or L1-RSRP for beam reporting on any FR2 serving frequency in the same band outside measurement gap are not fully overlapped by intra-frequency SMTC occasions, or

-	if all of the reference signals configured for RLM, BFD, CBD or L1-RSRP for beam reporting on serving frequency of PCC or SCC that [UE support the case 4 capability] outside measurement gap are not fully overlapped by intra-frequency SMTC occasions of PCC or SCC, or

-	if all of the reference signal configured for RLM, BFD, CBD or L1-RSRP for beam reporting on any FR2 serving frequency in the same band outside measurement gap and fully-overlapped by intra-frequency SMTC occasions are not overlapped with any of the SSB symbols and the RSSI symbols, and 1 symbol before each consecutive SSB symbols and the RSSI symbols, and 1 symbol after each consecutive SSB symbols and the RSSI symbols, given that SSB-ToMeasure and SS-RSSI-Measurement are configured, where SSB symbols are indicated by the union set of SSB-ToMeasure from all the configured measurement objects on the same serving carrier which can be merged. and RSSI symbols are indicated by SS-RSSI-Measurement;

-	if all of the reference signal configured for RLM, BFD, CBD or L1-RSRP for beam reporting on serving frequency of PCC or SCC that [UE support the case 4 capability] outside measurement gap and fully-overlapped by intra-frequency SMTC occasions of PCC or SCC are not overlapped with any of the SSB symbols and the RSSI symbols, and 1 symbol before each consecutive SSB symbols and the RSSI symbols, and 1 symbol after each consecutive SSB symbols and the RSSI symbols, given that SSB-ToMeasure and SS-RSSI-Measurement are configured, where SSB symbols are indicated by the union set of SSB-ToMeasure from all the configured measurement objects on the same serving carrier which can be merged. and RSSI symbols are indicated by SS-RSSI-Measurement;

Klayer1_measurement=1.5, otherwise.

Proposal 6: The UE can measure the serving cell of {SCC or CC with deactivated SCell} only, which is already achievable with the legacy network configuration.

References

[1] R4-2504898 WF on RRM requirements for NR_ATG_enh

[2] 38.300 3rd Generation Partnership Project; Technical Specification Group Radio Access Network; NR; User Equipment (UE) radio access capabilities (Release 18)