3GPP TSG-RAN WG4 Meeting #115		R4-2506824

St. Julian’s, Malta, 19th – 23rd May 2025

Agenda item:	7.27.3.1

Source:	Intel Corporation

Title:	Discussion on RRM requirements for On-demand SSB SCell operations

Document for:	Discussion

# Introduction

The Rel-19 NR network energy saving enhancements work item was approved in [1]. The work item Core part objectives for on-demand SSB are provided below.

The objectives of the work item are the following: - Specify procedures and signaling method(s) to support on-demand SSB SCell operation for UEs in connected mode configured with CA, for both intra-/inter-band CA. [RAN1/2/3/4] - Specify triggering method(s) (select from UE uplink wake-up-signal using an existing signal/channel, cell on/off indication via backhaul, Scell activation/deactivation signaling) - Note1: On-demand SSB transmission can be used by UE for at least SCell time/frequency synchronization, L1/L3 measurements and SCell activation, and is supported for FR1 and FR2 in non-shared spectrum.

In this paper we provide analysis on the RRM requirements for on-demand SSB in SCell.

# On-demand SSB RRM requirements

## Deactivated SCell measurements based on on-demand SSB

Deactivated SCell measurements based on On-demand SSB

It is important to specify correct UE behaviours, and deactivated SCell measurements requirements based on the OD-SSB to guarantee fair performance enhancement compared to legacy system.

One of the principles we have had for measurements between PCell and SCells are that we assume the UE to have separate searchers dedicated to either PCell or SCells so that measurements are not shared between PCell and SCell measurements. This can be told by looking up to the specification TS 38.133 clause 9.1.5.1.2 on the CSSF definitions. The outside gap CSSF of the intra-frequency without gap deactivated SCell measurements is the number of configured SCells.

In Rel-19, we introduced deactivated SCell measurements based on OD-SSB and when there is no always-on SSB on the target SCell, we need to make sure that the UE puts all its SCell measurement resources on the OD-SSB when transmitted.

Proposal 1: Prioritize UE deactivated SCell measurements based on OD-SSB over other measurements on the SCells, when OD-SSB is configured and activated.

This is to say that when there is OD-SSB configured and activated, the CSSF outside gap value is altered to prioritize OD-SSB measurements over others. For example, for FR1 only CA in SA mode measurements in Table 9.1.5.1.2-1, CSSF equalled NSSC_SSB + Y + Z + 2 x NSSC_CSIRS + NSSC_CCA_RSSI/CO should equal NSSC_ODSSB, which is the number of SCell that has activated OD-SSB being measured by the UE. Obviously if this is the case, we don’t allow the UE to measure on any other SCell which is not with activated OD-SSB, during the fast measurement windows. There is impact on the requirements for legacy measurements that the measurement reports are delayed considering that the measurements are not carried out during fast measurement windows.

Or if we allow sharing between activated OD-SSB and always-on SSB measurements among different SCell carriers, it is not fair to allow equal sharing among all carriers during the fast measurement windows for OD-SSB. One of the ways is to define a ratio to make more use of the fast measurement windows for OD-SSB while still allow the UE to have a decent measurement report on other SCell-s which are not with OD-SSB.

Going along this direction, when there are multiple activated OD-SSB for the UE to measure on, we can also consider sharing schemes among the carriers, though equal sharing seems to be the best in terms of trade-off between performance and complexity. The network in this way easily understands how to configure the UE to achieve optimal systematic mobility performance on the SCells.

Another consideration can be when the activated OD-SSB are not overlapped at all at time domain we can further enhance the sharing scheme to allow the UE to retune between activated OD-SSB frequencies to make faster measurement reports. This imposes on the other hand, the network to consider scattered time domain allocation of OD-SSB for different SCell carriers. However, this implies complex configurations at the deployment side and does not make excessively perfect sense over a plain simple way to just make cascade/linear activations of OD-SSB among target SCell carriers.

Fast measurement windows

In the previous WG meetings, fast measurement windows are agreed that the UE uses fast measurement windows which starts from the activation of OD-SSB for quick measurement reporting based on OD-SSB. The idea is to let the UE reports by one time but really quick during the fast measurement window.

Issue 1-2-3: OD-SSB based deactivated SCell measurement – Procedure Sub-issue 1 – How to perform measurement after time window within one time OD-SSB activation/deactivation? Agreement: - After the fast time window, the UE shall meet the legacy deactivated SCell measurement requirement as long as OD-SSB is activated/transmitted from UE’s perspective. - The agreement can be revisited based on further RAN1 agreement about OD-SSB deactivation mechanism

After the fast measurement and report, the UE is required to measure on the target SCell following legacy measurement requirements even in Case #1. However, since it is not guaranteed that the OD-SSB is always activated to the UE, we don’t think there is need to specify measurements after the fast window before the OD-SSB is activated again by network signalling. At least after the OD-SSB deactivation, the UE is not required to measure on the target SCell under Case #1.

Proposal 2: After the fast measurement window, the UE is not required to measure on the target SCell under Case #1, until the next OD-SSB activation command.

The end point of the fast measurement window is specified from the UE perspective. It is when the UE has finished the first reporting to the network based on the deactivated SCell measurement results using OD-SSB during the fast measurement window.

Proposal 3: The end point of the fast measurement window is specified from the UE perspective. It is when the UE has finished the first reporting to the network based on the deactivated SCell measurement results using OD-SSB during the fast measurement window.

Case #1 measurement requirements

Then when it comes to the measurement requirements implementation, we can specify for Case #1 the below requirements.

For FR1, measurement period requirements and SS detection requirements for deactivated SCell are both 5 x Kp x measurement cycle x CSSF while for FR2 it is ceil(M x Kp x Klayer1_measurement) x measurement cycle x CSSF.

Case #2 where always-on SSB is transmitted

The difference between deactivated SCell measurements for Case #2 and Case #1 is major from the fact that after the fast measurements, the UE in Case #2 falls back to legacy measurements based on the always-on SSB on the same target SCell, while UE in Case #1 is not required to measure on anything on the SCell.

Proposal 4: UE falls back to legacy deactivated SCell measurements after the fast measurement window under Case #2.

During the fast measurement window, the UE follows the same requirements as in the Case #1. Same CSSF will apply between Case #1 and Case#2 deactivated SCell measurements in the fast measurement windows. This facilitates unified UE baseband implementation in terms of fast measurements on the target deactivated SCell.

Proposal 5: For Case #2 during the fast measurement window, the UE follows the same requirements as in the Case #1.

Considering that during the fast measurement window, the UE follows the measurement cycle of OD-SSB periodicity configurations -- when the OD-SSB is with the same frequency of always-on SSB, there is no problem for the UE to carry out the measurements based on the measurement cycle of OD-SSB periodicity.

# UE features in Rel-19 NES

Lower bound for measurement periodicity in FMW

It was agreed in the last meeting that a UE capability should be introduced:

Sub-issue 1: Lower bound for measurement in FMW Agreement Introduce UE capability on whether a lower bound is needed, i.e., - The lower bound is not needed (default value), or, - A lower bound is needed and based on OD-SSB periodicity of [10] ms - Further confirm whether [10] is feasible, and only one value

So, a per-UE signalling indication should be introduced and a feature group should be implemented in the Rel-19 RAN4 UE feature list and further into TR 38.822.

Proposal 6: Introduce optional capabilities signalling for UE lower bound for measurement periodicity in FMW and the corresponding Rel-19 UE feature group.

Additional processing time

The group also agreed to introduce a UE capability on needed additional processing time for deactivated SCell measurements.

Agreement UE applies the additional processing timing only in deactivated SCell measurement. - Note: The additional UE processing timing is not applied to SCell activation procedure. The additional processing timing is T_processing, the value of T_processing: - Introduce different values with UE capability: [2 or 3]ms and 5ms.

Thus we need to introduce UE signalling indication and the corresponding UE feature group for the additional process time capabilities.

Proposal 7: introduce optional capabilities signalling for UE additional processing time during deactivated SCell measurements and the corresponding Rel-19 UE feature group.

Considering Proposal 6 and 7 we propose the below features for this work item. We also capture the below table in our AI 8 paper [2].

Table: Rel-19 RAN4 feature list for Netw_Energy_NR_enh

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (V2X WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 54. Netw_Energy_NR_enh | 54-1 | UE support on no lower bound for measurement periodicity during measurements on OD-SSB in the FMW | - 1) UE capability to whether no lower bound of measurement periodicity when measuring on OD-SSB within Fast Measurement windows is supported | TBA | Yes | NA | The UE applies lower bound of [10] ms measurement periodicity for OD-SSB measurements within FMW | Per UE | No | No | NA |  | Optional with capability signalling |
|  | 54-2 | UE capabilities on the supported additional processing time during deactivated SCell measurements based on OD-SSB | - 1) UE supported value of additional processing time during deactivated SCell measurements based on OD-SSB | TBA | Yes | NA | The UE applies 5ms additional processing time during deactivated SCell measurements based on OD-SSB | Per UE | No | No | NA |  | Optional with capability signalling |

# Conclusion

In this paper we provide analysis on the RRM requirements for on-demand SSB in SCell.

Proposal 1: Prioritize UE deactivated SCell measurements based on OD-SSB over other measurements on the SCells, when OD-SSB is configured and activated.

Proposal 2: After the fast measurement window, the UE is not required to measure on the target SCell under Case #1, until the next OD-SSB activation command.

Proposal 3: The end point of the fast measurement window is specified from the UE perspective. It is when the UE has finished the first reporting to the network based on the deactivated SCell measurement results using OD-SSB during the fast measurement window.

Proposal 4: UE falls back to legacy deactivated SCell measurements after the fast measurement window under Case #2.

Proposal 5: For Case #2 during the fast measurement window, the UE follows the same requirements as in the Case #1.

Proposal 6: Introduce optional capabilities signalling for UE lower bound for measurement periodicity in FMW and the corresponding Rel-19 UE feature group.

Proposal 7: introduce optional capabilities signalling for UE additional processing time during deactivated SCell measurements and the corresponding Rel-19 UE feature group.

# References

- RP-234065, New WID: Enhancements of network energy savings for NR, Moderator (Ericsson)

- R4-2506890, Views on RAN4 Rel-19 UE feature list, Intel Corporation