3GPP TSG-RAN WG4 Meeting #115		R4-2506828

St. Julian’s, Malta, 19th – 23rd May 2025

Agenda item:	7.18.2

Source:	Intel Corporation

Title:	Views on RRM enhancements ph5 RAN4 UE features in Rel-19

Document for:	Discussion

# Introduction

The Rel-19 RAN4 UE feature list discussions start in this meeting with the guidance for drafting RAN4 UE feature list as in the below box.

RAN4 Chair: * Single contribution per company is allowed in this agenda with the summary of proposed RAN4 related feature list. After the decision is made under the agenda of the individual WI, the corresponding UE capability will be captured in the Rel-19 feature list under this agenda. * The proponents also need submit the contribution with details to the agenda of the corresponding WI under "7.xx.xx General aspects" and the decision should be made under the agenda of the individual WI. * If there is no corresponding agenda, the proponents can directly submit the contributions with technique details in the dedicated agenda for Rel-19 feature list, and the decision will be made under this agenda. Moderator: It is highly appreciated that rapporteur companies can submit the FGs for your own WI under agenda AI.8. And in your contribution, please clarify the status of each FG, e.g. whether it had already been agreed in previous RAN4 meetings.

In this paper we provide information on the less than 5MHz CA/DC UE features in Rel-19. There is no new UE capabilities signalling needed in TS 38.306 and we propose to record the UE features in the feature list and TR 38.822 in Rel-19.

# UE features in Rel-19 RRM enhancements ph5

Regarding CSSF outside gap enhancements, the group agreed to introduce 2 UE feature groups in the last meeting:

Issue 1-4-1: UE capability for CSSF enhancement Agreement: - UE feature #1: Define UE capability for solution 1, further discuss whether it is per-UE or per-FR. - UE feature #2: Define UE capability for solution 3, the UE capability is defined separately for {FR1 only CA and FR1 only NR-DC}, {FR1 only EN-DC}, {FR1+FR2 CA}, {FR1+FR2 NR-DC} in the same FG.

Regarding BSF enhancements there were also agreements:

- Issue 1-2-2: Solutions to apply/specify L3 measurement delay reduction by optimizing Rx BSF Agreement in the ad-hoc: - Introduce the UE capability of reduced N, the values of which are {2, 4 and 6}. And UE chooses one value to report according to its capability. FFS the granularity of the UE capability in issue 1-2-4.

Note that there is NO need to introduce the UE capabilities signalling for supporting fast SCell activation requirements with R18 EMR.

We propose the below features for this work item. We also capture the below table in our AI 8 paper [1].

Table: Rel-19 RAN4 UE feature list for NR_RRM_Ph5

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (V2X WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 51. NR_RRM_Ph5 | 51-1 | UE support on solution 1 for CSSF outside gap enhancements | - 1) UE support for reduced CSSF outside gap in CA, NR-DC, EN-DC and NE-DC | NA | Yes | NA | The UE does not support reduced CSSF outside gap value in CA or NR-DC or EN-DC or NE-DC | Per-UE | NA | FR2 only | NA |  | Optional with capability signalling |
|  | 51-2 | UE support on solution 3 for CSSF outside gap enhancements | - 1) UE support for 3-searcher assumptions in FR1 only CA and/or NR-DC - 2) UE support for 3-searcher assumptions in FR1 only EN-DC - 3) UE support for 3-searcher assumptions in FR1+R2 CA - 4) UE support for 3-searcher assumptions in FR1+FR2 NR-DC | NA | Yes | NA | The UE does not support reduced CSSF outside gap solution 3 | Per-UE | NA | NA | NA |  | Optional with capability signalling |
|  | 51-3 | UE support on the reduced beam sweeping factor N during L3 measurements and mobility procedures | - 1) UE supported value of reduced beam sweeping factor N during L3 measurements and mobility procedures | NA | Yes | NA | The UE does not support reduced beam sweeping factor | Per-UE | NA | NA | NA | Candidate values: {2, 4, 6} | Optional with capability signalling |

# Conclusion

We propose the features for this work item. We also capture the below table in our AI 8 paper [1].

# References

- RP-2506890 Views on RAN4 Rel-19 UE feature list, Intel Corporation