# 3GPP TSG-RAN WG4 Meeting # 115	R4-2506829

Malta, MT, 19th – 23rd May, 2025

Source: 	Huawei, HiSilicon

Title: 	More on Delta Ppowerclass and Delta TRxSRS for 4Tx

Agenda Item:	5.6.1

Document for:	Approval

# Introduction

Despite alternatives provided in [1, 2], still RAN4 has not been able to reach a consensus on resolution for the issue raised in [3] for ΔPPowerClass and ΔTRxSRS for 4Tx. We further discuss another alternative to move forward.

# Discussion

Although we shared three observations in [2], one company still wants UE to be allowed to used e.g., ΔPPowerClass = 3 dB for 1TyR and 0 dB for 2TyR or 4TyR to make maximum use of 26 dBm x 4 implementation.

Observation 1: Specifying ΔPPowerClass = 3 dB alone precludes UE from implementing PA configuration of 23 dBm x 4, given that the UE will not use Antenna Virtualization (AV).

Observation 2: Specifying ΔPPowerClass = 6 dB alone does not preclude UE from implementing PA configuration of 26 dBm x 4 or (23 dBm + 26 dBm) x 2, although the output power is capped by 23 dBm. The UE with 26 dBm x 4 would be able to meet the requirements more easily than UE with 23 dBm x 4.

Observation 3: Capping the power by 23 dBm, i.e., ΔPPowerClass = 6 dB, for 1TyR may not be a big problem given that

- it’s likely that a UE with 4Tx implements at least 4Rx and some of the ILs towards Rx ports may be large so that the UE may not transmit 26 dBm at the first SRS transmission from the port with the least IL to make the power close to the power from the port with the largest IL.

- Since 29 dBm is not expected from any UE with 1TyR, network would not configure UE with 4Tx with 1TyR, but rather 2TyR or 4TyR would be used to shorten sounding time while the power per port for 1TyR are the same or similar to those for 2TyR or 4TyR.

The dilemma is that accommodating a request to make maximum use of 26 dBm x 4 implementations would preclude the other request to avoid Antenna Virtualization (called AV hereafter). This time, we approach to a solution from an angle of AV.

According to TS38.101-1, the max power for TxD is defined as follows.

As can be seen from the above, power from all UE transmit antenna connectors shall be summed for output power Tx Diversity. In an opposite way, if the specification explicitly doesn’t allow to sum the power, we can avoid AV. Currently, a following text aligned with this purpose can be seen in TS38.101-1.

For more specific to 4Tx delta Ppower, let’s assume 1TyR. The highlighted texts are the delta from our CR of [4] in RAN4#114.

- 3 or 6dB is applied during SRS transmission occasions with usage in SRS-ResourceSet set as ‘antennaSwitching’ with configured SRS resources in each SRS resource set(s) consisting of one SRS port when PC1.5 UE with four transmit antenna connectors further indicates SRS-TxSwitch capability ‘t1r2’, ‘t1r4’, ‘t1r1-t1r2’, ‘t1r1-t1r2-t1r4’ or further indicates srs-AntennaSwitchingBeyond4RX-r17 as ‘t1r2’, ‘t1r4’ or ‘t1r8’, where the configured power shall be verified per antenna connector per SRS transmission occasion;

In case of 2TyR.

-	0 or 3dB is applied during SRS transmission occasions with usage in SRS-ResourceSet set as ‘antennaSwitching’ with configured SRS resources in each SRS resource set(s) consisting of two SRS ports when PC1.5 UE with four transmit antenna connectors further indicates SRS-TxSwitch capability 't2r2', 't2r4', 't1r4-t2r4', 't1r1-t1r2-t2r2-t2r4' or 't1r1-t1r2-t2r2-t1r4-t2r4’ or further indicates srs-AntennaSwitchingBeyond4RX-r17 as ‘t2r2’, ‘t2r4’ or ‘t2r8’, where the configured power shall be verified per a pair of antenna connectors per SRS transmission occasion;

In the above, 3 dB for 1TyR and 0 dB for 2TyR correspond to 26 dBm x 4 and 6 dB for 1TyR and 3 dB for 2TyR correspond to 23 dBm x 4 implementation, respectively. Since the specification explicitly says that UE’s configured output power shall be verified per or a pair of antenna connector(s), unnecessary AV must be avoided.

Strictly speaking, in case 3 dB for 1TyR is applied a UE, then, 0 dB for 2TyR shall be applied to the same UE. Similarly, in case 6 dB for 1TyR is applied a UE, then, 3 dB for 2TyR shall be applied to the same UE. We are open to discuss whether that aspect should be incorporated into the specification or not, while that should be discussed once some basic agreements on the direction of the CR are agreed.

# Summary

As a result, we obtained following two proposals. A companion CR based on this contribution is also submitted [5].

Proposal 1: Following texts should be incorporated into TS38.101-1 to accommodate 26 dBm x 4 and 23 dBm x 4 implementation as well as the request to avoid Antenna Virtualization.

For more specific, let’s assume 1TyR. The highlighted texts are the delta from our CR of [R4-2500378] in RAN4#114.

- 3 or 6dB is applied during SRS transmission occasions with usage in SRS-ResourceSet set as ‘antennaSwitching’ with configured SRS resources in each SRS resource set(s) consisting of one SRS port when PC1.5 UE with four transmit antenna connectors further indicates SRS-TxSwitch capability ‘t1r2’, ‘t1r4’, ‘t1r1-t1r2’, ‘t1r1-t1r2-t1r4’ or further indicates srs-AntennaSwitchingBeyond4RX-r17 as ‘t1r2’, ‘t1r4’ or ‘t1r8’, where the configured power shall be verified per antenna connector per SRS transmission occasion;

In case of 2TyR.

-	0 or 3dB is applied during SRS transmission occasions with usage in SRS-ResourceSet set as ‘antennaSwitching’ with configured SRS resources in each SRS resource set(s) consisting of two SRS ports when PC1.5 UE with four transmit antenna connectors further indicates SRS-TxSwitch capability 't2r2', 't2r4', 't1r4-t2r4', 't1r1-t1r2-t2r2-t2r4' or 't1r1-t1r2-t2r2-t1r4-t2r4’ or further indicates srs-AntennaSwitchingBeyond4RX-r17 as ‘t2r2’, ‘t2r4’ or ‘t2r8’, where the configured power shall be verified per a pair of antenna connectors per SRS transmission occasion;

In addition, as proposed in [R4-2500377], we propose not to introduce new changes into the existing ΔTRxSRS part.

Proposal 2: Do not introduce new changes into the existing ΔTRxSRS part.

# References

[1] R4-2417517, More on Delta Ppowerclass and Delta TRxSRS for 4Tx, Huawei, HiSilicon, RAN4#113

[2] R4-2500377, More on Delta Ppowerclass and Delta TRxSRS for 4Tx, Huawei, HiSilicon, RAN4#114

[3] R4-2400341, Delta PpowerClsss for 4Tx for SRS antenna switching, Nokia, RAN4#110

[4] R4-2500378, DeltaP_PowerClass correction for SRS AS for 4Tx, Huawei, HiSilicon, RAN4#114

[5] R4-2506830, DeltaP_PowerClass correction for SRS AS for 4Tx, Huawei, HiSilicon, RAN4#115