3GPP TSG-RAN4 Meeting #115	R4-2506833

# Malta, MT, 19th – 23rd May, 2025

Source:	LG Electronics

Title: 	HPUE MPR for intra-band UL CA

Agenda Item:	7.1.2.1.2

Document for:	Discussion and Approval

# Introduction

RAN4 has been discussing requirements for PC1.5 intra-band contiguous and non-contiguous UL CA. Way forward [1] was agreed in RAN WG4 Meeting #114bis.

For intra-band contiguous CA with contiguous RB allocation, PC1.5 MPR was agreed as follows.

- The MPR values for PC3(1Tx) and PC2(2Tx) listed in the below table is just for reference

| Modulation |  | PC3 (1Tx) |  | PC2 (2Tx) |  | PC1.5 (2Tx) |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Handheld |  | Handheld |  | Handheld |  | FWA1 |  |
|  |  | inner | outer | inner | outer | inner | outer | inner | outer |
| DFT-s-OFDM | PI/2 PSK | 2.5 | 7 | 3.5 | 8 | 4.5 | 9.0 | 4 | 8.5 |
|  | QPSK | 2.5 | 7 | 3.5 | 8 | 4.5 | 9.0 | 4 | 8.5 |
|  | 16QAM | 2.5 | 7 | 3.5 | 8 | 4.5 | 9.0 | 4 | 8.5 |
|  | 64QAM | 5 | 7 | 6 | 8 | 7.0 | 9.0 | 6.5 | 8.5 |
|  | 256QAM | 7 | 7.5 | 8 | 8.5 | 9.0 | 9.5 | 8.5 | 9.0 |
| CP-OFDM | QPSK | 3.5 | 8 | 4 | 8.5 | 5.0 | 9.5 | 4.5 | 9.0 |
|  | 16QAM | 3.5 | 8 | 4 | 8.5 | 5.0 | 9.5 | 4.5 | 9.0 |
|  | 64QAM | 5 | 8 | 5.5 | 8.5 | 6.5 | 9.5 | 6.0 | 9.0 |
|  | 256QAM | 7 | 8 | 7.5 | 8.5 | 8.5 | 9.5 | 8.0 | 9.0 |
|  |  |  |  |  |  |  |  | NOTE 1: This table is targeted to large FWA form factor with 20 dB or above antenna isolation. |  |

For intra-band contiguous CA with non-contiguous RB allocation, PC1.5 MPR was agreed as follows.

- The MPR values for PC3(1Tx) and PC2(2Tx) listed in the below table is just for reference.

| Modulation |  | PC3 (1Tx) |  |  | PC2 (2Tx) |  |  | PC1.5 (2Tx) |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Handheld |  |  | Handheld |  |  | Handheld |  |  |
|  |  | inner | Outer12 | Outer23 | inner | Outer12 | Outer23 | inner | Outer12 | Outer23 |
| DFT-s-OFDM | PI/2 PSK | 2.5 | 6 | 11.5 | 41 | 8.5 | 14.5 | [4.5-5] | [9-9.5] | [15.5-17] |
|  | QPSK | 2.5 | 6 | 11.5 | 41 | 8.5 | 14.5 | [4.5-5] | [9-9.5] | [15.5-17] |
|  | 16QAM | 3 | 6 | 11.5 | 41 | 8.5 | 14.5 | [4.5-5] | [9-9.5] | [15.5-17] |
|  | 64QAM | 5 | 6 | 11.5 | 6 | 8.5 | 14.5 | [6.5-7] | [9-9.5] | [15.5-17] |
|  | 256QAM | 6.5 | 6.5 | 11.5 | 7.5 | 8.5 | 14.5 | [8-8.5] | [9-9.5] | [15.5-17] |
| CP-OFDM | QPSK | 3.5 | 7 | 12 | 4.51 | 9 | 15.5 | [5-5.5] | [9.5-10] | [16.5-17] |
|  | 16QAM | 3.5 | 7 | 12 | 4.51 | 9 | 15.5 | [5-5.5] | [9.5-10] | [16.5-17] |
|  | 64QAM | 5 | 7 | 12 | 6 | 9 | 15.5 | [6.5-7] | [9.5-10] | [16.5-17] |
|  | 256QAM | 7.5 | 7.5 | 12 | 8.5 | 9 | 15.5 | [9-9.5] | [9.5-10] | [16.5-17] |
| NOTE 1: the allowed MPR is [4]dB for aggregated allocation bandwidth < [2MHz]. NOTE 2: Outer 1 MPR for Pi/2 BPSK and QPSK is reduced by 2dB for aggregated allocation bandwidth > 10MHz NOTE 3: Outer 2 MPR is reduced by 4.5dB for aggregated allocation bandwidth > 10MHz NOTE 4: UE indicating TxD supported |  |  |  |  |  |  |  |  |  |  |

For intra-band non-contiguous CA, PC1.5 MPR was agreed to be further discussed in May meeting.

# Discussion

For intra-band contiguous CA with contiguous RB allocation, Handheld PC1.5 UE MPR was agreed with 1dB higher than Handheld PC2 UE MPR. FWA PC1.5 MPR was agreed with 0.5dB higher than Handheld PC2 UE MPR.

For intra-band contiguous CA with non-contiguous RB allocation, as the theoretical analysis was seen in [2], same 1dB delta can be applied to Handheld PC1.5 UE MPR compared to Handheld PC2 UE. And, 1dB delta is also reasonable values based the averaged MPR values from the attached excel sheet [1]. And, FWA PC1.5 MPR can be applied with 0.5dB higher than Handheld PC2 UE MPR,

Observation 1: For intra-band contiguous ULCA with non-contiguous RB allocation (with TxD), the relative MPR delta between the Handheld PC1.5 and Handheld PC2 UE is about 1dB both 1.) based on the averaged values, and 2.) based on the theoretical analysis.

Observation 2: For intra-band contiguous ULCA with non-contiguous RB allocation (with TxD), the relative MPR delta between the FWA PC1.5 and Handheld PC2 UE is about 0.5dB  based on the averaged values.

Proposal 1: For intra-band contiguous ULCA with non-contiguous RB allocation (with TxD), it is proposed to use the 1dB as MPR delta value between PC1.5 2TX Handheld and PC2 2TX Handheld UEs.

Proposal 2: For intra-band contiguous ULCA with non-contiguous RB allocation (with TxD), it is proposed to use the 0.5dB as MPR delta value between PC1.5 2TX FWA and PC2 2TX Handheld UE.

For intra-band non-contiguous CA, the theoretical analysis in [2] was done for NCCA against the MPR to meet -13dBm and -30dBm emission mask as this scenario matches fully with the two-tone formulas and is easy to explain, but similar relative difference applies also for other cases where the UE performance is limited by the transmitter third order non-linearity.

Proposal 3: For intra-band non-contiguous ULCA, it is proposed to use the 1dB as MPR delta value between PC1.5 2TX Handheld and PC2 2TX Handheld UEs.

Proposal 4: For intra-band non-contiguous ULCA, ANNEX A proposes a way on how the delta MPR could be defined in the 38.101-1 [3] specification for Handheld UE using the MPR to meet -30dBm/MHz and -13dBm/MHz requirements as an example, where the new requirement consists of the reference to existing requirement and delta MPR value.

For PC1.5 (2TX) FWA device there’s no proper reference existing, like another power class 2TX FWA UE. Therefore the delta approach to another FWA device cannot be used, but on the other hand, the MPR is still driven by the transmitter third order non-linearity, which is a function of the transmitter OIP3 and hence another delta value to other device type should also work.

Observation 3: For PC1.5 (2TX) FWA device there’s no proper reference existing, like another power class 2TX FWA UE. However, MPR is still driven by the transmitter third order non-linearity, which is a function of the transmitter OIP3 and hence another delta value to other device type should also work.

For PC1.5 intra-band NC CA of (2TX) FWA with indicating dualPA, based on [4],

Proposal 5: For PC1.5 intra-band NC CA of (2Tx) FWA, it is proposed to define MPR in Table 2-1 to meet -13dBm/MHz and -30dBm/MHz respectively.

Table 2-1 : PC1.5 (2Tx) FWA MPR for intra-band NC CA with 2x26dBm+2LO

| B | MPR (dB) |  |
| --- | --- | --- |
|  | -13dBm/MHz | -30dBm/MHz |
| 0 ≤ B <0.54 | ≤ 8.5 | ≤ 9.0 |
| 0.54 ≤ B < 1.08 | ≤ 8.0 | ≤ 9.0 |
| 1.08 ≤ B < 2.16 | ≤ 8.0 | ≤ 9.0 |
| 2.16 ≤ B < 3.24 | ≤ 7.5 | ≤ 9.0 |
| 3.24 ≤ B < 5.4 | ≤ 7.5 | ≤ 9.0 |
| 5.4 ≤ B ≤ 10.8 | ≤ 7.0 | ≤ 9.0 |
| 10.8 < B | ≤ 7.0 | ≤ 9.0 |

# Conclusions

This document discusses the MPR requirements for PC1.5 intra-band contiguous and non-contiguous UL CA for Handheld and FWA devices.

For intra-band contiguous CA with non-contiguous RB allocation,

Observation 1: For intra-band contiguous ULCA with non-contiguous RB allocation (with TxD), the relative MPR delta between the Handheld PC1.5 and Handheld PC2 UE is about 1dB both 1.) based on the averaged values, and 2.) based on the theoretical analysis.

Observation 2: For intra-band contiguous ULCA with non-contiguous RB allocation (with TxD), the relative MPR delta between the FWA PC1.5 and Handheld PC2 UE is about 0.5dB  based on the averaged values.

Proposal 1: For intra-band contiguous ULCA with non-contiguous RB allocation (with TxD), it is proposed to use the 1dB as MPR delta value between PC1.5 2TX Handheld and PC2 2TX Handheld UEs.

Proposal 2: For intra-band contiguous ULCA with non-contiguous RB allocation (with TxD), it is proposed to use the 0.5dB as MPR delta value between PC1.5 2TX FWA and PC2 2TX Handheld UE.

For intra-band non-contiguous CA,

Proposal 3: For intra-band non-contiguous ULCA, it is proposed to use the 1dB as MPR delta value between PC1.5 2TX Handheld and PC2 2TX Handheld UEs.

Proposal 4: For intra-band non-contiguous ULCA, ANNEX A proposes a way on how the delta MPR could be defined in the 38.101-1 [3] specification for Handheld UE using the MPR to meet -30dBm/MHz and -13dBm/MHz requirements as an example, where the new requirement consists of the reference to existing requirement and delta MPR value.

Observation 3: For PC1.5 (2TX) FWA device there’s no proper reference existing, like another power class 2TX FWA UE. However, MPR is still driven by the transmitter third order non-linearity, which is a function of the transmitter OIP3 and hence another delta value to other device type should also work.

Proposal 5: For PC1.5 intra-band NC CA of (2Tx) FWA, it is proposed to define MPR in Table 2-1 to meet -13dBm/MHz and -30dBm/MHz respectively.

Table 2-1 : PC1.5 (2Tx) FWA MPR for intra-band NC CA with 2x26dBm+2LO

| B | MPR (dB) |  |
| --- | --- | --- |
|  | -13dBm/MHz | -30dBm/MHz |
| 0 ≤ B <0.54 | ≤ 8.5 | ≤ 9.0 |
| 0.54 ≤ B < 1.08 | ≤ 8.0 | ≤ 9.0 |
| 1.08 ≤ B < 2.16 | ≤ 8.0 | ≤ 9.0 |
| 2.16 ≤ B < 3.24 | ≤ 7.5 | ≤ 9.0 |
| 3.24 ≤ B < 5.4 | ≤ 7.5 | ≤ 9.0 |
| 5.4 ≤ B ≤ 10.8 | ≤ 7.0 | ≤ 9.0 |
| 10.8 < B | ≤ 7.0 | ≤ 9.0 |

# References

[1] R4-2505100, WF on HPUE RF requirements, Qualcomm, Mediatek, Skyworks, LGE, TSG-RAN WG4 Meeting #114bis, April 2025

[2] R4-2412024, HPUE for intra-band UL CA, LGE, TSG-RAN4 Meeting #112, Aug 2024

[3] 3GPP TS 38.101-1, User Equipment (UE) radio transmission and reception; Part 1: Range 1 Standalone (Release 18)

[4] R4-2502091, HPUE for intra-band UL CA, LGE, TSG-RAN4 Meeting #114, Feb 2025

# Annex A: Draft CR

< START OF CHANGE #1 >

#### 6.2A.2.2.1	MPR to meet -30dBm/MHz

##### 6.2A.2.2.1.1		PC3 with indicating dualPA-Architecture supported

MPR in this clause is for intra-band non-contiguous CA power class 3 for UEs indicating IE dualPA-Architecture supported. The allowed maximum output power reduction is defined as:

MPR=MAWhere MA is defined as follows

MA = 	15; 	0 ≤ B < 1.08

14.5; 	1.08 ≤ B < 2.16

13.5; 	2.16 ≤ B < 3.24

12.5;       3.24 ≤ B < 5.04

11.5; 	5.04≤ B < 10.08

10.5; 	10.08 ≤ B < 16.38

10;        16.38 ≤ B < 21.78

9; 	      21.78 ≤ B

##### 6.2A.2.2.1.2		PC2 with indicating dualPA-Architecture supported

MPR in this clause is for intra-band non-contiguous CA power class 2 for UEs indicating IE dualPA-Architecture supported. The allowed maximum output power reduction is defined as:

MPR=MAWhere MA is defined as follows

MA = 	15.5; 	0 ≤ B < 1.44

15.0; 	1.44 ≤ B < 2.88

14.0;    2.88 ≤ B < 5.76

12.0; 	5.76 ≤ B < 10.8

10.5; 	10.8 ≤ B < 23.04

9.0; 	 23.04 ≤ B

##### 6.2A.2.2.1.3		PC3 without indicating dualPA-Architecture supported

MPR in this clause is for intra-band non-contiguous CA power class 3 for UEs without indicating IE dualPA-Architecture supported. The allowed maximum output power reduction is defined as:

MPR=MAWhere MA is defined as follows

MA = 	17.5; 	0 ≤ B < 1.08

17.0; 	1.08 ≤ B < 2.16

16.5; 	2.16 ≤ B < 3.24

16;      3.24 ≤ B < 5.04

15; 	5.04≤ B < 10.08

14.5; 	10.08 ≤ B < 36

10;      36 ≤ B < 56.88

9; 	56.88 ≤ B

##### 6.2A.2.2.1.4		PC2 without indicating dualPA-Architecture supported

MPR in this clause is for intra-band non-contiguous CA power class 2 for UEs without indicating IE dualPA-Architecture supported. The allowed maximum output power reduction is defined as:

MPR=MAWhere MA is defined as follows

MA = 	19.5; 	0 ≤ B < 1.08

19; 	1.08 ≤ B < 2.16

18; 	2.16 ≤ B < 5.04

16.5; 	5.04≤ B < 10.08

16; 	10.08 ≤ B < 36

12;      36 ≤ B < 56.88

10.5; 	56.88 ≤ B

##### 6.2A.2.2.1.5		PC1.5 with indicating dualPA-Architecture supported

MPR in this clause is for intra-band non-contiguous CA power class 1.5 for UEs with indicating IE dualPA-Architecture supported. The allowed maximum output power reduction is defined as:

MPR= MA,total , where MA,total is defined as follows

MA,total= MA + MA,delta , where MA equals to MA defined in sub clause 6.2A.2.2.1.2 and MA,delta is [1]dB for Handheld UE.

#### 6.2A.2.2.2	MPR to meet -13dBm/MHz

##### 6.2A.2.2.2.1		PC3 with indicating dualPA-Architecture supported

MPR in this clause is for intra-band non-contiguous CA power class 3 for UEs indicating IE dualPA-Architecture supported. The allowed maximum output power reduction is defined as:

MPR=MA

Where MA is defined as follows

MA = 	9	;	 0 ≤ B < 0.54

8	;	 0.54 ≤ B < 1.08

7	; 	1.08 ≤ B < 2.16

6.5	; 	2.16 ≤ B < 3.24

5.5	; 	3.24 ≤ B < 5.4

4	; 	5.4 ≤ B

##### 6.2A.2.2.2.2		PC2 with indicating dualPA-Architecture supported

MPR in this clause is for intra-band non-contiguous CA power class 2 for UEs indicating IE dualPA-Architecture supported. The allowed maximum output power reduction is defined as:

MPR=MA

Where MA is defined as follows

MA = 	9	;	 0 ≤ B < 0.54

8	;	 0.54 ≤ B < 1.08

7	; 	1.08 ≤ B < 2.16

6.5	; 	2.16 ≤ B < 3.24

6	; 	3.24 ≤ B < 5.4

5.5	; 	5.4 ≤ B ≤ 10.8

4	; 	10.8 < B

##### 6.2A.2.2.2.3		PC3 without indicating dualPA-Architecture supported

MPR in this clause is for intra-band non-contiguous CA power class 3 for UEs without indicating IE dualPA-Architecture supported. The allowed maximum output power reduction is defined as:

MPR=MAWhere MA is defined as follows

MA = 	11; 	0 ≤ B < 1.08

10.5; 	1.08 ≤ B < 2.16

10; 	2.16 ≤ B < 3.24

9.5; 	3.24≤ B < 5.04

8.5; 	5.04 ≤ B < 10.08

7.5;      10.08 ≤ B < 36

7; 	36 ≤ B

##### 6.2A.2.2.2.4		PC2 without indicating dualPA-Architecture supported

MPR in this clause is for intra-band non-contiguous CA power class 2 for UEs without indicating IE dualPA-Architecture supported. The allowed maximum output power reduction is defined as:

MPR=MAWhere MA is defined as follows

MA = 	14; 	0 ≤ B < 1.08

12; 	1.08 ≤ B < 2.16

11.5; 	2.16 ≤ B < 3.24

11; 	3.24≤ B < 5.04

9.5; 	5.04 ≤ B < 10.08

8.5;      10.08 ≤ B < 36

6.5; 	36 ≤ B

##### 6.2A.2.2.2.5		PC1.5 with indicating dualPA-Architecture supported

MPR in this clause is for intra-band non-contiguous CA power class 1.5 for UEs with indicating IE dualPA-Architecture supported. The allowed maximum output power reduction is defined as:

MPR= MA,total , where MA,total is defined as follows

MA,total= MA + MA,delta , where MA equals to MA defined in sub clause 6.2A.2.2.2.2 and MA,delta is [1]dB for Handheld UE.

< END OF CHANGE #1 >