3GPP TSG-RAN4 Meeting #115	R4-2506839

Malta, MT, 19th – 23th May, 2025

Agenda item:			7.1.2.1.1

Source:	vivo

Title:	Discussion on HPUE MSD simplification

Document for:	Discussion

- Introduction

In RAN4#114bis meeting, the WF[1] captures the guidelines and assumptions to provide 1UL and 2UL HPUE MSD simplification rules. In this paper, we propose the corresponding 1UL and 2UL simplification schemes based on it.

Agreement: - When the CR for HPUE MSD is implemented (after RAN#109) - All specified PC2 MSD tables and PC1.5 MSD tables remain untouched in TS 38.101-1 and TS38.101-3 - If for some new PC2/1.5 CA configuration there is a need to define a MSD differently from LUTs it can be further discussed how to apply the exception case for the specific band combinations, but the expectation is that this is a rare case. - Handling of the Rel-19 baskets during the extension after CR for HPUE MSD is implemented (after RAN#109): - The procedure is unchanged for the default power class (i.e.PC3): MSD must be studied, and, where applicable, the default PC3 MSD is captured in the TPs for TR Agreement 1 on simplification Look-Up Tables (LUTs): - Scope: In Rel-19, the 1 band UL 1CC HPUE MSD simplification only applies to PC2 1Tx, PC2 2Tx and PC1.5 achieved with 2Tx. Other Rel-19 1UL HPUE power classes are out of scope, e.g. PC1. - For new Rel-19 inter-band CA combinations, and for all valid 1UL HPUE configurations, specify the 1 band 1 UL CC HPUE MSD requirements due to uplink harmonic, harmonic mixing, cross-band isolation caused by the same aggressor UL band as: PCx MSD = PC31Tx MSD + MSD where, - x = 2 1Tx, 22Tx, or 1.52Tx and, MSD is specified in three LUT columns with a maximum MSD (“MSDmax”) of 3, 6 and 9dB for harmonic, cross band isolation, harmonic mixing. One example of such LUT is captured below. Coarser granularity and other tabulated values are not precluded. PC31Tx MSD (dB); MSDmax (dB). 3; 6; 9. 0.1≤ ≤0.3; 0.3; 0.4; 1.0. 0.4≤ ≤0.6; 0.5; 0.8; 1.8. 0.7≤ ≤0.9; 0.7; 1.1; 2.5. 1.0≤ ≤1.9; 1.2; 2.1; 4.1. 2.0≤ ≤2.9; 1.6; 2.7; 5.1. 3.0≤ ≤3.9; 1.8; 3.3; 5.9. 4.0≤ - ≤4.9; 2.0; 3.8; 6.5. 5.0≤ - ≤6.9; 2.3; 4.5; 7.3. 7.0≤ - ≤9.0; 2.5; 5.0; 7.9. ≥10; 3; 6; 9. - and where the mapping of the 1UL MSD type to the “MSDmax” LUT is specified with the following companion table. MSD type; MSDmax (dB). 3; 6; 9. 1UL; UL harmonic; PC21Tx; PC22Tx; [PC1.52Tx]. Harmonic mixing; PC21Tx; PC22Tx; PC1.52Tx. Cross-band isolation; PC21Tx; PC22Tx; PC1.52Tx. - It should be noted that for some band combinations, LUTs may represent a slight tightening of the PC22Tx/PC1.52Tx MSD requirements, especially for UL harmonic, while for other band combinations LUTs may represent a slight relaxation of the PC22Tx/PC1.52Tx MSD requirements, especially for harmonic mixing. - Agreement 2 on handling band combinations without MSD for PC3 but with small PC2/PC1.5 MSD: - For new HPUE band combinations, do not specify the MSD requirements for 1UL HPUE PC21Tx PC22Tx and PC1.5 (2Tx). - For legacy PC3 band combinations (i.e. band combinations specified prior to the agreement of the MSD simplification CR for 1UL HPUE), where no PC3 MSD is specified, companies are encouraged to browse through these band combinations and bring correction CRs if 1UL HPUE MSD requirements are needed. Agreement: - 2UL HPUE MSD requirements can be mapped to a separate LUT with maximum of [8] columns, e.g. at least with ΔMSDmax = [3], 6, 9, 12, 15, [18, 24, 30] dB. - Other values are not precluded and the need for additional columns is FFS. - - FFS on optimized mapping to avoid penalizing 2UL combos with too high ΔMSDmax.

- Discussion

In the last meeting, some progress has been achieved about HPUE MSD simplification. Specifying a total LUT for harmonic, harmonic mixing and cross band isolation is agreed, and the LUT is classified into three columns by ΔMSDmax = 3,6,9 dB.

Based on the framework, the mainly remaining issue for 1UL is how to divide PC3 MSD into different rows, which will decide the granularity of the LUT. And for 2UL there is another important issue is the selection of ΔMSDmax for IMD2~5.

For the IMD2~5, if we consider architecture change, asymmetric PA distribution and IMD index, the ΔMSDmax should involve diverse values, which could be seen in the table below [2].

Table 1. HPUE MSD simplification equation table[3]

| case |  | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| ULCA power class | Type | PC3 2Tx | PC2 2Tx | PC2 2Tx | PC2 3Tx | PC2 2Tx | PC2 3Tx | PC1.5 2Tx | PC1.5 3Tx | PC1.5 3Tx | PC1.5 3Tx |
|  | PCmax | 23 | 26 | 26 | 26 | 26 | 26 | 29 | 29 | 29 | 29 |
| per band power class | Bd1 | PC3 1Tx | PC3 1Tx | PC3 1Tx | PC3 1Tx | PC2 1TX | PC2 1TX | PC2 1TX | PC2 1TX | PC3 1Tx | PC2 1TX |
|  | Bd2 | PC3 1Tx | PC3 1Tx | PC2 1Tx | PC2 2Tx | PC2 1Tx | PC2 2Tx | PC2 1Tx | PC2 2Tx | PC1.5 2Tx | PC1.5 2Tx |
| Valid Release |  | R15 | R17 | R17 | R18 | R19 | R19 | R19 | R19 | R18 | R19 |
| 1UL power | Bd1 | 23 | 23 | 23 | 23 | 26 | 26 | 26 | 26 | 23 | 26 |
|  | Bd2 | 23 | 23 | 26 | 2x23 | 26 | 2x23 | 26 | 2x23 | 2x26 | 2x26 |
| 2UL power | Bd1 | 20 | 23 | 23 | 23 | 23 | 23 | 26 | 26 | 23 | 26 |
|  | Bd2 | 20 | 23 | 23 | 2x20 | 23 | 2x20 | 26 | 2x23 | 2x24.8 | 2x23 |
| 2UL interference level |  | Ref | +3xorder | <+3xorder | <+3xorder | +3dB | +3dB | +3dB +3xorder | +3dB +3xorder | <4.8¦b¦+3n | ≤+3dB +3xorder |
| Unified 2UL interference level |  | Ref | 3order |  |  | 3dB |  | +3dB +3xorder |  | 4.8¦b¦+3n | +3dB +3xorder |

However, as some companies mentioned in 114bis meeting, the 2UL IMD MSD is rather complicated with unequal power configuration. In the spirit of HPUE MSD simplification, it is proposed RAN4 to only specify 2UL IMD MSD with equal UL power, that is, (20+20) for PC3, (23+23) for PC2, and (26+26) for PC1.5. [3] Hence it should also be considered that the categories of different architectures would not make big difference in testing, for each combination of power class + IMD order only specifying one ΔMSD value is a simplest and compromise solution.

From one side, from PC3 to PC2 or PC1.5, the linearity of PA should be considered already optimized when calculating MSD, but if the optimization is ignored than the calculation would be easier, i.e., ΔMSD =3n for PC2 and 6n for PC1.5. And the corresponding specific ΔMSDmax value for IMD2~5 is 6,9,12,15 for PC2 and 12,18,24,30 for PC1.5.

From another side, even if for PC2 there is MSDmax = 3 in the R19 architecture for PC2+PC2, as shown in case 5&6 in Table 1, the column should be deleted from 2UL companion table if we consider to combine MSD of 1UL and 2UL into one LUT. Because as can be seen that for IMD2&3, the PC3 MSD is almost always >10dB and the corresponding MSD of PC2 always enters the saturation region and takes the value of 3dB. However, for IMD4&5, there is still some small PC3 MSD that would cause PC2 MSD to be smaller than 3dB and share the same value as 1UL. And it will cause the MSD values for both 1UL and 2UL to become imprecise.

Observation 1: If considering the accuracy of the joint LUT for 1UL&2UL, MSDmax = 3 for PC2 should be deleted from companion table of 2UL.

Observation 2: If the optimization of linearity of PA, architecture change, asymmetric PA distribution and IMD index could be ignored, then MSDmax of IMD2~5 is much simpler: 6,9,12,15 for PC2 and 12,18,24,30 for PC1.5.

Proposal 1: Consider MSDmax of IMD2~5 = 6,9,12,15 for PC2 and 12,18,24,30 for PC1.5.

Proposal 2: The companion table could be considered as the following format:

Companion table for 1UL and 2UL

| MSD type |  | MSDmax (dB) |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 3 | 6 | 9 | 12 | 15 | 18 | 24 | 30 |
| 1UL | UL harmonic | PC21Tx | PC22Tx | [PC1.52Tx] |  |  |  |  |  |
|  | Rx mixing | PC21Tx | PC22Tx | PC1.52Tx |  |  |  |  |  |
|  | Cross-band isolation | PC21Tx | PC22Tx | PC1.52Tx |  |  |  |  |  |
| 2UL | IMD2 |  | PC2 |  | PC1.52Tx |  |  |  |  |
|  | IMD3 |  |  | PC2 |  |  | PC1.52Tx |  |  |
|  | IMD4 |  |  |  | PC2 |  |  | PC1.52Tx |  |
|  | IMD5 |  |  |  |  | PC2 |  |  | PC1.52Tx |

For LUT part, from [2] it could be seen that if we consider to capture the HPUE MSD of 1UL and 2UL into one joint LUT, which is a highly-integrated scheme for HPUE MSD simplification, the granularity issue should be considered more carefully, mainly referring to the columns where ΔMSDmax = 3,6,9 as the three columns are shared between 1UL and 2UL. Our preference is to maintain a certain level of accuracy for the merged LUT.

And the recommended LUT for both 1UL and 2UL in [4] is captured and changed as follows, most of the MSD is not changed but the PC3 MSD range in the second-to-last line has been expanded to 9.9 dB:

LUT for both 1UL and 2UL[4]

| PC31Tx MSD (dB) | MSDmax (dB) |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 3 | 6 | 9 | 12 | 15 | 18 | 24 | 30 |
| 0.1≤ ≤0.3 | 0.3 | 0.4 | 1.0 | 2.8 | 4.3 | 6.3 | 11.0 | 16.5 |
| 0.4≤ ≤0.6 | 0.5 | 0.8 | 1.8 | 4.2 | 6.1 | 8.4 | 13.6 | 19.3 |
| 0.7≤ ≤0.9 | 0.7 | 1.1 | 2.5 | 5.1 | 7.2 | 9.7 | 15.1 | 21.0 |
| 1.0≤ ≤1.9 | 1.2 | 2.1 | 4.1 | 7.1 | 9.6 | 12.3 | 18.1 | 24.0 |
| 2.0≤ ≤2.9 | 1.6 | 2.7 | 5.1 | 8.2 | 10.9 | 13.7 | 19.6 | 25.5 |
| 3.0≤ ≤3.9 | 1.8 | 3.3 | 5.9 | 8.9 | 11.7 | 14.6 | 20.5 | 26.5 |
| 4.0≤ - ≤4.9 | 2.0 | 3.8 | 6.5 | 9.5 | 12.4 | 15.3 | 21.3 | 27.3 |
| 5.0≤ - ≤6.9 | 2.3 | 4.5 | 7.3 | 10.4 | 13.4 | 16.3 | 22.3 | 28.3 |
| 7.0≤ - ≤9.9 | 2.5 | 5.0 | 7.9 | 11.0 | 14.0 | 16.9 | 22.9 | 28.9 |
| ≥10 | 3 | 6 | 9 | 12 | 15 | 18 | 24 | 30 |

- Conclusion

Observation 1: If considering the accuracy of the joint LUT for 1UL&2UL, MSDmax = 3 for PC2 should be deleted from companion table of 2UL.

Observation 2: If the optimization of linearity of PA, architecture change, asymmetric PA distribution and IMD index could be ignored, then MSDmax of IMD2~5 is much simpler: 6,9,12,15 for PC2 and 12,18,24,30 for PC1.5.

Proposal 1: Consider MSDmax of IMD2~5 = 6,9,12,15 for PC2 and 12,18,24,30 for PC1.5.

Proposal 2: The companion table could be considered as the following format:

Companion table for 1UL and 2UL

| MSD type |  | MSDmax (dB) |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 3 | 6 | 9 | 12 | 15 | 18 | 24 | 30 |
| 1UL | UL harmonic | PC21Tx | PC22Tx | [PC1.52Tx] |  |  |  |  |  |
|  | Rx mixing | PC21Tx | PC22Tx | PC1.52Tx |  |  |  |  |  |
|  | Cross-band isolation | PC21Tx | PC22Tx | PC1.52Tx |  |  |  |  |  |
| 2UL | IMD2 |  | PC2 |  | PC1.52Tx |  |  |  |  |
|  | IMD3 |  |  | PC2 |  |  | PC1.52Tx |  |  |
|  | IMD4 |  |  |  | PC2 |  |  | PC1.52Tx |  |
|  | IMD5 |  |  |  |  | PC2 |  |  | PC1.52Tx |

- References

- R4-2420380, WF on MSD rule, Skyworks, Murata, Nokia, Qualcomm, RAN4#114bis.

- R4-2503839, Discussion on HPUE MSD simplification, vivo, RAN4#114bis.

- R4-2503196, On Rel-19 HPUE CA DC MSD consideration, Apple, RAN4#114bis.

- R4-2503239, Unified HPUE 1UL 2UL MSD simplification proposals for Rel-19, 3GPP TSG-RAN WG4 Meeting # 114bis, Wuhan, China, Skyworks Solutions Inc., Samsung.