3GPP TSG-RAN WG4 Meeting # 115	R4-2506840

Malta, MT, 19th – 23rd May, 2025

Source: 	vivo

Title: 	Discussion on the system parameter of AIoT

Agenda Item:	7.24.2

Document for:	Approval

- Introduction

In this contribution, we provide our views on the system parameter design for AIoT.

- Discussion

  - System parameter for R2D

- R2D Channel raster

In band n8, the NR channel raster is 100kHz and recall that in previous release, the enhanced channel raster with 10kHz granularity was introduced due to the issue illustrated in Figure I:

Figure I Motivation of enhanced channel raster

Since the in-band operation also need to be supported, and similar issue may also happen if the RB number of BS CBW is even/odd but the AIoT RB number is odd/even.

Observation 1: Similar problem in enhance channel raster WI may happen for AIoT in-band operation if only 100kHz channel raster is defined for R2D in band n8.

Proposal 1: For standalone operation, reuse same channel raster as NR in corresponding band, i.e., 100kHz in band n8. For in-band operation, the enhanced channel raster is used, i.e., 10kHz.

2.2 System parameter for D2R

- D2R Channel bandwidth

In the last meeting, the potential value of D2R chip during was discussed in RAN1 as listed below:

|  | Tchip (μs) |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Tb (μs) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 133.33 | 66.67 | 33.33 | 16.67 | 11.11 | 8.33 | 5.56 | 4.17 | 2.78 | 2.08 | 1.39 | 1.04 | 0.69 | 0.52 |
| 266.67 | R=1 | R=2 | R=4 | R=8 | R=12 | R=16 | R=24 | R=32 | R=48 | R=64 | R=96 | R=128 |  | R=256 |
| 133.33 |  | R=1 | R=2 | R=4 | R=6 | R=8 | R=12 | R=16 | R=24 | R=32 | R=48 | R=64 | R=96 | R=128 |
| 66.67 |  |  | R=1 | R=2 |  | R=4 | R=6 | R=8 | R=12 | R=16 | R=24 | R=32 | R=48 | R=64 |
| 33.33 |  |  |  | R=1 |  | R=2 |  | R=4 | R=6 | R=8 | R=12 | R=16 | R=24 | R=32 |
| 22.22 |  |  |  |  | R=1 |  | R=2 |  | R=4 |  | R=8 |  | R=16 |  |
| 16.67 |  |  |  |  |  | R=1 |  | R=2 |  | R=4 | R=6 | R=8 | R=12 | R=16 |
| 11.11 |  |  |  |  |  |  | R=1 |  | R=2 |  | R=4 |  | R=8 |  |
| 8.33 |  |  |  |  |  |  |  | R=1 |  | R=2 |  | R=4 | R=6 | R=8 |
| 5.56 |  |  |  |  |  |  |  |  | R=1 |  | R=2 |  | R=4 |  |
| 4.17 |  |  |  |  |  |  |  |  |  | R=1 |  | R=2 |  | R=4 |
| 2.78 |  |  |  |  |  |  |  |  |  |  | R=1 |  | R=2 |  |
| 2.08 |  |  |  |  |  |  |  |  |  |  |  | R=1 |  | R=2 |
| 1.39 |  |  |  |  |  |  |  |  |  |  |  |  | R=1 |  |
| 1.04 |  |  |  |  |  |  |  |  |  |  |  |  |  | R=1 |
| Note: For further down-selection regarding the bit duration, - bit duration 266.67μs, 133.33μs and 66.67μs to be kept, which provides better multiplexing capability in the frequency domain. - Down-select to 1 between bit duration 1.39μs and 1.04μs, which achieves the competitive peak data rate larger than 640 kbps. - The remaining bit durations will be further discussed. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Although RAN1 still doesn’t make the final decision, RAN4 can take the table above as starting point for D2R channel bandwidth. The following table list all potential D2R transmission bandwidth (Btx,D2R) and channel bandwidth (BCH,D2R). The difference between channel bandwidth and transmission bandwidth is 10% SFO is counted as guard band.

| Tb (μs) | 266.67 | 133.33 | 66.67 | 33.33 | 22.22 | 16.67 | 11.11 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Btx,D2R (kHz) | 15 | 30 | 60 | 120 | 180 | 240 | 360 |
| BCH,D2R (kHz) | 15.75 | 31.5 | 63 | 126 | 189 | 252 | 378 |
| Tb (μs) | 8.33 | 5.56 | 4.17 | 2.78 | 2.08 | 1.39 | 1.04 |
| Btx,D2R (kHz) | 480 | 720 | 960 | 1440 | 1920 | 2880 | 3840 |
| BCH,D2R (kHz) | 504 | 756 | 1008 | 1512 | 2016 | 3024 | 4032 |

The table above can be the starting point for RAN4 and can be further updated based on RAN1 progress.

Proposal 2: Take the following table as the starting point of D2R channel bandwidth and transmission bandwidth:

| Btx,D2R (kHz) | 15 | 30 | 60 | 120 | 180 | 240 | 360 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| BCH,D2R (kHz) | 16 | 32 | 63 | 126 | 189 | 252 | 378 |
| Btx,D2R (kHz) | 480 | 720 | 960 | 1440 | 1920 | 2880 | 3840 |
| BCH,D2R (kHz) | 504 | 756 | 1008 | 1512 | 2016 | 3024 | 4032 |
| Note: Further update is needed based on RAN1 progress |  |  |  |  |  |  |  |

- Conclusion

In this contribution, we share our views on AIoT system parameters

Observation 1: Similar problem in enhance channel raster WI may happen for AIoT in-band operation if only 100kHz channel raster is defined for R2D in band n8.

Proposal 1: For standalone operation, reuse same channel raster as NR in corresponding band, i.e., 100kHz in band n8. For in-band operation, the enhanced channel raster is used, i.e., 10kHz.

Proposal 2: Take the following table as the starting point of D2R channel bandwidth and transmission bandwidth:

| Btx,D2R (kHz) | 15 | 30 | 60 | 120 | 180 | 240 | 360 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| BCH,D2R (kHz) | 16 | 32 | 63 | 126 | 189 | 252 | 378 |
| Btx,D2R (kHz) | 480 | 720 | 960 | 1440 | 1920 | 2880 | 3840 |
| BCH,D2R (kHz) | 504 | 756 | 1008 | 1512 | 2016 | 3024 | 4032 |
| Note: Further update is needed based on RAN1 progress |  |  |  |  |  |  |  |

- References

- R4-2502859 WF on requirements for A-IoT BS and CW, Huawei, RAN4#114

- R1-2503023 Summary #4 for coding aspects of physical channel design	Moderator (CMCC)