3GPP TSG-RAN WG4 Meeting # 115 	R4-2506841

Malta, MT, 19th – 23rd May, 2025

Source: 	vivo

Title: 	Discussion on RF requirement for AIoT BS

Agenda Item:	7.26.3.1

Document for:	Approval

- Introduction

In this contribution, we share our analysis and views on the pending requirements for AIoT BS.

- Discussion

  - Tx Requirements

- Modulation quality

In the previous meeting, the following requirements from RFID need to be further discussed as modulation quality:

- Modulation Depth

- RF Envelope Ripple

- RF Envelope Rise Time

- RF Envelope Fall Time

- RF Pulse width

As we mentioned in the last meeting, the modulation quality for R2D is related to transmission bandwidth and larger bandwidth can provide better waveform shape under same data rate. In TR 38.769, the minimum transmission bandwidth for each M value is listed below:

| M | Minimum Btx,R2D # of PRBs |
| --- | --- |
| 1 | 1 |
| 2 | 1 |
| 4 | 1 |
| 6 | 1 |
| 8 | 2 |
| 12 | 2 |
| 16 | 2 |
| 24 | 3 |
| 32 | 4 |

In the last meeting, RAN1 also agreed that the maximum M value is 24. From modulation quality perspective, it is unnecessary to verify each data rate, and only define the requirement for each candidate transmission bandwidth with corresponding highest M value should be enough, e.g., 1 PRB with M = 6.

Observation 1: The waveform shape of R2D is related to the combination of transmission bandwidth and data rate, and larger transmission bandwidth can provide better OOK envelope under same date rate.

Proposal 1: For each candidate transmission bandwidth, the modulation quality is only defined for maximum M value, as listed below：

| R2D transmission bandwidth | M value |
| --- | --- |
| 1 PRB | 6 |
| 2 PRB | 16 |
| 3 PRB | 24 |

As for the details of requirement, the definition and the value in RFID spec can be the starting point, but the value can be further updated since the R2D is OFDM based which is quite different with RFID reader.

Proposal 2: Take the RF envelope parameter in RFID spec as starting point and FFS whether the values need to be updated considering the R2D is OFDM based.

| Parameter | Symbol | Minimum | Maximum | Units |
| --- | --- | --- | --- | --- |
| Modulation Depth | (A–B)/A | 80 | 100 | % |
| RF Envelope Ripple | Mh = Ml | 0 | 0.05(A–B) | V/m or A/m |
| RF Envelop Rise Time | Tr,10-90 | 0 | 0.33Tari | µs |
| RF Envelop Fall Time | Tf,10-90 | 0 | 0.33Tari | µs |
| RF Pulsewidth | PW | 0.265 Tari | 0.525Tari | µs |
| - Note 1: Tari= 103M*15 (us) Note 2: Reference figure is provided below |  |  |  |  |

Traditionally, the signal quality requirement is averaged over 10 subframes and also can be considered for applying to AIoT.

Proposal 3: The modulation quality of R2D is also averaged over 10 subframes.

  - Rx Requirements

- REFSENS

In legacy NR simulation, the SNR is calculated over per RE and the target SNR is same for different CBW since the OFDM waveform is quite flat in frequency domain. However, for D2R which is OOK waveform, the SNR is calculated based overall D2R transmission bandwidth, it is questionable whether the SNR need to be differentiated for the BWs defined in RAN1. Figure 1 shows the simulation results for different D2R transmission bandwidth when SFO is not considered.

Figure I BLER under different D2R transmission bandwidth without SFO

The simulation shows that although the D2R use different SNR calculation method, the target SNR is still similar for different D2R bandwidth.

Observation 2: Even though the SNR definition is different compared to the NR, the target SNR@10% for different D2R transmission bandwidth is still similar when SFO is not counted.

After adding SFO, the simulation results are shown in figure II

Figure I BLER under different D2R transmission bandwidth with SFO

The simulation shows that for smaller D2R bandwidth, the reception performance will be better when the D2R bandwidth is larger, and the reason here is the SFO compensation is harder for smaller BLF and the residual SFO dominates the performance. However, when the BW keep become larger, the performance will be worse since under same residual SFO, the influence is bigger for larger CBW due to shorter chip duration. Based on the simulation above, the simulation results for 15kHz can be chosen as the worst case to derive the target SNR and our results shows the target SNR is -8 dB. Leave some margin here, we propose -7 dB can be considered as final value.

Observation 3: The SFO compensation is harder for smaller D2R transmission bandwidth.

Proposal 4: The SNR@10% BLER for 15kHz D2R bandwidth can be used for all other BW, and -7 dB can be considered.

In the simulation assumption, both OOK and BPSK are exist. From BS perspective, there is no different for these two modulations and the coherent detection is used for both, so it is unnecessary to define different REFSENS for OOK and BPSK, and the OOK is enough which is similar to the legacy approach in NR.

Observation 4: From BS perspective, both OOK and BPSK use coherent detection.

Proposal 5: No need to define different REFSESN for OOK and BPSK, the REFSENS is defined based on OOK only.

Another pending factor of REFSENS is the desens due to CW interference. During SI, we share our view on the CW cancellation capability and for D1T1-B around 160 dB cancellation could be achieved, and the residual CW interference could be -127 dBm. Based on the simulation above, the REFSESN and desens for different D2R transmission BW can be calculated as below:

| D2R BW (kHz) | 15 | 30 | 60 | 120 | 180 | 240 | 360 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| REFSENS (dBm) | -128.239 | -125.229 | -122.218 | -119.208 | -117.447 | -116.198 | -114.437 |
| Desens (dB) | 5.63 | 3.67 | 2.21 | 1.24 | 0.87 | 0.67 | 0.46 |
| D2R BW (kHz) | 480 | 720 | 960 | 1440 | 1920 | 2880 | 3840 |
| REFSENS (dBm) | -113.188 | -111.427 | -110.177 | -108.416 | -107.167 | -105.406 | -104.157 |
| Desens (dB) | 0.35 | 0.23 | 0.18 | 0.12 | 0.09 | 0.06 | 0.04 |

Take some margin into account, we propose the desens in table below.

Proposal 6: The desens due to CW in table below can be considered for REFSENS.

| D2R BW (kHz) | 15 | 30 | 60 | 120 | 180 | 240 | 360 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Desens (dB) | 6 | 4 | 2.5 | 1.5 | 1 | 1 | 0.5 |
| D2R BW (kHz) | 480 | 720 | 960 | 1440 | 1920 | 2880 | 3840 |
| Desens (dB) | 0.5 | 0.5 | 0.2 | 0.2 | 0.1 | 0.1 | 0.1 |

However, a problem raised in the last meeting is how to reflect the CW influence in the conformance test. Normally the OOK signal can be generated by the TE and CW interference is not added. It is better to further discussed whether and how the CW densen can be reflected.

Observation 5: The desens due to CW that evaluated in the simulation may not be easily mapped to the test.

Proposal 7: It is suggested to discussed whether and how to reflect the influence of CW in the conformance test.

- Conclusion

In this contribution, we provide our views on the AIoT BS RF requirements.

Observation 1: The waveform shape of R2D is related to the combination of transmission bandwidth and data rate, and larger transmission bandwidth can provide better OOK envelope under same date rate.

Proposal 1: For each candidate transmission bandwidth, the modulation quality is only defined for maximum M value, as listed below：

| R2D transmission bandwidth | M value |
| --- | --- |
| 1 PRB | 6 |
| 2 PRB | 16 |
| 3 PRB | 24 |

Proposal 2: Take the RF envelope parameter in RFID spec as starting point and FFS whether the values need to be updated considering the R2D is OFDM based.

| Parameter | Symbol | Minimum | Maximum | Units |
| --- | --- | --- | --- | --- |
| Modulation Depth | (A–B)/A | 80 | 100 | % |
| RF Envelope Ripple | Mh = Ml | 0 | 0.05(A–B) | V/m or A/m |
| RF Envelop Rise Time | Tr,10-90 | 0 | 0.33Tari | µs |
| RF Envelop Fall Time | Tf,10-90 | 0 | 0.33Tari | µs |
| RF Pulsewidth | PW | 0.265 Tari | 0.525Tari | µs |
| - Note 1: Tari= 103M*15 (us) Note 2: Reference figure is provided below |  |  |  |  |

Proposal 3: The modulation quality of R2D is also averaged over 10 subframes.

Observation 2: Even though the SNR definition is different compared to the NR, the target SNR@10% for different D2R transmission bandwidth is still similar when SFO is not counted.

Observation 3: The SFO compensation is harder for smaller D2R transmission bandwidth.

Proposal 4: The SNR@10% BLER for 15kHz D2R bandwidth can be used for all other BW, and -7 dB can be considered.

Observation 4: From BS perspective, both OOK and BPSK use coherent detection.

Proposal 5: No need to define different REFSESN for OOK and BPSK, the REFSENS is defined based on OOK only.

Proposal 6: The desens due to CW in table below can be considered for REFSENS.

| D2R BW (kHz) | 15 | 30 | 60 | 120 | 180 | 240 | 360 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Desens (dB) | 6 | 4 | 2.5 | 1.5 | 1 | 1 | 0.5 |
| D2R BW (kHz) | 480 | 720 | 960 | 1440 | 1920 | 2880 | 3840 |
| Desens (dB) | 0.5 | 0.5 | 0.2 | 0.2 | 0.1 | 0.1 | 0.1 |

Observation 5: The desens due to CW that evaluated in the simulation may not be easily mapped to the test.

Proposal 7: It is suggested to discussed whether and how to reflect the influence of CW in the conformance test.

- References

- R4-2505097 	 WF on A-IoT BS and CW requirements	Huawei