3GPP TSG-RAN WG4 Meeting # 115 	R4-2506842

Malta, MT, 19th – 23rd May, 2025

Source: 	vivo

Title: 	Discussion on the RF requirement for device 1

Agenda Item:	7.24.3.2

Document for:	Approval

- Introduction

In this contribution, we provide our views on the RF requirements for device1.

- Discussion

  - Tx Requirement

- Backscatter loss

In the last meeting, the formula of transmit power for AIoT device was agreed:

Minimum Backscattering power at device antenna= Input CW Tx power at device antenna-Backscattering loss

To conclude the requirement, the input CW power and backscattering loss need to be defined. As we mentioned in the last meeting, the power transmission coefficient of backscatter is described as below:

ZA is the antenna impedance and the ZL is the load antenna. The formula shows the backscatter efficiency only depends on the mismatch between ZA and ZL. When ZA and ZL are the complex-conjugate with each other, the backscatter power can be maximized.

Observation 1: The backscatter loss depends on the mismatch between antenna impedance and load impedance which should be similar for different incoming CW power.

So, it should be enough to only define one CW power configuration and no need to consider a range of CW which could further burden the device 1 tag.

Proposal 1: Only one incoming CW power level configuration at device antenna is enough to verify the backscatter loss.

In [1], there are a lot of factors of backscatter loss listed, including modulation process, antenna efficiency, etc. All these factors are valid but it could be risky to define the backscatter loss from device side since the device type and use case could be various, and the whole market is not clear enough at this stage. The 15m coverage target may not achieve if only consider this requirement at device side.

Observation 2: The factors of influencing the backscatter loss is various and the 15m coverage target is shared between device and reader, which make it would be difficult conclude the backscatter loss from device perspective only.

Another safe choice is to use the similar way as REFSENS. We agreed that the REFSENS of device is derived based on the link budget with 15m coverage target of R2D, and the backscatter loss can also be derived based on same coverage of D2R when the D2R power at reader is minimized.

Proposal 2: The backscatter loss is also derived based on link-budget with 15m coverage target, and the following formula can be used:

Backscatter loss=minimum D2R@Reader(no backscatter loss)-REFSESNSReader+IM

In the following part we further discuss how to get the minimum D2R power at reader. During SI, the layout for D1T1-B was agreed as below:

| BS deployment | 18 BSs on a square lattice with spacing D, located D/2 from the walls. L=120m x W=60m; D=20m BS height = 8 m |
| --- | --- |
| Device distribution | Device Height= 1.5 m A-IoT devices drop uniformly distributed over the horizontal area Minimum distance between reader and device is 1m |
| CW deployment | For D1T1-B and D2T2-B, CW topology layout is the same as A-IoT reader. For each device, the nearest CW node will be activated during the simulation, and CW node is not co-located with any activated reader |

Since the CW node is not co-located with AIoT reader in R19, we cannot directly conclude the position where the D2R power can be minimized because when device moves away from the reader, the input CW power could be higher and the D2R power at reader may not be reduced. The Figure I show the power distribution of CW input power and D2R power at reader based on the assumption above. The pathloss model is InF-DH NLOS. 4 dB shadow fading, 3 dB polarization mismatch and 0 dB backscatter loss are assumed.

Figure I D2R power at reader (Left) and input CW power at device (Right) when devices distributed within 15m coverage

Based on the simulation, the minimum D2R power at reader is around -98.6 dBm when no backscatter loss, and the maximum CW input power at device is around -26.7 dBm

Observation 3: Based on the D1T1-B topology assumption agreed in SI, the minimum D2R power received at reader is -98.6 dBm within 15m coverage target when 0 dBm backscatter loss is assumed.

Based on our simulation results in [2], the SNR@10% BLER of D2R reception is around -7 dB. Assuming 6 dB desens due to CW, the backscatter loss for different D2R bandwidth is listed below:

| D2R transmission BW | 15 kHz | 30 kHz | 60 kHz | 120 kHz | 240 kHz | 480 kHz |
| --- | --- | --- | --- | --- | --- | --- |
| REFSENS@ reader | -122.24 | -119.23 | -116.22 | -113.21 | -110.20 | -107.19 |
| Backscatter loss | 25.64 | 22.63 | 19.62 | 16.61 | 13.60 | 10.59 |

The allowed backscatter loss will be smaller for larger D2R transmission bandwidth. However, the 15m is the maximum coverage target and it is unnecessary to force all BW can achieve such performance. To have some freedom of data rate selection even at maximum coverage, we propose 10 dB backscatter loss.

Proposal 4: Take the 10 dB backscatter loss for device 1 as starting point.

- Transmit signal quality

One of the modulation quality requirements is the sampling frequency error (SFO). As we discussed in the last meeting, the SFO is not a typical RF requirement since the SFO mainly comes from MCU rather than the RF components.

Observation 4: The SFO (sampling frequency offset) is mainly caused by MCU operation rather than imperfect RF component.

In addition, the SFO will impact the D2R spectrum and if the SFO is too worse, the power of D2R will leak out of the bandwidth and the emission requirements will be failed, which means the SFO can be implicitly verified in the unwanted emission.

Proposal 5: The 10% SFO can be reflected in D2R channel bandwidth and implicitly verified in unwanted emission. No need to define a dedicated RF requirement for SFO.

- Unwanted emission

As we mentioned in the previous meeting, In ITU-R SM328, the occupied bandwidth is applied to the device with amplifier, not for passive device

Classes of emission A1A and A1B with fluctuations

When large short-period variations of the received field are present, the specifications given below for single-channel, amplitude-modulated, continuous-wave telegraphy (Class A1A and A1B), represent the desirable performance obtainable from a transmitter with an adequate input filter and sufficiently linear amplifiers following the stage in which keying occurs

Observation 5: ITU-R has no clear occupied bandwidth limitation for passive device type.

Our concern on defining such requirement is that the OOK naturally has poor spectrum efficiency. For a periodic square wave, it can be expanded using the Fourier series

The harmonic has quite high power and it may need include 5th harmonic into D2R channel bandwidth to achieve 99% occupied bandwidth, which may lead to a wide D2R bandwidth and waste the spectrum resource.

Observation 5: To achieve 99% occupied bandwidth, at least 5th harmonic needs to be included in the D2R Channel bandwidth.

Companies may have concern on the interference to other radio system, but this issue can also be solved by defining as proper emission mask.

Proposal 6: The occupied bandwidth is not defined for AIoT device and the concern on the impact to other radio system can be addressed by emission mask.

As we analyzed above, the hormonic of D2R could be the emission that causes the greatest out-of-band interference, and the PSD of harmonic will higher when the D2R transmission bandwidth is smaller when the D2R total is fixed, so the 15kHz can be used as the worst case for SEM.

Observation 6: Considering the total backscatter power is fixed, the PSD of harmonic will be higher for smaller D2R transmission bandwidth, which could be the worst case.

Proposal 7: The emission requirement is only verified with 15 kHz transmission bandwidth.

The spectrum regrowth due to the component nonlinearity is considered when define the SEM, but for device 1, such regrowth could be weak and it may not necessary to define a mask shape requirement. In addition, the input CW power as we analyzed above may only -26.7 dBm, the emission could be naturally less than NR spurious emission requirement. We suggest to reuse the spurious emission requirement directly to device

Proposal 8: The following emission requirement is defined for device 1.

| Frequency Range | Maximum Level | Measurement bandwidth |
| --- | --- | --- |
| 9 kHz f < 150 kHz | -36 dBm | 1 kHz |
| 150 kHz f < 30 MHz | -36 dBm | 10 kHz |
| 30 MHz f < 1000 MHz | -36 dBm | 100 kHz |
| 1 GHz f < 12.75 GHz | -30 dBm | 1 MHz |
| Note: The requirement is not applied within D2R channel bandwidth |  |  |

Based on the simulation in the previous part, the maximum input CW power based on the SI assumption is -26.7 dBm. Take some margin in to account, -25 dBm can be chosen as side condition for both emission requirement or backscatter loss.

Observation 7: Based on the D1T1-B topology assumption agreed in SI, the maximum CW input power at device is -26.7 dBm within 15m coverage.

Proposal 9: The input CW power for emission and backscatter loss is set to -25 dBm as side condition.

  - Rx Requirement

- REFSENS

- Define minimum requirements of REFSENS based on link budget (target is [15m]) and measurement results reported by companies

- Consider to use inventory procedure for REFSENS testing, success rate ([90%]) or miss detection rate ([10%]) as the metric for inventory

- At least to define maximum gain direction sensitivity requirements

- FFS on how to different sensitivity requirements on different orientations

In [1], we agreed that:

The link budget for REFSENS is provided in the table below:

| R2D power | 38 dBm |  |
| --- | --- | --- |
| 15m Pathloss | 59.3 dB | 3D distance with InF-DH NLOS |
| Shadow fading | 4 dB |  |
| Polarization mismatch | 3 dB |  |
| Implementation margin | 2 dB |  |
| REFSENS | 30.3 dBm |  |

Based on the link budget, the REFSENS of device 1 can be -30 dBm

Proposal 10: The REFESNS of device 1 is defined as -30 dBm.

- Sensitivity in different orientation

In the last meeting, some companies propose to consider introduce spherical coverage to guarantee the device performance at different direction. To further check the necessity of such requirement, an example meander line dipole antenna is designed which is commonly used in RFID, as show in Figure II

Figure II Example meander line antenna

The maximum gain is around 2 dBi and the spherical coverage for this antenna is shown in Figure III

Figure III spherical coverage for example antenna

The simulation shows that the antenna gain degradation@50% CDF is only around 1.5 dB, which make such requirement is not useful

Observation 8: The antenna gain degradation @ 50% CDF is only 1.5 dB for a dipole antenna, which is commonly used for RFID.

Proposal 11: No need to define sensitivity requirements in different orientation.

- Maximum input power

During SI stage, 1m is assumed as the minimum distance between reader and device and R2D power can be calculated with same link budget as REFSENS by replacing the distance from 15m to 1m:

38 dBm – 50.7dB -3dB -4dB = -19.7 dBm

In the last meeting, the square-law of diode when performing envelope detection is proposed as one of the considerations when defining the maximum input power. In [3], the boundary of square law is around -20 dBm typically.

Figure IV: When operated at powers below −20 dBm (10μW) the Schottky diode is in the

square law range [3]

Observation 9: Based on 1m minimum distance, the R2D power at device is around -19.7 dBm which is close to the boundary of Schottky square-law, i.e., -20 dBm typically.

Proposal 12: The maximum input power of device is -20 dBm

  - Metric of RF requirements

One important issue still pending is the metric for the device requirement, e.g., EIRP based or TRP based and we provide our suggestion in table below:

| Backscatter loss | EIRP based, at peak direction |
| --- | --- |
| Transmit signal quality | EIRP based (if applicable), at peak direction |
| Unwanted emission | TRP based |
| REFSENS | EIS based, at peak direction |
| Maximum input power | EIS based, at peak direction |

Proposal 13: Take the following table as baseline for device RF requirement design.

| Backscatter loss | EIRP based, at peak direction |
| --- | --- |
| Transmit signal quality | EIRP based (if applicable), at peak direction |
| Unwanted emission | TRP based |
| REFSENS | EIS based, at peak direction |
| Maximum input power | EIS based, at peak direction |

- Conclusion

In this contribution, we further update views on AIoT device RF requirements.

Observation 1: The backscatter loss depends on the mismatch between antenna impedance and load impedance which should be similar for different incoming CW power.

Proposal 1: Only one incoming CW power level configuration at device antenna is enough to verify the backscatter loss.

Observation 2: The factors of influencing the backscatter loss is various and the 15m coverage target is shared between device and reader, which make it would be difficult conclude the backscatter loss from device perspective only.

Proposal 2: The backscatter loss is also derived based on link-budget with 15m coverage target, and the following formula can be used:

Backscatter loss=minimum D2R@Reader(no backscatter loss)-REFSESNSReader+IM

Observation 3: Based on the D1T1-B topology assumption agreed in SI, the minimum D2R power received at reader is -98.6 dBm within 15m coverage target when 0 dBm backscatter loss is assumed.

Proposal 4: Take the 10 dB backscatter loss for device 1 as starting point.

Observation 4: The SFO (sampling frequency offset) is mainly caused by MCU operation rather than imperfect RF component.

Proposal 5: The 10% SFO can be reflected in D2R channel bandwidth and implicitly verified in unwanted emission. No need to define a dedicated RF requirement for SFO.

Observation 5: ITU-R has no clear occupied bandwidth limitation for passive device type.

Observation 6: To achieve 99% occupied bandwidth, at least 5th harmonic needs to be included in the D2R Channel bandwidth.

Proposal 6: The occupied bandwidth is not defined for AIoT device and the concern on the impact to other radio system can be addressed by emission mask.

Observation 6: Considering the total backscatter power is fixed, the PSD of harmonic will be higher for smaller D2R transmission bandwidth, which could be the worst case.

Proposal 7: The emission requirement is only verified with 15 kHz transmission bandwidth.

Proposal 8: The following emission requirement is defined for device 1.

| Frequency Range | Maximum Level | Measurement bandwidth |
| --- | --- | --- |
| 9 kHz f < 150 kHz | -36 dBm | 1 kHz |
| 150 kHz f < 30 MHz | -36 dBm | 10 kHz |
| 30 MHz f < 1000 MHz | -36 dBm | 100 kHz |
| 1 GHz f < 12.75 GHz | -30 dBm | 1 MHz |
| Note: The requirement is not applied within D2R channel bandwidth |  |  |

Observation 7: Based on the D1T1-B topology assumption agreed in SI, the maximum CW input power at device is -26.7 dBm within 15m coverage.

Proposal 9: The input CW power for emission and backscatter loss is set to -25 dBm as side condition.

Proposal 10: The REFESNS of device 1 is defined as -30 dBm.

Observation 8: The antenna gain degradation @ 50% CDF is only 1.5 dB for a dipole antenna, which is commonly used for RFID.

Proposal 11: No need to define sensitivity requirements in different orientation.

Observation 9: Based on 1m minimum distance, the R2D power at device is around -19.7 dBm which is close to the boundary of Schottky square-law, i.e., -20 dBm typically.

Proposal 12: The maximum input power of device is -20 dBm

Proposal 13: Take the following table as baseline for device RF requirement design.

| Backscatter loss | EIRP based, at peak direction |
| --- | --- |
| Transmit signal quality | EIRP based (if applicable), at peak direction |
| Unwanted emission | TRP based |
| REFSENS | EIS based, at peak direction |
| Maximum input power | EIS based, at peak direction |

- References

- R4-2505230	WF on A-IOT device requirements	CMCC, RAN4#114bis

- R4-2506841_Discussion on the RF requirement of AIoT BS, vivo

- RFID Handbook: Fundamentals and Applications in Contactless Smart Cards, Radio Frequency Identification and Near-Field Communication.

- ITU-R SM.328-11