3GPP TSG-RAN WG4 Meeting # 114bis 	R4-2506846

Malta, MT, 19th – 23rd May, 2025

Source: 	vivo

Title: 	Discussion on the OTA test method for device 1

Agenda Item:	7.26.4

Document for:	Approval

- Introduction

In this contribution, we provide our views on the OTA test method of device.

- Discussion

- Testing frequencies

In [1], we had the following agreements:

Agreement:

Only test low, middle and high frequencies within a band and average the performance metrics from the three frequencies

Check whether three frequencies are applicable for all requirements.

However, the details of the test parameter still absent, the following table is proposed as starting point.

Proposal 1: Take the following table as the starting point of test parameter

Table 1: Tx requirements measurement parameters for Ambient IoT device

| NR Band | D2R CBW (kHz) | R2D CBW (MHz) | SCS (kHz) | DL modulation | UL modulation | Range | UL Carrier wave centre [ARFCN] | UL Carrier wave centre (MHz) | UL Small frequency shift factor | DL R2D centre [ARFCN] | DL R2D Center (MHz) | DL R2D Configuration (FULL RB, LCRB @ RBstart) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| n8 | [16] | [0.2] | 15 | OOK | OOK/BPSK | Low | 177500 | 887.5 | 1 | 186500 | 932.5 | 1@0 |
|  |  |  |  |  |  | Mid | 179500 | 897.5 |  | 188500 | 942.5 |  |
|  |  |  |  |  |  | High | 181500 | 907.5 |  | 190500 | 952.5 |  |

Table 2: Rx requirements measurement parameters for Ambient IoT device

| NR Band | D2R CBW (MHz) | R2D CBW (MHz) | SCS (kHz) | DL modulation | UL modulation | Range | UL Carrier wave centre [ARFCN] | UL Carrier wave centre (MHz) | UL Small frequency shift factor | DL R2D centre [ARFCN] | DL R2D Center (MHz) | DL R2D Configuration (FULL RB, LCRB @ RBstart) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| n8 | [16] | [0.2] | 15 | OOK | OOK | Low | 177500 | 887.5 | 1 | 186500 | 932.5 | 1@0 |
|  |  |  |  |  |  | Mid | 179500 | 897.5 |  | 188500 | 942.5 |  |
|  |  |  |  |  |  | High | 181500 | 907.5 |  | 190500 | 952.5 |  |

- Testing antenna and CW antenna relationship

In [1], we agreed that:

Whole sphere scan is supported

The position of CW antenna is a set of fixed position(s) as baseline (depending on core requirements discussion).

- FFS on relative positions between reader antenna and CW

In our understanding, it is unnecessary to define such specific relative position only if the isolation between CW and test antenna is clearly specified. Such details can be left to implementation.

Proposal 2: No need to define the relative positions between measurement antenna and CW source.

- How to ensure the isolation between CW node and measurement antenna

In [1], it is suggested to discuss the power difference between D2R and CW leakage. The main impact of CW leakage is the CW will mix with D2R and make the D2R measurement higher that its real power. Assuming the D2R power is P dBm and CW leakage is P-X dBm, the percentage of D2R power can be calculated as:

η=10P1010P10 +10P-X10=11 +10-X10

For different X dB power difference, the results are listed below

| Power difference X dB | D2R power percentage η |
| --- | --- |
| 6 | 79.9% |
| 9 | 88.9% |
| 12 | 94.1% |
| 15 | 96.9% |

With 15 dB power difference, the contribution of CW in the whole measurement power is less than 5%, which could be a starting point of isolation.

Observation 1: with 15 dB power difference between CW leakage and D2R, the impact of CW leakage can be less than 5%.

In [2], our simulation results shows that the SNR@10% BLER is around -7 dB, 15dB power difference could also enough for device sensitivity test to avoid the CW leakage impact the device response.

Proposal 3: The isolation between CW and measurement antenna should ensure the CW leakage is at least 15dB lower than the D2R.

- Testing performance metric of reference sensitivity

During the sensitivity test, the device needs to feedback to the TE when receive the R2D correctly and the side condition of CW power need to be defined. For sensitivity test, the key purpose is to verify the performance of reception rather than the D2R transmission, so the CW only need to guarantee the device can transmit D2R with good quality and doesn’t impact the D2R reception, and use same CW power as Tx requirement verification can be a simple choice.

Proposal 4: Same CW condition for device Tx requirement verification is applied to sensitivity test.

- Testing procedure

As we stated in previous meeting, the 3D scan is quite time consuming especially for EIS, which may really burden the cost of device. For the REFSENS, this requirement is clearly defined at the single direction with best sensitivity. To alleviate the potential test burden, we suggest the REFSENS is tested at the EIRP max direction, which means the EIS peak search can be skipped.

Proposal 4: The REFSENS is tested at EIRP max direction and no need to perform the EIS peak search.

- Conclusion

Proposal 1: Take the following table as the starting point of test parameter

Table 1: Tx requirements measurement parameters for Ambient IoT device

| NR Band | D2R CBW (kHz) | R2D CBW (MHz) | SCS (kHz) | DL modulation | UL modulation | Range | UL Carrier wave centre [ARFCN] | UL Carrier wave centre (MHz) | UL Small frequency shift factor | DL R2D centre [ARFCN] | DL R2D Center (MHz) | DL R2D Configuration (FULL RB, LCRB @ RBstart) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| n8 | [16] | [0.2] | 15 | OOK | OOK/BPSK | Low | 177500 | 887.5 | 1 | 186500 | 932.5 | 1@0 |
|  |  |  |  |  |  | Mid | 179500 | 897.5 |  | 188500 | 942.5 |  |
|  |  |  |  |  |  | High | 181500 | 907.5 |  | 190500 | 952.5 |  |

Table 2: Rx requirements measurement parameters for Ambient IoT device

| NR Band | D2R CBW (MHz) | R2D CBW (MHz) | SCS (kHz) | DL modulation | UL modulation | Range | UL Carrier wave centre [ARFCN] | UL Carrier wave centre (MHz) | UL Small frequency shift factor | DL R2D centre [ARFCN] | DL R2D Center (MHz) | DL R2D Configuration (FULL RB, LCRB @ RBstart) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| n8 | [16] | [0.2] | 15 | OOK | OOK | Low | 177500 | 887.5 | 1 | 186500 | 932.5 | 1@0 |
|  |  |  |  |  |  | Mid | 179500 | 897.5 |  | 188500 | 942.5 |  |
|  |  |  |  |  |  | High | 181500 | 907.5 |  | 190500 | 952.5 |  |

Proposal 2: No need to define the relative positions between measurement antenna and CW source.

Observation 1: with 15 dB power difference between CW leakage and D2R, the impact of CW leakage can be less than 5%.

Proposal 3: The isolation between CW and measurement antenna should ensure the CW leakage is at least 15dB lower than the D2R.

Proposal 4: Same CW condition for device Tx requirement verification is applied to sensitivity test.

Proposal 4: The REFSENS is tested at EIRP max direction and no need to perform the EIS peak search.

- References

- R4-2505230	WF on A-IOT device requirements	CMCC

- R4-2506841_Discussion on the RF requirement of AIoT BS