3GPP TSG-RAN WG4 Meeting # 114bis 	R4-2506847

Malta, MT, 19th – 23rd May, 2025

Source: 	vivo

Title: 	Discussion on the FR2 OTA test method

Agenda Item:	7.15.2

Document for:	Approval

- Introduction

In the past meetings, the test method for sTxMP was almost finished and in this contribution, we provide our view on remaining issues.

- Discussion

Agreement:

The value for the uncertainty source of systematic error related to beam peak search is set as 0. The impact due to finite testing directions including limited choices for AoA1&AoA2 test directions can be handled in an additional Test Tolerance, TTFiniteRes. This additional TT should be determined in RAN4 and then communicated to RAN5 as recommended value to be added to the TT determined as a function of MU, i.e., TTTotal = x*MUTotal+ TTFiniteRes where x is determined in RAN5.

In the last meeting, we agreed that [1]:

As we agreed before, the AoA1 is same as beam peak search of single carrier or based on UE declaration, so the AoA1 should align with the beam peak of DUT well and the TTFiniteRes should be set same as TT of min peak EIRP

Proposal 1: The TTFiniteRes of AoA1(i.e., declared by UE or use same beam peak direction of legacy single carrier) is set to be the same as the TT of legacy min peak EIRP.

However, the AoA2 totally depend on the AoA1 and the separation between two AoA (i.e., 30°, 60°, 90°, 120°, 150°), and there are two possible cases lead to a large deviation from beam peak of AoA, as shown in figure I

Figure I illustration of deviation of beam peak

For these two cases, the maximum deviation from DUT beam peak could be 30°.

Observation 1：Due to the constrain of test system, the maximum deviation of the beam peak could be 30°.

To further evaluate the TTFiniteRes, we assume the deviation is uniformly distributed within [-30°,30°] and the antenna array size is 4x4 with total 25 beams. The simulation results are shown in figure II

Figure II Antenna gain degradation when the AoA is deviated from beam peak within [-30°,30°]

The maximum antenna degradation is 4.5 dB based on the simulation.

Proposal 2: The TTFiniteRes for AoA2 is set to TT of legacy min peak EIRP + 4.5 dB.

- Conclusion

In this contribution, we provide our views on MU and remaining issue in test method

Proposal 1: The TTFiniteRes of AoA1(i.e., declared by UE or use same beam peak direction of legacy single carrier) is set to be the same as the TT of legacy min peak EIRP.

Observation 1：Due to the constrain of test system, the maximum deviation of the beam peak could be 30°.

Proposal 2: The TTFiniteRes for AoA2 is set to TT of legacy min peak EIRP + 4.5 dB.

- References

- R4-2504727, Way Forward for [114bis][327] NR_FR2_OTA, Qualcomm Incorporated