3GPP TSG-RAN WG4 Meeting # 115											R4-2506848

Malta, MT, 19th – 23rd May, 2025

Source: 	vivo

Title: 	TP to TR 38.771 on FR2 sTxMP MU and test tolerance

Agenda Item:	7.13.1

Document for:	Approval

- Introduction

In this contribution, we update the TR based on the agreement in the last meeting.

- Text proposal

## <<Start of Change>>

Annex A:
Measurement uncertainty budget for STxMP UE RF testing methodology

<Editor’s note: the outcome of MU budget for STxMP UE RF testing methodology can be captured in this clause>

The uncertainty tables shall be presented with two stages:

-	Stage 1: the calibration of the absolute level of the DUT measurement results is performed by means of using a calibration antenna whose absolute gain is known at the frequencies of measurement

-	Stage 2: the actual measurement with the DUT as either the transmitter or receiver is performed.

MU for STxMP testing which is leveraged from legacy EIRP/TRP MU budget for PC1 and PC5 specified in Table B.3.2-6 of [38.903].

Compared to legacy uncertainty assessments for UE RF testing defined in [38.903], the value for the uncertainty source of systematic error related to beam peak search can be set as 0. The error due to finite testing directions can be handled in test tolerance. How to impact TT will be finalized in RAN5.

The preliminary MU budget for UE RF testing methodology is defined in Table A.1-1.

Table A.1-1: Uncertainty assessment for Minimum EIRP, EIRPmax and TRP measurement (f=23.45GHz, 32.125GHz, 40.8GHz, Quiet Zone size ≤ 30 cm) for PC1 and PC5 UEs and normal temperature condition

| UID | Uncertainty source | Uncertainty value | Distribution of the probability | Divisor | Standard uncertainty (σ) [dB] |
| --- | --- | --- | --- | --- | --- |
| Stage 2: DUT measurement |  |  |  |  |  |
| 1 | Positioning misalignment | 0.02 | Normal | 2.00 | [0.01] |
| 2 | Measure distance uncertainty | 0.00 | Rectangular | 1.73 | [0.00] |
| 3 | Quality of Quiet Zone (NOTE 1) | 0.6 | Actual | 1.00 | [0.6] |
| 4 | Mismatch | 1.30 | Actual | 1.00 | [1.30] |
| 5 | Standing wave between the DUT and measurement antenna | 0.00 | U-shaped | 1.41 | [0.00] |
| 6 | Uncertainty of the RF power measurement equipment (NOTE 3) | 2.16 | Normal | 2.00 | [1.08] |
| 7 | Phase curvature | 0.00 | U-shaped | 1.41 | [0.00] |
| 8 | Amplifier uncertainties | 2.10 | Normal | 2.00 | [1.05] |
| 9 | Random uncertainty | 0.50 | Normal | 2.00 | [0.25] |
| 10 | Influence of the XPD | 0.01 | U-shaped | 1.41 | [0.00] |
| 11 | Insertion Loss Variation | 0.00 | Rectangular | 1.73 | [0.00] |
| 12 | RF leakage (from measurement antenna to the receiver/transmitter) | 0.00 | Actual | 1.00 | [0.00] |
| 13 | Influence of TRP measurement grid (NOTE 4) | 0.25 | Actual | 1 | [0.25] |
| 14 | Influence of beam peak search grid (NOTE 5) | 0.00 | Actual | 1 | [0.00] |
| 15 | Multiple measurement antenna uncertainty (NOTE 9) | 0.15 | Actual | 1 | [0.15] |
| 16 | DUT repositioning | 0.00 (NOTE 4) 0.35 (NOTE 5) | Rectangular | 1.73 | [0.00] (NOTE 4) [0.20] (NOTE 5) |
| Stage 1: Calibration measurement |  |  |  |  |  |
| 17 | Mismatch | 0.00 | U-shaped | 1.41 | [0.00] |
| 18 | Amplifier Uncertainties | 0.00 | Normal | 2.00 | [0.00] |
| 19 | Misalignment of positioning System | 0.00 | Normal | 2.00 | [0.00] |
| 20 | Uncertainty of the Network Analyzer | 1.50 | Normal | 2.00 | [0.75] |
| 21 | Uncertainty of the absolute gain of the calibration antenna | 0.60 | Normal | 2.00 | [0.30] |
| 22 | Positioning and pointing misalignment between the reference antenna and the measurement antenna | 0.01 | Rectangular | 1.73 | [0.00] |
| 23 | Phase centre offset of calibration antenna | 0.00 | Rectangular | 1.73 | [0.00] |
| 24 | Quality of quiet zone for calibration process (NOTE 1) | 0.4 | Actual | 1.00 | [0.4] |
| 25 | Standing wave between reference calibration antenna and measurement antenna | 0.00 | U-shaped | 1.41 | [0.00] |
| 26 | Influence of the calibration antenna feed cable | 0.14 | Normal | 2.00 | [0.07] |
| 27 | Insertion Loss Variation | 0.00 | Rectangular | 1.73 | [0.00] |
|  | Systematic uncertainties (NOTE 6) |  |  |  | Value |
| 28 | Systematic error due to TRP calculation/quadrature (NOTE 4) |  |  |  | [0.00] |
| 29 | Influence of noise (23.45GHz <= f <= 32.125GHz) |  |  |  | [0.13] |
| 29 | Influence of noise (32.125GHz < f <= 40.8GHz) |  |  |  | [0.27] (NOTE 4) [0.20] (NOTE 5) |
| 30 | Systematic error related to beam peak search (NOTE 5) |  |  |  | [0.00] |
| Total measurement uncertainty |  |  |  |  | Value |
| EIRP Expanded uncertainty (23.45GHz <= f <= 32.125GHz) (1.96σ - confidence interval of 95 %) [dB] |  |  |  |  | [4.63] |
| EIRP Expanded uncertainty (32.125GHz < f <= 40.8GHz) (1.96σ - confidence interval of 95 %) [dB] |  |  |  |  | [4.70] |
| TRP Expanded uncertainty (23.45GHz <= f <= 32.125GHz) (1.96σ - confidence interval of 95 %) [dB] |  |  |  |  | [4.64] |
| TRP Expanded uncertainty (32.125GHz < f <= 40.8GHz) (1.96σ - confidence interval of 95 %) [dB] |  |  |  |  | [4.78] |
| NOTE 1: The quality of quiet zone is the same for EIRP and TRP. Value based on procedure defined in clause D.2 of TR 38.810 for Quiet Zone size less or equal to 30 cm. NOTE 2: The analysis was done only for the case of operating at max output power, in-band, non-CA and is valid for SISO and MIMO. NOTE 3: The assessment assumes maximum DUT output power. NOTE 4: This contributor shall only be considered for TRP measurements. NOTE 5: This contributor shall only be considered for EIRP measurements. NOTE 6: In order to obtain the total measurement uncertainty, systematic uncertainties have to be added to the expanded root sum square of the standard deviations of the Stage 1 and Stage 2 contributors. NOTE 7: Void. NOTE 8: Void NOTE 9: Applies to the system which has a structure of mechanical feed antenna positioning. declaration. |  |  |  |  |  |

The impact due to finite testing directions including limited choices for AoA1&AoA2 test directions can be handled in an additional Test Tolerance, TTFiniteRes.

- Min peak EIRP

For AoA1 (declared by UE or align with the beam peak of legacy single CC operation), the TTFiniteRes is set to 0 and the test tolerance defined in TS 38.521-2 for legacy min peak EIRP is reused.

For AoA2, since this AoA is fully decided by AoA1 and predefined candidate separation, the deviation from beam peak could happen and two possible cases are shown in Figure A.1-1

Figure A.1-1 Cases of deviation of beam peak

The worst case of deviation can be 30°, and Figure A.1-2 show the simulation results of antenna gain degradation. The deviation is uniformly distributed within [-30°,30°] and the antenna array size is 4x4 with total 25 beams.

Figure A.1-2 Degradation when the AoA is deviated from beam peak within [-30°,30°]

It is observed that up to 4.5 dB degradation could be caused by the beam peak mismatch, so on top of the test tolerance of min peak EIRP in TS 38.521-2, additional TTFiniteRes = 4.5 dB is applied.

## <<End of Change>>