3GPP TSG-RAN WG4 Meeting # 115	R4-2506858

Malta, MT, 19th – 23rd May, 2025

Source: 	vivo, SmarterMicro

Title: 	Iot NTN HPUE Tx Requirements and further evaluation

Agenda Item:	7.10.3.3.1

Document for:	Approval

Introduction

From RAN4#112 to #114 agreements, summary and evaluation results were included in [1] to [14].

In RAN4#114bis, an evaluation of NR PA for NB-Iot Iot-NTN was submitted in [15], and WF and summary is provided in [16][17]. Major breakthrough were made.

In this paper, more evaluation of NB-Iot based Iot-NTN based on NR PA has been submitted for evaluation of MPR requirements.

Further Evaluation of NR PA for NB-Iot Iot-NTN

PA selection and Features

In RAN4#114, a comprehensive evaluation of NR PA ACLR and SEM results were provided. In order to further supplement the behavior and also MPR requirements, more evaluations are provided for NB-Iot Iot-NTN based on NR PA.

In this paper, the same NR PA module was used from previous paper. It should be noted that by adjusting different Vcc, the PA can be regarded as PC3/PC2.

The block diagram was re-submitted for information. And still the middle band PA is used:

Figure 1. Block diagram of a Multi-mode PA(NR/LTE/3G) for verification

The scope of new evaluations was described in next section test conditions.

Test conditions

Although there were quite some evaluation results provided in previous meeting as in [15]. There were some issues still identified:

- Insufficient resource allocation types

- The allocation in last meeting were focused on edge single tone and full tone allocation

- The center single tone which is the regarded as possible MPR = 0dB point were not verified

- The 3/6 RE allocation were not verified

- Insufficient SCS verified

- Only 15KHz SCS was verified, and 3.75kHz SCS was not verified

- Especially for single Tone case, there was ambiguity in whether 3.75KHz or 15KHz should be used. The current RAN5 verification has verified both SCS.

- In sufficient Modulation order verified

- Some ambiguities on whether QPSK/BPSK be used for single tone case as described in [15].

- Only QPSK was verified, while BPSK was not verified, though 36.521-1 has this for single tone test

- RMS detector vs Peak detector for SEM

- Last meeting’s results were based on peak detector for SEM. However, it was clarified in 36.521-1 that in SEM “using a rms detector”.

- Signal degradation not considered

- Some imperfections were not considered included such as (Carrier leakage, IQ image, CIM3)

In this meeting, more evaluation results were conducted, considered the issues in last meeting. The only issue that is not considered is the signal degradation. This may need some further relaxations for the final results.

The test conditions and some corresponding notes are as following. It should noted that in order to control the testing numbers, this time only frequency 1643MHz were selected. The 15KHz and QPSK is still the main stream, while BPSK and 3.75KHz SCS were mostly for supplemental purposes.

Table 1. Test conditions (with update)

| Item | Test condition | Note |
| --- | --- | --- |
| Test waveform | NB-Iot based IoT-NTN Center 1RE; (15KHz BPSK/QPSK, Inner 3.75KHz BPSK) Edge 1RE; (3.75KHz - BPSK/QPSK; 15KHz - BPSK/QPSK;) Full RE (15KHz- QPSK); 3/6 RE (15KHz- QPSK); | Some are referenced from 36.521-4 (Not all) |
| Input port | RFIN_MB | Using Middle Band PA |
| Output port | MB1 | Using Middle Band PA |
| Test Vcc | Vcc=3.4V for normal Pout (Typical for PC3) Vcc=4.0V for higher Pout (Close to PC2 need) | All these Vcc are within the normal working range for MB PA with guaranteed reliability. |
| Test Pout | The highest achievable power (no larger than 34dBm) | An IL of 3dB can be assumed in addition to 31dBm nominal MOP |
| Test Frequency | 1643MHz | Typical frequency for band 255 This is a slightly less optimized frequency for this PA compared to 1995MHz; |
| Test item | SEM | SEM reusing PC3 requirements. (Bottleneck according to earlier study) |
| Power measurement method | RMS detector | Aligned with 36.521-1 |
| Signal Imperfections | Carrier leakage, IQ image, CIM3 not considered in the signal source | This may lead to slightly better performance than expected. |
| insertion Loss assumption | 3 dB | This is actually not really covered in the test, but |
| Note: Spurious emission is usually controlled by means of filter in front end and not a bottleneck for the HPUE, thus not considered here. EVM is also not considered since it is also not a bottleneck and large margin would apply as demonstrated in other similar tests. ACLR is also not considered this time since already verification that |  |  |

Results

SEM Measurements

A summary of the evaluation of SEM is as following.

In order to simplify the illustration, only the highest achievable power which satisfy the SEM were listed. This would means that this power can satisfy SEM, and any higher power is either not achievable, or not able to satisfy SEM, or not be tested (highest tried power is 34dBm considering PC1 nominal MOP and maximum 3dB IL).

The detailed measurement results can reference the Annex. In order to simplify the results

Table 2. Summary of evaluation of SEM (Updated)

In order to make comparison with last meeting’s results, the summary from last meeting was also copied here for reference.

Table 3. Previous Summary of evaluation of SEM in RAN4#114bis [15]

Analysis

From the test results, some conclusions can be derived.

Nominal Output Power

From the updated evaluation, it can be seen that a maximum of 34dBm MOP can be reached for center single tone case for 15KHz SCS, and also non edge case for 3.75KHz(waveform No. 9,10,11), this is aligned with previous analysis based on the margin for edge single tone cases. This 34dBm means that even under a more strict 3dB front end IL assumption, the 31dBm PC1 nominal MOP can still be achieved.

Observation 1-1: An output power of 34dBm is achieved for NR PA evaluation. Under a 3dB front end IL assumption, a 31dBm nominal MOP can be achieved.

Impact of Power measurement method

For similar configurations (such as edge single tone and full tone allocation), this time the evaluation has a roughly 1~2dB improvement. The main difference is changing the power measurement from peak detector to RMS detector. Since RMS is the current defined method for RAN5 in 36.521-1, it is believed that the evaluation in this time could be more typical.

Observation 1-2: RMS detector seems have better performance than peak detector.

Performance difference between BPSK & QPSK

Based on the performance comparison for edge single tone (1,2 vs 3,4; 5,6 vs 7,8) and center single tone (9 vs 11), it can be seen for all the cases, BPSK show the same performance with same max power in all power configuration. So based on this, we may only define QPSK requirements for simplicity, and the legacy problem of ambiguities of mixing BPSK and QPSK verification for PC3 edge single tone case may not be a problem.

Observation 1-3: BPSK and QPSK show same performance for single tone cases.

Proposal 1: Only define MPR requirements for QPSK is sufficient, and current ambiguity of edge single tone modulation need not be refined.

Performance difference between 3.75KHz & 15KHz

This is the first comparison between 3.75KHz and 15KHz SCS. The 15KHz is still baseline, and 3.75KHz SCS is concentrated for single tone case, which is more typical.

The results show some considerable difference for edge single tone case for 3.75KHz and 15KHz, which is a bit unexpected. E.g. by comparing waveform No 3 vs. 7 (edge single tone, QPSK, 3.75 vs 15), it was found that 3.75KHz option need 2dB more power reduction compared to 15KHz. This 2dB difference also exist for BPSK case.

In the detailed results, it can be seen that the edge single tone case for 3.75KHz pose a more harsh condition for the first part of SEM. This may because of the much higher power spectrum density, and in sufficient guard band. Some figures were copied from the Annex.

Figure 1: Edge single tone 3.75KHz vs. 15KHz (No.3 vs. No.7 waveform), Vcc 4.0V

By comparison, for the case that when single tone is not in the edge, the 3.75KHz have a similar performance with 15KHz (as in waveform No. 10). It is also expected that for other allocation cases, the difference will not be that prominent.

Observation 1-4: For edge single tone case, 3.75KHz SCS need considerably more power reduction (2dB in evaluation) to satisfy SEM.

In this condition, it was proposed that the MPR requirements should clearly indicate SCS. 15KHz SCS is still proposed as baseline. If 3.75KHz SCS is used, then separate requirements are needed for can In the simulation, it was found that

Proposal 2a: The SCS should be explicitly defined in MPR tests and 15KHz SCS can continue to be baseline.

Proposal 2b: If 3.75KHz is also considered, separate edge single tone requirements with relaxation compared to 15KHz is needed.

MPR evaluation consideration

It should be noted that some imperfections such as Carrier leakage, IQ image were not considered in the waveform generation for the evaluation. With those imperfections were considered, the actual MPR needed might be larger than evaluation here. In addition, some extra margin also needed, to consider that there might be some other implementations.

Proposal 3: The MPR evaluation can be a reference for PC1 MPR requirements. Some margin is still needed when considering MPR requirements.

HPUE Impact for Tx Requirements

General

The current status is for requirements applicability is as following which was referenced from last meeting’s WF [14].

Table 4: Current status for requirements applicability for Iot-NTN

|  | Power Class | Antenna Numbers | Handheld and/or non-handheld |
| --- | --- | --- | --- |
| NB-Iot based | PC1 | 1Tx | Both* *Note: Tones limitation FFS |
|  | PC2 | 1Tx | Both |
| eMTC based with half duplex | PC1 | 1Tx | Non-handheld |
|  | PC2 | 1Tx | Both |
| eMTC based with full duplex | PC1 | 1Tx | Non-handheld |
|  | PC2 | 1Tx | Both |

The only remaining issue coming from NB-Iot based Iot-NTN for handheld, there is still an FFS issue:

Issue 1-2-1: For NB-Iot based Iot-NTN, PC1 requirements can be applied to:

Agreement:

- For NB-Iot based Iot-NTN, PC1 requirements can be applied to both handheld and non-handheld.

  - FFS whether to limit to the use case with single tone and a limited number of tones

This issue is coming from the previous evaluation that for full allocation, there would be a 4dB power back off value. This is fairly large value in the context of NB-Iot, since only 2dB were considered for PC3 in the legacy case.

However, it is in fact unusual and even a bit strange to restrict allocation, simply because larger MPR. This is an unnecessary restriction that would be negative to the system flexibility. In addition, even the 4dB value is still in a reasonable range. There exists considerable larger MPR in the spec currently.

Observation 2-1: It is unusual to do resource allocation restriction simply because of larger MPR.

Furthermore, based on the new results provided in this meeting, the performance for larger allocation and full allocation are smaller compared to last meeting, making it even more meaning less to do any restriction.

Observation 2-2: Based on new evaluation, the power reduction needed for full allocation is smaller than previous evaluation based on updated method and assumption.

Based on these observations, there is the following proposal:

Proposal 4: Not to limit the allocation of NB-Iot based Iot-NTN to single tone or a limited number of tones.

In addition, there was the following issue in the WF of last meeting:

Other (For information):

- Interesting companies can provide inputs on UE RF components capabilities for FDD and HD-FDD especially for handheld HPUE, including NR-NTN and Iot-NTN in next meeting.

Based on our understanding, this issue is try to clarify the feasibility for various components under HPUE condition. In fact, our earlier papers such as [5] already have such analysis, mainly for duplexer etc. For PC2, it seems no major issues exist. For PC1, there exists some issues. E.g. for full duplex operation, the current SAW/FBAR filter used in smartphones may not well serve power class higher than PC2. Since only NB-Iot based Iot-NTN have PC1 requirements that can apply to handheld UE, eMTC ned not be considered.

Observation 3-1: PC2 do not have feasibility issues in UE RF components capabilities.

Observation 3-2: Only NB-Iot based Iot-NTN have PC1 applicability for handheld.

For half duplex only cases, duplexer may not be used and be limiting factor. The PA poses a main contributor for the implementation challenges. For NB-Iot based Iot-NTN, according to our previous evaluation, no matter GSM PA based / NR PA based / Proprietary PA bases, there were still sufficient implementation margins in Vcc configuration etc.

For GSM PA which previously only use 1/8 of the time, the increased operation time may be a challenge, but the output power can actually be used is also somewhat lower compared to GSM, and the half time shutting down coming from the half-duplex operation also eased this problem. For NR PA which designed for normal FDD/TDD, this half time shutting down scheme also provides another protection for the thermal effects. It is admitted that the overall design of UE may still be impacted and more thermal dissipation need to be considered, but the components function and capabilities need not be doubted.

Proposal 5: No need to doubt the current components capabilities for Handheld HPUE already defined for the requirements applicability.

NB-Iot Based

MPR for PC2

In last meeting WF, there were calibration point also defined. However, there are still many offline discussion and uncertainties. One of the most basic question is what is the reference component should be used for PC2? If it is a PC2 NR PA or GSM PA, it is obvious that based on existing evaluation, no MPR is really needed.

Using a NR PC3 PA might be another choice, and both evaluation and simulation may be considered. However, considering for simplicity, one choice is simply to use PC3 requirements for MPR for PC2, this could be a balance between simplicity and performance and

Proposal 6: Reuse PC3 MPR requirements for PC2 as starting point.

MPR for PC1

As discussed before, the MPR can using the evaluation as an reference, with certain implementation margin.

It has already been agreed to consider center single tone case as MPR = 0. In addition, there is also proposal that only consider QPSK since BPSK has similar performance. The 3.75KHz requirements can be skipped if more simplicity is needed. If not, 3.75KHz can concentrate to edge single tone case, and some relaxations are needed.

Proposal 7: Based on previous evaluation and some margin, the following MPR can be considered:

Table 6.2B.2-X: Maximum Power Reduction (MPR) for UE category NB1 and NB2 Power Class 1

| Modulation | QPSK |  |  |  |
| --- | --- | --- | --- | --- |
| SCS | 15KHz |  |  |  |
| Tone positions for 1 Tone allocation | [0-2] | [3-8] |  | [8-11] |
| MPR | ≤ [1.5] dB | 0 dB |  | ≤ [1.5] dB |
| Tone positions for 3 Tones allocation | 0-2 | 3-5 and 6-8 |  | 9-11 |
| MPR | ≤ [2] dB | [1.5] dB |  | ≤ [2] dB |
| Tone positions for 6 Tones allocation | 0-5 and 6-11 |  |  |  |
| MPR | ≤ [2] dB |  | ≤ [2] dB |  |
| Tone positions for 12 Tones allocation | 0-11 |  |  |  |
| MPR | ≤ [4] dB |  |  |  |

P-MPR

In previous meetings, tentative P-MPR proposal was raised for NB-Iot, but no conclusions made yet.

Since we already have the requirements on the table for some time, in this meeting, we would directly discuss this issue with draft CR.

Proposal 8: Further discuss the P-MPR proposal for NB-Iot based Iot-NTN with draft CR.

eMTC-Based

ACLR

For eMTC based Iot-NTN, PC1 ACLR still not confirm yet.

Legacy LTE PC1 ACLR is requirements was 37dB. However, from co-existence point of view, a 30/31dBc value looks sufficient. In NR NTN, there was a lot of discussions on how to set up this requirement, and a tentative [34]dBm was raised, as a compromise between various factors.

For eMTC based Iot-NTN, situations were simpler, since only 1Tx is considered, and it is also not that likely to reuse the current components in smartphones.

However, similar compromise may also be considered. In addition, NR-NTN and eMTC based NTN actually share many similarities, and alignment of key parameters may raise the possibility to share components between those two RATs.

Proposal 9: Using [34]dBm as starting point for PC1 eMTC-based Iot-NTN, which is aligned with PC1 NR-NTN.

MPR

In RAN4#114bis, the following agreements are reached:

Issue 1-1-6: MPR assumptions for eMTC based Iot-NTN for PC2

Agreement:

- Reuse eMTC TN PC2 MPR requirements (same to PC3)

Issue 1-1-7: MPR assumptions for eMTC based Iot-NTN for PC1

Agreement:

- Use TN PC2 requirements for Category M1 as starting point

Although it seems strange to have a starting point for MPR before ACLR is settled, this is also based on the condition that the current components are not likely to be directly reused for PC1, and PC1 is restricted to non-handheld only. In addition, there were not sufficient attention for eMTC from the industry is also another reason. The previous starting point can be directly used.

Based on this situation, the following proposal is made:

Proposal 10: For PC1 eMTC based-Iot NTN, apply TN PC2 MPR requirements for Category M1.

SAR and Duty cycle

A complete background and detailed analysis of SAR was introduced in [12]. A comprehensive discussion was also taken place during the meeting.

However, the current available scheme both have problems and seems difficult to converge. Here we still think the proposal we made last meeting may be one viable way and re-submitted them for further discussion. Those key points have been also re-submitted in last meeting in [15].

We found that the situation does not change much in last meeting, thus

Observation 4-1 (Re-submit): The two basic ways to control SAR is by either reduce the power, or reduce transmission time.

Observation 4-2 (Re-submit): UE voluntarily dropping some uplink transmissions based on self-assessment can reduce SAR.

Proposal 11 (Re-submit): Consider to introduce a scheme that UE voluntarily dropping uplink transmissions by self-assessment to control the SAR. This may be simple enough, can be controlled by network and still useful.

Conclusion

Further Evaluation of NR PA for NB-Iot Iot NTN

Observation 1-1: An output power of 34dBm is achieved for NR PA evaluation. Under a 3dB front end IL assumption, a 31dBm nominal MOP can be achieved.

Observation 1-2: RMS detector seems have better performance than peak detector.

Observation 1-3: BPSK and QPSK show same performance for single tone cases.

Proposal 1: Only define MPR requirements for QPSK is sufficient, and current ambiguity of edge single tone modulation need not be refined.

Observation 1-4: For edge single tone case, 3.75KHz SCS need considerably more power reduction (2dB in evaluation) to satisfy SEM.

Proposal 2a: The SCS should be explicitly defined in MPR tests and 15KHz SCS can continue to be baseline.

Proposal 2b: If 3.75KHz is also considered, separate edge single tone requirements with relaxation compared to 15KHz is needed.

Proposal 3: The MPR evaluation can be a reference for PC1 MPR requirements. Some margin is still needed when considering MPR requirements.

HPUE Impact for Tx Requirements

General:

Observation 2-1: It is unusual to do resource allocation restriction simply because of larger MPR.

Observation 2-2: Based on new evaluation, the power reduction needed for full allocation is smaller than previous evaluation based on updated method and assumption.

Proposal 4: Not to limit the allocation of NB-Iot based Iot-NTN to single tone or a limited number of tones.

Observation 3-1: PC2 do not have feasibility issues in UE RF components capabilities.

Observation 3-2: Only NB-Iot based Iot-NTN have PC1 applicability for handheld.

Proposal 5: No need to doubt the current components capabilities for Handheld HPUE already defined for the requirements applicability.

NB-Iot based:

Proposal 6: Reuse PC3 MPR requirements for PC2 as starting point.

Proposal 7: Based on previous evaluation and some margin, the following MPR can be considered:

Table 6.2B.2-X: Maximum Power Reduction (MPR) for UE category NB1 and NB2 Power Class 1

| Modulation | QPSK |  |  |  |
| --- | --- | --- | --- | --- |
| SCS | 15KHz |  |  |  |
| Tone positions for 1 Tone allocation | [0-2] | [3-8] |  | [8-11] |
| MPR | ≤ [1.5] dB | 0 dB |  | ≤ [1.5] dB |
| Tone positions for 3 Tones allocation | 0-2 | 3-5 and 6-8 |  | 9-11 |
| MPR | ≤ [2] dB | [1.5] dB |  | ≤ [2] dB |
| Tone positions for 6 Tones allocation | 0-5 and 6-11 |  |  |  |
| MPR | ≤ [2] dB |  | ≤ [2] dB |  |
| Tone positions for 12 Tones allocation | 0-11 |  |  |  |
| MPR | ≤ [4] dB |  |  |  |

Proposal 8: Further discuss the P-MPR proposal for NB-Iot based Iot-NTN with draft CR.

eMTC based:

Proposal 9: Using [34]dBm as starting point for PC1 eMTC-based Iot-NTN, which is aligned with PC1 NR-NTN.

Proposal 10: For PC1 eMTC based-Iot NTN, apply TN PC2 MPR requirements for Category M1.

SAR and duty cycle

Observation 4-1 (Re-submit): The two basic ways to control SAR is by either reduce the power, or reduce transmission time.

Observation 4-2 (Re-submit): UE voluntarily dropping some uplink transmissions based on self-assessment can reduce SAR.

Proposal 11 (Re-submit): Consider to introduce a scheme that UE voluntarily dropping uplink transmissions by self-assessment to control the SAR. This may be simple enough, can be controlled by network and still useful.

- References

[1] RP-240857, New WID: Enhanced requirements and test methodology for NR and IoT NTN, RAN#103

[2] R4-2412829, Topic summary for [112][127] NR_IoT_NTN_HPUE , Samsung, RAN4#112

[3] R4-2414452, WF on UE RF requirements for NTN HPUE, Samsung, RAN4#112

[4] R4-2412099, Feasibility study and Initial analysis for NTN HPUE RF Tx Requirements, vivo, RAN4#112

[5] R4-2415790, Feasibility study and analysis for Iot NTN HPUE Tx Requirements, vivo, RAN4#112bis

[6] R4-2415229, Topic summary for [112bis][125] NR_IoT_NTN_HPUE_part2,vivo, RAN4#112bis

[7] R4-2417205, WF on HPUE for IoT NTN, vivo, RAN4#112bis

[8] R4-2418202, Iot NTN HPUE Tx Requirements and feasibility analysis, vivo, RAN4#113

[9] R4-2418149, Topic summary for [113][127] NR_IoT_NTN_HPUE_part2, Moderator (vivo), RAN4#113

[10] R4-2420404, WF on IoT NTN HPUE, vivo, RAN4#113

[11] R4- 2500766, CR on Remove GSM_ACLR for NB-Iot based Iot-NTN, vivo, Huawei, MediaTek, RAN4#114

[12] R4-2500764, Iot NTN HPUE Tx Requirements and feasibility analysis, vivo, RAN4#114

[13] R4-2500681, Topic summary for [114][129] NR_IoT_NTN_HPUE_part2, Moderator(Vivo), RAN4#114

[14] R4-2503006, WF on HPUE UE requirement for IoT-NTN, vivo, RAN4#114

[15] R4-2503842, Iot NTN HPUE Tx Requirements and feasibility analysis, vivo, SmarterMicro, RAN4#114bis

[16] R4-2505114, WF on HPUE RF requirements for IoT-NTN, vivo, RAN4#114bis

[17] R4-2504682, Topic summary for [114bis][124] NR_IoT_NTN_HPUE_part2, Moderator(Vivo), RAN4#114bis

- Annex. SEM results for NR PA (Updated)

- Summary and Index

- 1. 3.75KHZ_1@0_PI2_BPSK

- 4.0V (31dBm Pass, 32dBm Fail)

- 3.4V (31dBm Pass, 32dBm Fail)

- 2. 3.75KHZ_1@47_PI2_BPSK

- 4.0V (31dBm Pass, 32dBm Fail)

- 3.4V (31dBm Pass, 32dBm N/A)

- 3. 3.75KHZ_1@0_PI4_QPSK

- 4.0V (31dBm Pass, 32dBm Fail)

- 3.4V (31dBm Pass, 32dBm N/A)

- 4. 3.75KHZ_1@47_PI4_QPSK

- 4.0V (31dBm Pass, 32dBm Fail)

- 3.4V (31dBm Pass, 32dBm N/A)

- 5. 15KHZ_1@0_PI2_BPSK

- 4.0V (33dBm Pass, 34dBm N/A)

- 3.4V (32dBm Pass, 33dBm Fail)

- 6. 15KHZ_1@11_PI2_BPSK

- 4.0V (33dBm Pass, 34dBm N/A)

- 3.4V (32dBm Pass, 33dBm Fail)

- 7. 15KHZ_1@0_PI4_QPSK

- 4.0V (33dBm Pass, 34dBm N/A)

- 3.4V (32dBm Pass, 33dBm N/A)

- 8. 15KHZ_1@11_PI4_QPSK

- 4.0V (33dBm Pass, 34dBm N/A)

- 3.4V (32dBm Pass, 33dBm N/A)

- 9. 15KHZ_1@5_PI2_BPSK

- 4.0V (34dBm Pass)

- 3.4V (33dBm Pass, 34dBm N/A)

- 10. 3.75KHZ_1@5_PI2_BPSK

- 4.0V (34dBm Pass)

- 3.4V (33dBm Pass, 34dBm N/A)

- 11. 15KHZ_1@5_PI4_QPSK

- 4.0V (34dBm Pass)

- 3.4V (33dBm Pass, 34dBm N/A)

- 12. 15KHZ_12@0_QPSK

- 4.0V (31dBm Pass, 32dBm Fail)

- 3.4V (30dBm Pass, 31dBm Fail)

- 13. 15KHZ_3@0_QPSK

- 4.0V (33dBm Pass, 34dBm N/A)

- 3.4V (32dBm Pass, 33dBm N/A)

- 14. 15KHZ_3@3_QPSK

- 4.0V (33dBm Pass, 34dBm N/A)

- 3.4V (32dBm Pass, 33dBm N/A)

- 15. 15KHZ_6@0_QPSK

- 4.0V (33dBm Pass, 34dBm N/A)

- 3.4V (32dBm Pass, 34dBm N/A)