3GPP TSG-RAN WG4 Meeting # 115	R4-2506862

Malta, MT, 19th – 23rd May, 2025

Source: 	vivo

Title: 	Further discussion of UE RF impact for NR MIMO phase 5

Agenda Item:	7.24.3

Document for:	Approval

Introduction

In RAN#102, a new WID [1] was approved on NR MIMO Phase 5. There were already three RAN1 meetings and significant progress have been made, and this is the first meeting for RAN4 to discuss this.

In RAN4#112, an initial analysis and study [2] were made for the WI, and a WF [3] has been agreed. In RAN#105, the NR MIMO phase 5 WID has been revised to [4] to add some new scope.

In RAN4#112bis, the 3Tx issues and mTRP issues were discussed and document summary is in [6], a WF is agreed in [7] and an LS was sent out in [8].

In RAN4#113, further discussion was made. A discussion paper was submitted in [10]. The overall summary was in [11] and an WF was reached in [12]. In addition, an reply LS was received after the meeting in [13], in which the answers from RAN1 were included.

In RAN4#114, discussion continues. However, no major progress was made. The main difficulty is on the TxD support and whether FR2 CR is needed or not. A summary is in [15] and WF in [16].

In RAN4#114bis. Considerable progress was made in several controversial topics. It seems that no major issues left. However, in this paper, a few remaining possible remaining issues were raised.

Discussion

3Tx RF Impact

PC1.5

In last meeting, the following agreement is reached:

<Conclusion>: PC1.5 for 3Tx MIMO

- There is no consensus for PC1.5 3Tx in RAN4 in Rel-19.

This is a agreement after long discussion. It seems that no more progress can be reached in the remaining time. And it is assumed that no more discussion would taken place on this issue in this release.

Observation 1: There would be no PC1.5 3Tx in RAN4 in Rel-19 since there was no consensus.

TxD

Although there is no dedicated new guidance on this issue in last meeting’s WF, it is actually closely related to PC1.5. The basic reason of no consensus for PC1.5 is no consensus on TxD.

So based on current situation, it seems that no more discussion would taken place on this issue.

Observation 2: It looks like no more discussion would take place for TxD.

Remaining requirements:

It was found that the Pcmax,c tolerance would also be impacted by 3Tx, but not considered before for 3Tx.

Currently the following tolerances were defined for UL-MIMO 2Tx and 4Tx respectively from 38.101-1.

Table 6.2D.4-1: PCMAX,c tolerance in closed-loop spatial multiplexing scheme

| PCMAX,c (dBm) | Tolerance TLOW(PCMAX_L,c) (dB) | Tolerance THIGH(PCMAX_H,c) (dB) |
| --- | --- | --- |
| 23 ≤ PCMAX,c ≤ 29 | 3.0 | 2.0 |
| 22 ≤ PCMAX,c < 23 | 5.0 | 2.0 |
| 21 ≤ PCMAX,c < 22 | 5.0 | 3.0 |
| 20 ≤ PCMAX,c < 21 | 5.0 | 4.0 |
| 16 ≤ PCMAX,c < 20 | 5.0 |  |
| 11 ≤ PCMAX,c < 16 | 6.0 |  |
| -40 ≤ PCMAX,c < 11 | 7.0 |  |

Table 6.2D.4-2: PCMAX,c tolerance in closed-loop spatial multiplexing scheme for 4Tx

| PCMAX,c (dBm) | Tolerance TLOW(PCMAX_L,c) (dB) | Tolerance THIGH(PCMAX_H,c) (dB) |
| --- | --- | --- |
| 24.5 ≤ PCMAX,c ≤ 29 | 3.0 | 2.0 |
| 23.5 ≤ PCMAX,c < 24.5 | 5.0 | 2.0 |
| 22.5 ≤ PCMAX,c < 23.5 | 5.0 | 3.0 |
| 21.5 ≤ PCMAX,c < 22.5 | 5.0 | 4.0 |
| 18 ≤ PCMAX,c < 21.5 | 5.0 |  |
| 13 ≤ PCMAX,c < 18 | 6.0 |  |
| -40 ≤ PCMAX,c < 13 | 7.0 |  |

For 3Tx, it is proposed to follow similar general guidelines, the following proposal is provided:

Proposal: Discuss the Pcmax,c tolerance for 3Tx, and the following table can be a starting point:

Table 6.2D.4-1a: PCMAX,c tolerance in closed-loop spatial multiplexing scheme for 3Tx

| PCMAX,c (dBm) | Tolerance TLOW(PCMAX_L,c) (dB) | Tolerance THIGH(PCMAX_H,c) (dB) |
| --- | --- | --- |
| 24 ≤ PCMAX,c ≤ 29 | 3.0 | 2.0 |
| 23 ≤ PCMAX,c < 24 | 5.0 | 2.0 |
| 22 ≤ PCMAX,c < 23 | 5.0 | 3.0 |
| 21 ≤ PCMAX,c < 22 | 5.0 | 4.0 |
| 17 ≤ PCMAX,c < 21 | 5.0 |  |
| 12 ≤ PCMAX,c < 17 | 6.0 |  |
| -40 ≤ PCMAX,c < 12 | 7.0 |  |

A draft CR for 38.101-1 is also provided in [21] based on the current previous agreements and the proposal here.

Conclusion

In this paper, more study on UE RF impact of MIMO phase 5 is provided:

3Tx RF Impact

Observation 1: There would be no PC1.5 3Tx in RAN4 in Rel-19 since there was no consensus.

Observation 2: It looks like no more discussion would take place for TxD.

Remaining requirements

Proposal: Discuss the Pcmax,c tolerance for 3Tx, and the following table can be a starting point:

Table 6.2D.4-1a: PCMAX,c tolerance in closed-loop spatial multiplexing scheme for 3Tx

| PCMAX,c (dBm) | Tolerance TLOW(PCMAX_L,c) (dB) | Tolerance THIGH(PCMAX_H,c) (dB) |
| --- | --- | --- |
| 24 ≤ PCMAX,c ≤ 29 | 3.0 | 2.0 |
| 23 ≤ PCMAX,c < 24 | 5.0 | 2.0 |
| 22 ≤ PCMAX,c < 23 | 5.0 | 3.0 |
| 21 ≤ PCMAX,c < 22 | 5.0 | 4.0 |
| 17 ≤ PCMAX,c < 21 | 5.0 |  |
| 12 ≤ PCMAX,c < 17 | 6.0 |  |
| -40 ≤ PCMAX,c < 12 | 7.0 |  |

A draft CR for 38.101-1 is also provided in [21]

- References

[1] RP-234007, New WID: NR MIMO Phase 5, Samsung (Moderator), RAN#102

[2] R4-2412095, Discussion of UE RF impact for NR MIMO phase 5, vivo, RAN4#112

[3] R4-2414310, WF on UE RF requirements for Rel-19 MIMO enhancement, Samsung, RAN4#112

[4] RP-242394, WID revision: NR MIMO Phase 5, RAN#105

[5] R4-2413224, On asymmetric connection scenarios for NR MIMO Phase 5, Qualcomm, RAN4#112

[6] R4-2415232, Topic summary for [112bis][128] NR_MIMO_Ph5_UE, Samsung, RAN4#112bis

[7]R4-2417116, WF on UE RF requirements for MIMO enhancement, Samsung, RAN4#112bis

[8] R4-2417117, LS on UE RF issues for Rel-19 MIMO enhancement, Huawei, RAN4#112bis

[9] R4-2415792, Further discussion of UE RF impact for NR MIMO phase 5, vivo, RAN4#112bis

[10] R4-2418204, Further discussion of UE RF impact for NR MIMO phase 5, vivo, RAN4#113

[11] R4-2418153, Topic summary for [113][131] NR_MIMO_Ph5_UE, Moderator (Samsung), RAN4#113

[12] R4-2420400, WF on UE RF requirements for MIMO enhancement, Samsung, RAN4#113

[13] R4-2500005, Reply to RAN4 LS on UE RF issues for Rel-19 MIMO Enhancement, RAN1#119

[14] R4-2417632, dCR on RF requirements for a UE with different TCI states for UL and DL, Qualcomm, RAN4#113

[15] R4-2500685, Topic summary for [114][133] NR_MIMO_Ph5_UE, Moderator(Samsung), RAN4#114

[16] R4-2503037, WF on UE RF requirements for MIMO enhancement phase 5, Samsung, RAN4#114

[17] R4-2500767, Further discussion of UE RF impact for NR MIMO phase 5, vivo, RAN4#114

[18] R4-2504686, Topic summary for [114bis][128] NR_MIMO_Ph5_UE, Moderator(Samsung), RAN4#114bis

[19] R4-2505111, WF on UE RF requirements for MIMO enhancement phase5, Samsung, RAN4#114bis

[20] R4-2503844, Further discussion of UE RF impact for NR MIMO phase 5, vivo, RAN4#114bis

[21] R4-2506863, draft CR for NR MIMO for 38.101-1, vivo, RAN4#115