3GPP TSG-RAN WG4 Meeting # 115	R4-2506865

Malta, MT, 19th – 23rd May, 2025

Source: 	vivo

Title: 	Discussion on SRS IL imbalance reporting

Agenda Item:	7.1.2.3.4

Document for:	Approval

Introduction

In RAN#103, a new WID [1] was approved on UE RF enhancements for NR FR1/FR2 and EN-DC.

In RAN4#110bis, the 6Rx related issue has been discussed. The topic summary is in [2] and a WF is agreed in [3]. A discussion paper in [4] is also provided for 6Rx related topics. Among this WF, the SRS IL imbalance is started. This has been discussed for a long time in REl-18 but no conclusion has been reached.

In RAN4#111, a dedicated agenda is set up for the SRS IL imbalance issue. A WF is agreed in [9] and summary is in [10], in which the common understandings were proposed to be discussed as the base for tentative solutions.

In RAN4#112, the common understandings were discussed. The summary is in [13] and WF in [14].

In RAN4#112bis, slight progress has been made and in WF [16], in that a new proposal 10 was introduced:

  - Proposal 10: For Rel-19 UE, at least, optionally indicates self-compensation of SRS IL up to its maximum Tx power capabilities.

In RAN4#113, the WF[18] was updated mainly for Issue 3-2-1, to have some more candidate solution framework, rather than a list of proposals from different companies.

In RAN4#114, a WF [20] was updated. In that the IL imbalance issue was also updated for issue 3-2-1. More complete candidate solution framework was updated. However, the whole set of frame work is still as “candidate” and no conclusion made, it is apparent that anything more than implementation would be difficult to be agreed anymore.

In RAN4#114bis, significant progress were made. There are some baseline agreements reached and possible solutions converged in WF [23].

In this contribution, our views on next stage way forward are proposed.

Discussion

In [23], the following WF has been confirmed for self-compensation:

Issue 4-2-1: SRS IL imbalance UE self-compensation

Agreement:

- RAN4 assumptions on UE behavior for Rel-19 SRS IL handling:

  - For SRS transmissions Rel-19 UE is expected to perform self-compensation of SRS IL up to its configured maximum output power

    - FFS if any additional clarifications on behavior are needed

    - No RAN1/4 specification impacts (i.e. this behavior is aligned with existing specifications)

Although there is clarification of no spec impact, it is already a significant progress here, to have a clear expectation that the self-compensation for SRS IL will be the de facto behaviour of Rel-19. It should be noted in the previous discussion, compensation behaviour should be optional. This is a starting point of changing the default behavior of UE.

Observation 1: Having expectation that Rel-19 UE would do compensation for SRS IL is already a large step for UE.

In addition, those self-compensation, is by far the most flexible, and already effective solution that already have precedents. It is proposed this is as far as we go in Rel-19.

Proposal 1: It is proposed not to introduce any signalling related to for SRS IL reporting.

Regarding further assistance and reporting , there is the following issue:

Issue 4-2-2: SRS IL imbalance UE assistance and reporting

Agreement:

- The following two possible general solutions may be considered for further consideration

  - Solution 1: Introduce UE capability indication on whether UE performs SRS IL self-compensation

    - FFS on the detailed solution including clarification on self-compensation and whether static or dynamic indication will be needed

  - Solution 2: Extend Type 3 SRS PHR reporting to carriers with PUSCH subject to RAN1 confirmation on feasibility

    - Further check whether solution 2 is in the scope or not

  - Considerations of any solutions should be based on system level throughput evaluations, and implementation complexity

Now the assistance information and reporting have converged to the two solutions, with solution 1 is the minimum reporting possible and solution 2 is a comprehensive and more complicated one.

In our previous papers, a compromised solution which is similar to solution 1 is also proposed. However, it should be noted, even this solution, there would be a potential need to specify a lot of compensation behaviours, such as the what is the expected compensation, when it would apply, to what extent the compensation would be, etc. These clarification may not be that easy to coverage, and still quite a lot discussion would be needed. However, the actual benefit is questionable, e.g. how this information would be possibly used? What is the real benefit compared to the implementation solution?

Observation 2: Even the simpler “Solution 1” on UE capability indication on whether UE performs SRS IL self-compensation may still controversial in details, and may not be more useful compared to fully implementation dependent.

For “solution 2”, a lot of discussion had already done, and majority companies have shown that this is not acceptable.

Observation 3: More complicated “Solution 2” can not be accepted by most companies in previous discussion.

Based on the observations, the following proposal is provided:

Proposal 2: Neither solution 1 nor solution 2 is preferred in Rel-19.

- Solution 1 can considered a compromise if quick convergence on details can be reached.

- Solution 2 should not be considered.

In addition, for the Rx number, there is the following issue:

Issue 4-1-2: SRS IL solution applicability to 2Rx/4Rx/6Rx/8Rx

Agreement

- FFS on the potential SRS IL solution applicability to 2Rx/4Rx/8Rx UEs in addition to 6Rx UEs

Our understanding is that 8Rx and 6Rx can be applied without much controversy, and they are also the cases which have the worst IL, that compensation is more meaningful.

For 4Rx, there were previous study that some impact is there but not large as 6Rx/8Rx. For 2Rx, this is almost have no impact at all.

Proposal 3: The potential SRS IL solution (including implementation only) can apply to 6Rx and 8Rx. 4Rx and 2Rx need not be mandatory.

Conclusion

In this paper, the following proposals were provided:

Observation 1: Having expectation that Rel-19 UE would do compensation for SRS IL is already a large step for UE.

Proposal 1: It is proposed not to introduce any signalling related to for SRS IL reporting.

Observation 2: Even the simpler “Solution 1” on UE capability indication on whether UE performs SRS IL self-compensation may still controversial in details, and may not be more useful compared to fully implementation dependent.

Observation 3: More complicated “Solution 2” can not be accepted by most companies in previous discussion.

Proposal 2: Neither solution 1 nor solution 2 is preferred in Rel-19.

- Solution 1 can considered a compromise if quick convergence on details can be reached.

- Solution 2 should not be considered.

Proposal 3: The potential SRS IL solution (including implementation only) can apply to 6Rx and 8Rx. 4Rx and 2Rx need not be mandatory.

- References

[1] RP-240828, New WID: UE RF enhancements for NR FR1/FR2 and EN-DC, Phase 4, Huawei, RAN#103

[2] R4-2405282, Topic summary for [110bis][129] NR_ENDC_RF_Ph4, Moderator (Huawei, HiSilicon), RAN4 #110bis

[3] R4-2406585, WF on requirements for 6Rx, AT&T, RAN4 #110bis

[4] R4-2404666, Discussion on 6Rx for handheld and FWA UE, vivo, RAN4 #110bis

[5] R4-2303519, [Draft] LS on the UE SRS IL imbalance issue, RAN4#106

[6] R1-2306200, FL summary on UE SRS IL imbalance issue, Moderator (Huawei), RAN1#112bis-e

[7] R4-2321793, [Draft] LS on the UE SRS IL imbalance issue, RAN4#109

[8] R4-2405933, Initial discussion on SRS insertion loss imbalance reporting and 6Rx requirements, Ericsson, RAN4 #110bis

[9] R4-2410751, WF on requirements for 6Rx, AT&T, RAN4 #110bis

[10] R4-2408937, Topic summary for [111][126] NR_ENDC_RF_Ph4_part3, Moderator(AT&T), RAN4 #110bis

[11] R4-2408122, Discussion on SRS IL imbalance reporting, vivo, RAN4#111;

[12] R4-2305082, Discussion on 8Rx UE RF requirements, vivo, RAN4#106bis

[13] R4-2412820, Topic summary for [112][118] NR_ENDC_RF_Ph4_part3, Moderator (AT&T), RAN4#112

[14] R4-2414285, WF on 6Rx UE requirements, AT&T, RAN4#112

[15] R4-2412094, Discussion on SRS IL imbalance reporting, vivo, RAN4#112

[16] R4-2417209, WF on 6Rx UE RF requirements, AT&T, RAN4#112bis

[17] R4-2415787, Discussion on SRS IL imbalance reporting, vivo, RAN4#112bis

[18] R4-2420376, WF on 6Rx UE RF requirements, AT&T, RAN4#112bis

[19] R4-2418199, Discussion on SRS IL imbalance reporting, vivo, RAN4#113

[20] R4-2503014, WF on 6Rx requirements, AT&T, RAN4#114

[21] R4-2500761, Discussion on SRS IL imbalance reporting, vivo, RAN4#114

[22] R4-2504672, Topic summary for [114bis][114] NR_ENDC_RF_Ph4_part3, Intel, RAN4#114bis

[23] R4-2505101, WF on 6Rx RF requirements and SRS IL imbalance, Intel, RAN4#114bis

[24] R4-2503846, Discussion on SRS IL imbalance reporting, vivo, RAN4#114bis