# 3GPP TSG-RAN WG4 Meeting #115	R4-2506867

Malta, MT, 19th – 23rd May, 2025

Source: 	vivo

Title: 	Discussion on general aspects of 6Rx requirements

Agenda Item:	7.1.2.3.1

Document for:	Approval

Introduction

The feature for 6Rx is one of important aspects for UE RF enhancements. In the last meeting, we discussed the general aspects of 6Rx requirements. The WFs are captured in [1]. In this contribution, we will further discuss the remaining issues for 6Rx.

Discussion

The WF [1] for general aspects of 6Rx were captured as follows:

Sub-topic 1-1: General aspects of 6Rx requirements Issue 1-1-1: Release independence of 6Rx RF requirements Agreement: FFS whether to define 6Rx RF requirements in a release independent manner Issue 1-1-2: FWA terminology Agreement: Further discuss possible clarifications and improvements on the use of FWA terminology in TS 38.101-1

6Rx RF requirements should release independence from Rel-19.

6Rx RF requirements mainly include 6Rx REFSENS and SRS-AS IL ΔTRxSRS. The ΔTRxSRS requirement involves many new UE capabilities, e.g. t3r6 SRS antenna switching, which are introduced from R19. If release independence from earlier releases is defined, there will be forward compatibility issues; this is one reason why release independence is not recommended. However, if there are special industry requests to support 6Rx from earlier releases, we can further discuss it.

Proposal 1：It is suggested 6Rx RF requirements to be release independent from Rel-19.

For FWA terminology, FWA/CPE/Vehicle/Industrial devices form factor is used for ΔRIB,8R. 8Rx feature is for non-handheld UE. FWA form factor shares the same understanding with FWA/CPE/Vehicle/Industrial devices form factor in terms of ΔRIB,8R.

Table 7.3.2-2a: Eight antenna port reference sensitivity allowance ΔRIB,8R

| Operating band | ΔRIB,8R (dB) |
| --- | --- |
| n7 | -4.5 |
| n41 | -4.3 |
| n77, n78, n79 | -4.0 |
| NOTE 1: 8 Rx operation is targeted for FWA/CPE/Vehicle/Industrial devices form factor. |  |

Unlike 8Rx, we only consider two UE types for 6Rx, Handheld or FWA, which is clearly stated in the WID. We cannot extend FWA form factor to FWA/CPE/Vehicle/Industrial devices form factor for 6Rx. The situation is the same for 4Rx. The wording doesn’t have to be aligned for 4Rx, 6Rx and 8Rx:

- For 4Rx and 6Rx, use ‘FWA form factor’

- For 8Rx, use FWA/CPE/Vehicle/Industrial devices form factor.

Proposal 2: It is suggested to use ‘FWA form factor’ for 6Rx when applying ΔRIB,6R requirement for FWA.

Conclusion

This contribution discusses general aspects for 6Rx. The following proposals are made:

Proposal 1：It is suggested 6Rx RF requirements to be release independent from Rel-19.

Proposal 2: It is suggested to use ‘FWA form factor’ for 6Rx when applying ΔRIB,6R requirement for FWA.

- References

[1] R4-2505101, WF on 6Rx RF requirements and SRS IL imbalance, Intel, RAN4#114bis.