# 3GPP TSG-RAN WG4 Meeting #115	R4-2506868

Malta, MT, 19th – 23rd May, 2025

Source: 	vivo

Title: 	Further discussion on SRS antenna switching requirements

Agenda Item:	7.1.2.3.3

Document for:	Approval

Introduction

SRS antenna switching requirements are one of important aspects for 6Rx requirements. In the last meeting, companies proposed draft CRs for SRS antenna switching and ΔTRxSRS requirements for 6Rx and all the draft CRs for 6Rx SRS-AS requirements were postponed. For t3r6, we agreed to send LS to RAN2 to introduce t3r6 SRS-AS UE capability after the release independent issues for 3Tx and 6Rx were clear. The WFs are captured in [1]. In this contribution, we will further discuss the remaining issues with the addition of t3r6.

Discussion

The WF [1] for SRS antenna switching and ΔTRxSRS of last meeting was captured as follows:

Issue 3-1-1: UE capability signalling updates for SRS antenna switching Agreement - Send LS to RAN2 to introduce UE capability signalling for t3r6 SRS antenna switching after the release independent for 3Tx in single band and 6Rx is clear.

However, it is not necessary for RAN4 to send the LS to introduce UE capability for t3r6. In Wuhan meeting, RAN1 has already sent the LS [2] on UE feature list to RAN2 and asked them to design corresponding UE capability. 3T6R Antenna Switching was introduced in NR_MIMO_Ph5.

Proposal 1: It is not necessary for RAN4 to send LS to RAN2 to introduce UE capability signalling for t3r6 SRS antenna switching.

In the last meeting, we tried to give the wording for condition when t1r6, t2r6, t3r6 SRS-AS IL requirements apply in our draft CR. The part for t3r6 needs clear conclusion from RAN1 and RAN2.

h)	UE transmits SRS on the second, third, fourth, fifth, sixth SRS resources of the total 6 SRS resources from all configured SRS resource set(s) consisting of one SRS port when the srs-AntennaSwitchingBeyond4RX-r17 capability is indicated as at least 't1r6', or

i)	UE transmits SRS from the SRS port pair on the second, third SRS resource in every configured SRS resource set consisting of two SRS ports when the srs-AntennaSwitchingBeyond4RX-r17 capability is indicated as at least 't2r6', or

[j) UE transmits SRS from the set of SRS ports on the second, third SRS resource in every configured SRS resource set consisting of three SRS ports when the srs-AntennaSwitchingBeyond4RX-r17 capability is indicated as at least 't3r6',]

Following table is the UE feature 3T6R antenna switching for NR_MIMO_Ph5:

| 59. NR_MIMO_Ph5 | 59-3-3 | 3T6R Antenna switching | 1. Support of 3T6R SRS Tx port switching with port 1003 disabled when 4 port SRS resources with port 1003 disabled are configured to the UE 2. Report the entry number of the first-listed band with UL in the band combination that affects this DL 3. Report the entry number of the first-listed band with UL in the band combination that switches together with this UL [4. Support of 3T6R antenna switching configuration(s) as an allowing downgrading configuration of 4T8R] | FFS | yes | 3TX 3T6R antenna switching is not supported | Per FS | Component 2 candidate value: {1,2, … 32} Component 3 candidate value: {1,2, … 32} [FFS: New component for downgrade antenna switching configurations or a new Note: This UE feature can be signalled together with srs-AntennaSwitching8T8R-r18, srs-AntennaSwitchingBeyond4RX-r17, supportedSRS-TxPortSwitch-v1610, or supportedSRS-TxPortSwitch to indicate SRS antenna switching downgrading capability for a UE with 4Rx, 6Rx or 8Rx.] | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

From the UE feature for t3r6 SRS antenna switching, it can be observed it requires further discussion on this UE feature and UE capability design. The condition when RAN4 apply t3r6 antenna switching requirements can wait for the conclusion from RAN1 and RAN2. When drafting the CR for this part, we can put brackets on the condition when the t3r6 SRS-AS IL requirement applies.

The feature srs-AntennaSwitchingBeyond4RX-r17 leverages antenna switching techniques to support more than 4Rx antennas without a proportional increase in hardware complexity or cost. However, the UE capability srs-AntennaSwitchingBeyond4RX-r17, it can not support indication of t3r6 now, which is shown as following table. In order to complete the requirements for t3r6, we can wait the conclusion for UE feature discussion for t3r6.

srs-AntennaSwitchingBeyond4RX-r17 Indicates whether the UE supports SRS Antenna switching for more than 4 Rx. The capability signalling comprises the following parameters: - supportedSRS-TxPortSwitchBeyond4Rx-r17 indicates a combination of supported xTyRs. It includes 11-bit bitmap, where starting from the leading / leftmost bit (bit 0), each bit corresponds to {t1r1, t2r2, t1r2, t4r4, t2r4, t1r4, t2r6, t1r6, t4r8, t2r8, t1r8}. For any indicated value, x shall be equal to or smaller than the one associated with the largest y. - entryNumberAffectBeyond4Rx-r17 indicates the lowest band entry number of the UL group (see entryNumberSwitchBeyond4Rx-r17) that impacts the DL of this band entry; - entryNumberSwitchBeyond4Rx-r17 indicates the lowest band entry of the UL group, which is defined as band entries with UL (see NOTE 1) that impact each other's UL (i.e. SRS TX port switching on any of the cells in the group will impact UL on all the cells in the group). This parameter is absent if an UL group contains only one band entry. The UE indicating support of this shall indicate support of srs-TxSwitch.

For the indication of ‘t3r6’, it can be different with ‘t1r6’ or ‘t2r6’ since the support of t3r6 SRS Tx port switching is based on 4Tx port with one port disable as described in the feature list. The indication of t3r6 can wait the conclusion in RAN1. The way to capture this condition whenΔTRxSRS for t3r6 is applicable in RAN4 can wait for the conclusion in RAN1.

Proposal 2: Put brackets on the condition when the t3r6 SRS-AS IL requirement applies when preparing CR for 6Rx RF requirements.

Conclusion

This contribution discusses ΔTRxSRS requirements for 6Rx considering SRS antenna switching configurations t3r6, t1r6-t3r6, t2r6-t3r6 and t1r6-t2r6-t3r6. The following proposals are made:

Proposal 1: It is not necessary for RAN4 to send LS to RAN2 to introduce UE capability signalling for t3r6 SRS antenna switching.

Proposal 2: Put brackets on the condition when the t3r6 SRS-AS IL requirement applies when preparing CR for 6Rx RF requirements.

- References

[1] R4-2505101, WF on 6Rx RF requirements and SRS IL imbalance, Intel, RAN4#114bis.

[2] R4-2505303_R1-2502979, LS on updated Rel-19 RAN1 UE features lists for NR after RAN1#120bis, RAN1, AT&T, NTT DOCOMO, INC.