3GPP TSG-RAN WG4 Meeting #115	R4-2506878

# Malta, MT, 19th – 23rd May, 2025

Agenda item:	 7.14.3.2

Source:	 vivo

Title:	Remaining issues for NTN OTA test method and performance metric

Document for:	Approval

- Introduction

RAN4 made good progress on NTN usage scenarios and performance metric last meeting [1].

Issue 2-1-2: performance metric for NR-NTN and IoT-NTN only support scenario 2

Agreements:

o   For IoT-NTN and NR-NTN, typical usage scenarios are:

§  Scenario 1: both head+hand mode and hand only mode.

§  Scenario 2: hand only mode targeted for specific UE orientation usage

§  Scenario 3: hand only mode targeted for arbitrary UE orientation usage

o   UE shall declare support for only one scenario per band.  The declaration details of each scenario should be further discussed.

o   For IoT-NTN and NR-NTN handheld UEs declared Scenario 2, the performance metrics are partial sphere metric. FFS details

o   For IoT-NTN and NR-NTN handheld UEs declared Scenario 3, the performance metrics are full sphere TRP and TRS

Based on the agreed scope of NTN OTA test method in WID [2]:

- Study and define test methodology and radiated performance metric for FR1 NTN devices (including NR NTN and IoT NTN)

  - Study the usage scenarios and develop enhanced test methodology

  - Study and specify the proper performance metric

  - Develop preliminary Measurement Uncertainty (MU)  (RAN5)

In this contribution, we discuss the remaining issues and next steps to finalize core part.

- Discussion

For now, there are a few detailed open issues for discussion:

- Details of declaration of each usage scenario

- For Scenario 2, detailed performance metric and positioning guidelines

- Finalization of the details of test procedure

- MU assessment table for NTN, especially scenario 2

Regarding declaration of each scenario, it can be clarified as following:

- For Scenario 1, the declaration should be based on the UE capability/usage that the NTN UE operates nearly the same as normal smartphone in TN scenario

- For scenario 2, the declaration should be based on the UE capability/usage that the NTN UE operates for non-real-time short message and UE can just operate under specific direction to satellite with assistance software/application

- For scenario 3, the declaration should be based on the UE capability/usage that the NTN UE operates under hand only mode with real-time service, e.g., data transmission or voice call with External speakers

Regarding the detailed performance metric and positioning guidelines for scenario 2, to cover most of devices operating under this case, half sphere would be a safer metric to quantify the performance. But we are open to discussing whether 120 degrees range is sufficient for this scenario. In addition, we believe Max EIRP value within the partial sphere is also a reasonable metric to show the NTN UE performance only operate under specific direction.

Proposal 1: RAN4 consider to add Max EIRP within the defined partial sphere as additional metric for NTN scenario 2. FFS whether consider it in future requirements.

Besides, the hand only positioning guideline should be different from traditional browsing mode hand only (DUT’s main display tilted 45 degrees from vertical). For scenario 2, the DUT’s main display should be aligned with vertical, i.e., similar to Free space case. This new positioning guideline should be reflected in the TR.

Proposal 2: use "UHRP" as baseline performance metric for scenario 2, and update the positioning guideline as “new” hand only, i.e., for NTN scenario 2 hand phantom only case, the DUT shall be mounted in a suitable hand phantom and oriented such that the DUT’s main display is aligned with vertical.

Regarding the detailed test procedure of NTN OTA, it is provided in the CR [3], refinement is needed for the final conclusion.

Regarding the MU assessment of NTN OTA, RAN4 should discuss and conclude whether any MU update is needed, whether the new hand only positioning has MU impact. If no, then the MU assessment of TN talk mode and browsing mode can be reused.

Proposal 3: RAN4 should discuss and conclude whether any MU update is needed for NTN OTA.

- Conclusion

In this contribution, we discuss remaining open issues for NTN OTA:

Proposal 1: RAN4 consider to add Max EIRP within the defined partial sphere as additional metric for NTN scenario 2. FFS whether consider it in future requirements.

Proposal 2: use "UHRP" as baseline performance metric for scenario 2, and update the positioning guideline as “new” hand only, i.e., for NTN scenario 2 hand phantom only case, the DUT shall be mounted in a suitable hand phantom and oriented such that the DUT’s main display is aligned with vertical.

Proposal 3: RAN4 should discuss and conclude whether any MU update is needed for NTN OTA.

# References

- R4-2504763,WF for [114bis][328] TRP_TRS_MIMO_OTA_Ph3, vivo, RAN4#104bis

- RP-250197, Revised WID of Rel-19 OTA, vivo, CAICT, RAN#107

- R4-2506880, draft CR to 38.870 on NTN OTA performance metric and test method, vivo, RAN#105