3GPP TSG-RAN WG4 Meeting #115	R4-2506884

# Malta, MT, 19th – 23rd May, 2025

Agenda item:	 7.28.3.2

Source:	 vivo

Title:	Discussion on LP-WUS REFSENS, ASCS and ACS

Document for:	 Approval

- Introduction

RAN4 made good progress on two types of LP-WUR, but the SNR and corresponding Rx requirements are still open. Meanwhile, RAN1 made key conclusions of LP-WUS design, few open aspects can be decided in RAN4 when discussing RF requirements and RMC parameters.

In this contritbution, we provide updated simulation results of Rx performance analysis by using latest agreements in RAN1 and RAN4 last meeting.

- LP-WUS UE RF

  - Link-level simulation for LP-WUS target SNR

Last meeting RAN4 agreed to update simulation results based on RAN1 signal design to specify the target SNR value of LP-WUR [1]:

Issue 2-1-1: SNR for OOK-based LP-WUS

Agreements:

- Companies are encouraged to provide the simulation results with the assumption of the RM coding of 8bit, 16bit and 32bit to align simulations in RAN4. It is targeted to conclude SNR value for envelope detection and OFDM sequence detection next RAN4 meeting.

We update the simulations with the following simulation parameters for OOK-4 M=4 in the table below:

Table 1: simulation parameters for LP-WUR in this contribution

| Attributes | Assumptions |
| --- | --- |
| Waveform | OOK-4 |
| Center frequency | 900MHz, 2.6GHz and 3.5GHz |
| Channel structure | 5 bits |
| RM coding | 8/16/32bit for OOK |
| CRC | NO |
| Chip rate | M=4 |
| overlaid OFDM sequence | 4 candidates overlaid sequences for M=4 |
| WUS duration | OOK：RM coding length*2/M OFDM：ceil(5/log2(4)/(M/2)) |
| Coding | 1/2 rate Manchester coding |
| Time error | 0 |
| Residual Frequency error | 0ppm |
| SCS | 30kHz |
| UE Channel BW | 20MHz (51 RB) |
| WUS RB | Fixed 11RB ~ 3.96MHz |
| Position within channel | Center for ASCS, edge for ACS |
| Guardband of NR channel, both wanted cell and interfer cell (ACS) | - For wanted signal: 665kHz for 10Mhz - For interference cell2 5MHz: fixed 505kHz |
| Guard RB | - For ASCS: 0 or 1RB on each side of LP-WUS bandwidth - For ACS: 1/2/3/4 RB |
| Filter | - 3th/5th Order lowpass Butterworth matching fixed 3.96MHz RF bandwidth the filter bandwidth is adapted with actual WUS RBs, for 5MHz case |
| ASCS | PDSCH mapped on RBs not used for LP-WUS and Guard RB; EPRE of PDSCH /EPRE of LP-WUS = 0 dB Same PSD with WUS signal |
| ACS | PDSCH mapped on interference RBs (11RB for 5MHz CBW), one side; EPRE of PDSCH /EPRE of in-band LP-WUS = [20~33] dB NOTE: decide the interference level depending on SNR |
| Wanted signal level | For ACS, REFSENS + 14 dB for LP-WUS as baseline, higher wanted power level is also considered |
| Sampling rate | 7.68MHz |
| ADC bit width | 4/8 bits ADC for ASCS/ACS |
| Phase noise | Reciprocal mixing is modelled for ASCS/ACS |
| Non-linearities | Not modelled |
| Power boosting | No |
| Channel Model | AWGN |
| Performance metric | - 1% MDR/BLER - 1% FAR as side condition |
| Receiver | - OOK: Hard decoding + Calculate the Hamming distance with the target sequence - OFDM: Soft decoding + ML detection |

The SNR simulation results for OFDM receiver and envelop receiver are in Figure 1, which do not consider the timing error and frequency error impacts on the SNR performance.

Figure 1: Simulated SNR of OFDM receiver and Envelope receiver (OOK-4 M=4)

The SNR value at 1% MDR of LP-WUS is presented in Table 2.

Table 2 OOK-4 M=4 SNR@1% MDR with 4bit ADC under AWGN (no repetition)

|  | OFDM | OOK |  |  |
| --- | --- | --- | --- | --- |
| RM coding | None | 8 bit | 16 bit | 32 bit |
| SNR@1% MDR | -5.5 dB | -2.5 dB | -5 dB | -7.5 dB |

It should be noted that the simulated SNR with 4bit ADC or 8bit ADC is nearly the same, within 0.2dB difference, so we just list 4bit results here. Mostlikely RAN1 would not make down-selection to single RM coding for OOK LP-WUS design, thus RAN4 should decide which SNR value to specify RF requirements. In RAN4 RF requirements, the SNR value is part of REFSENS performance, which is to verify the RF chain performance under a fixed set of parameters as specified in Reference Measurement Channels. Therefore, in general, each SNR value can be used given the corresponding RM coding would be specified in the reference measurement channel. Considering a balance of better coverage and typical receiver design, the SNR generated from RM coding of 16bit is suggested to specify requirements. In addition, another benefits is that RAN4 could use single SNR value for both OOK-based and OFDM-based receiver. Given timing error and frequency error is not considered in the simulation, thus we think a value a bit lower than the simulated results should be considered as target SNR for Rx requirements discussion.

Proposal 1: Use -4dB or -4.5dB as single SNR for both OOK-based and OFDM-based LP-WUR when discuss REFSENS requirements.

  - ASCS requirements

Last meeting RAN4 had agreed the guard RB and LP-WUS wanted signal level for ASCS requirements [1]:

Issue 2-3-1: ASCS requirements for OOK-based receiver

Agreement:

- For Type1 UE requirements, which are derived based on the assumption that demodulation of OOK content of LP-WUS is targeted

  - 1 RB guard for ASCS and 0.5dB de-sens

- For Type2 UE requirements, which are derived based on the assumption that demodulation of OFDM content of LP-WUS is targeted

  - 0 RB guard for ASCS and 0 dB de-sens

- For Type 1 and Type2 UE are not limited to specific LP-WUS waveform.

- FFS how to define test case and requirement applicability for Type 1 and Type 2

  - FFS how type 1 and type 2 capability related to RAN1 UE feature

- A UE can only support either Type 1 or Type 2 but not both for LP-WUR

Using the same simulation parameters in table 1, we also update the ASCS link level simulation results in Figure 2 and Figure 3.

Figure 2 SNR performance degradation for OFDM-based receiver ASCS (without guard RBs inside)

Figure 3 SNR performance degradation for Envelope receiver ASCS (without guard RBs inside)

The simulation results show that when using 3rd order filter the phase noise would have a bit larger degradation on SNR, which could introduce about 1.5dB worse. The SNR degradation in Figure 2 and 3 has been summarized in Table 3.

Table 3 ASCS degradation without guard RB

|  | OFDM with PN |  | OOK with PN |  |  |
| --- | --- | --- | --- | --- | --- |
| RM coding | None |  | 8bit | 16bit | 32bit |
| SNR@1% MDR | -3.9 dB (3rd filter) | -5.4 dB (5th filter) | -2.3 dB | -4.8 dB | -7.3 dB |
| Degradation | 1.6 dB | 0.1 dB | 0.2 dB | 0.2 dB | 0.2 dB |

Even without guard RB, the impact of adjacent NR signal on the OOK-based receiver is negligible. However, it has  a bit large impacts on OFDM-based receiver with 3rd order filtering, it means in some cases OFDM-based receiver also need guard RB to ensuer in-band performance.

In addition, if we also consider the impact of guard RB from BS transmission perspective, assuming the downlink signal is UE receiver agnostic, then same guard RB would be configured no matter the receiver side is OFDM detection or envelop detection. In this sense, we suggest to revisit the agreements laste meeting for ASCS that RAN4 should reconsider the same guard RB for ASCS type 1 and type 2 receiver.

Proposal 2: From LP-WUS signal transmission perspective, RAN4 should use same number of in-band guard RB for both type 1 and type 2 LP-WUR ASCS protection. We sugggest to consider 0RB guard for ASCS and 1dB de-sens, or 1RB guard for ASCS and [0] dB de-sens.

Last meeting, RAN4 also agreed to check whether high SNR case is needed for ASCS [1]:

Issue 2-3-4: whether high in-band signal (both LP-WUS and NR) case is considered for alternative ASCS requirements

Agreements:

  - FFS For ASCS, whether additional test case that the wanted signal is 14dB higher than LP-WUS REFSENS, i.e., REFSENSLP_WUR + 14 dBm, should be considered or not.

We did ASCS simulation with REFSENSLP_WUR + 14 dBm as wanted signal, and there is no ASCS degradation observed, we believe the high SNR case for ASCS is not needed.

Proposal 3: For ASCS, additional test case that the wanted signal is 14dB higher than LP-WUS REFSENS, i.e., REFSENSLP_WUR + 14 dBm, is not needed.

  - ACS requirements

For ACS, RAN4 agreed to use same interference level as MR ACS case for both type 1 and type 2 LP-WUR.

Issue 2-4-1: Baseline ACS requirements (same interference) and required guard RB for SET-1 (worse NF) LP-WUR

WF:

- Encourage companies to update the simulation analysis based on latest agreements. Target to conclude ACS requirements and test parameters next meeting.

Issue 2-4-2: Whether adopt the same wanted power level for Set-2 LR?

Agreement:

- The same set of the interference signal level defined for both two types of the LP-WUS requirements.

Then, the LP-WUR ACS performance is impacted by the following aspects in the equation:

ACSLPWUS=NFlegacy-NFLPWUS+SNRlegacy-SNRLPWUS+10logCBWBWLPWUS-DgMR+ACSlegacy+deltawantedsignal

In our analysis, we consider ACS legacy = 33 dB, CBW = BW LPUWS, Dg MR = 3dB (from 2Rx UE), and the following SNR value for OOK-based and OFDM-based receiver to analyze the ACS performance of LP-WUR type 1 and type 2.

Table 4 different SNR value for ACS analysis

| Assumed SNR | Note |
| --- | --- |
| OFDM#1 = -5 dB | 0.5 dB difference from ideal SNR in table 2 |
| OFDM#2 = -4 dB | 1.5 dB difference from ideal SNR in table 2 |
| OOK #3 = -4.5 dB | 0.5 dB difference from ideal SNR in table 2 with 16bit RM coding |
| OOK #4 = -3.5 dB | 1.5 dB difference from ideal SNR in table 2 with 16bit RM coding |

The ACS simulation results for type 1 and type 2 LP-WUR can be summarized as following:

Table 5 Guard RB for set 1, with wanted signal as LR REFESENS+14dB

| Set1 NF=15.5 dB | Guard RB number |  |  |  |
| --- | --- | --- | --- | --- |
| ADC | 4 bit |  | 8 bit |  |
| Filter | 3rd | 5th | 3rd | 5th |
| OFDM#1 = -5 dB | 0 | 0 | 0 | 0 |
| OFDM#2 = -4 dB | 0 | 0 | 0 | 0 |
| OOK #3 = -4.5 dB | >6 | >6 | 2 | 0 |
| OOK #4 = -3.5 dB | >6 | 6 | 2 | 0 |

Table 6 Guard RB for set 2, with wanted signal as LR REFESENS+14dB

| Set2 NF=11dB | Guard RB number |  |  |  |
| --- | --- | --- | --- | --- |
| ADC | 4 bit |  | 8 bit |  |
| Filter | 3rd | 5th | 3rd | 5th |
| OFDM#1 = -5 dB | 6 | 5 | 5 | 4 |
| OFDM#2 = -4 dB | 4 | 3 | 3 | 3 |
| OOK #3 = -4.5 dB | >6 | >6 | 6 | 4 |
| OOK #4 = -3.5 dB | >6 | >6 | 5 | 3 |

With LR REFSENS+14dB as wanted signal level, if we assume type 1 is a low-end LP-WUR with 4bit ADC and type 2 is high-end LP-WUR with 8bit ADC, then it still needs 6 guard RBs each side for ACS protection.

To further discrease the required number of guard RBs, updated simulation with higher wanted signal (LR REFESENS+18.5dB) is performed, as summrized in table 7 and table 8.

Table 7 Guard RB for set 1, with wanted signal as LR REFESENS+18.5dB

| Set1 NF NF with REFSENS+18.5 dB | Guard RB number |  |  |  |
| --- | --- | --- | --- | --- |
| ADC | 4 bit |  | 8 bit |  |
| Filter | 3rd | 5th | 3rd | 5th |
| OFDM#1 = -5 dB | 0 | 0 | 0 | 0 |
| OFDM#2 = -4 dB | 0 | 0 | 0 | 0 |
| OOK #3 = -4.5 dB | 1 | 0 | 0 | 0 |
| OOK #4 = -3.5 dB | 0 | 0 | 0 | 0 |

Table 8 Guard RB for set 2, with wanted signal as LR REFESENS+18.5dB

| Set2 NF with REFSENS+18.5 dB | Guard RB number |  |  |  |
| --- | --- | --- | --- | --- |
| ADC | 4 bit |  | 8 bit |  |
| Filter | 3rd | 5th | 3rd | 5th |
| OFDM#1 = -5 dB | 0 | 0 | 0 | 0 |
| OFDM#2 = -4 dB | 0 | 0 | 0 | 0 |
| OOK #3 = -4.5 dB | >6 | >6 | 2 | 0 |
| OOK #4 = -3.5 dB | >6 | 6 | 2 | 0 |

To summarize Table 7 and table 8 ACS performance and required wanted signal level, Table 9 summarize the detailed parametermeters for ACS, wanted signal level and gurard RB for different SNR case.

Table 9 Sumamrized ACS performance of set 1 and set 2 with different consideration of wanted signal level (same interference as MR)

| NF legacy | SNR legacy | NF LPWUS | ADC | SNR LPWUS | ACSLPWUS | ACS Wanted signal level | Guard RB |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 9 dB | -1 dB | Set1: 15.5 dB | 4 bit | OFDM#1 = -5 dB | 23 dB | LR REFSENS+18.5 dB | 0 or 1 |
|  |  |  |  | OFDM#2 = -4 dB | 22 dB |  |  |
|  |  |  |  | OOK #3 = -4.5 dB | 22.5 dB |  |  |
|  |  |  |  | OOK #4 = -3.5 dB | 21.5 dB |  |  |
|  |  | Set2: 11 dB | 8 bit | OFDM#1 = -5 dB | 27.5 dB | LR REFSENS + 18.5 dB | 0 or 1 |
|  |  |  |  | OFDM#2 = -4 dB | 26.5 dB |  |  |
|  |  |  |  | OOK #3 = -4.5 dB | 27 dB |  |  |
|  |  |  |  | OOK #4 = -3.5 dB | 26 dB |  |  |

Proposal 4: Specify the wanted signal level as LR REFSENS+18.5dB as single value for both type 1 and type 2. The guard RB can be ≤1. The ACS requirements can be calculated accordingly.

  - REFSENS requirements

Assume the SNR=-4dB, the REFSENS requirements based on set 1 and set 2 NF can be decided. Given the number of RB for LP-WUS is a fixed value, i.e., 11 RBs with 15kHz/30kHz SCS, then single REFSENS for each band at each SCS is needed. Rather than listing the REFSENS value per-band for LP-WUS, it is better to use 10 MHz MR REFSENS (10MHz support both 15kHz/30kHz SCS, and for many bands, 10MHz is the min CBW with 30kHz SCS) as a reference to discuss the offset value, which could be a more efficient way to finalize LP-WUR requirements for all the bands.

∆REFSENSLPWUS=NFlegacy-NFLPWUS+SNRlegacy-SNRLPWUS+10logCBWBWLPWUS-DgMR

Observation 1: Given the NF of MR is slightly different each band, rather than listing all bands in the table and recalculate the REFSENS value, a simple way to define LP-WRU REFSENS is to specify the delta REFSENS based on a reference MR REFSENS value of each band, e.g., 10MHz CBW REFSENS value.

Proposal 5: Specify the delta REFSENS based on a MR REFSENS value of each band as reference, e.g., 10MHz CBW REFSENS value.

The proposed spec update could be:

For an OOK-based LP-WUR UE equipped with 1 Rx antenna port, 10MHz reference sensitivity for 2Rx antenna ports in Table 7.3.2-1a and in Table 7.3.2-1b shall be modified by the amount given in ΔR1R in Table 7.3M.2-1 for LP-WUR at the applicable operating bands.

Table 7.3M.2-1: OOK-based LP-WUR Single antenna port reference sensitivity allowance ΔR1R

| Operating band | LP-WUS bandwidth (RB) | ΔR1R (dB) of 10MHz REFSENS |
| --- | --- | --- |
| TDD band | 11 | [TBD] |
| FDD band | 11 | [TBD] |

  - RMC for LP-WUR requirements

The reference measurement channels for LP-WUR is provided in table below:

Table A.3.4.2-1 Fixed reference channel for LP-WUR requirements

| Parameter | Unit | Value |
| --- | --- | --- |
| MR Channel bandwidth | MHz | All CBW |
| L-WUS bandwidth | RB | 11 |
| Subcarrier spacing | kHz | 15/30kHz |
| RM coding | bits | 16bits |
| CRC |  | No CRC |
| Chip rate |  | M=4 (4 chips in an OFDM symbol) |
| overlaid OFDM sequence |  | Length 33: generated by 31-length ZC sequence with extension |
| Number of overlaid OFDM sequence per chip to carry information |  | 4 (up to 2 roots) |
| WUS duration for OOK |  | 8OFDM symbols |
| WUS duration for OFDM |  | 2 OFDM symbols |
| Manchester coding for OOK |  | 1/2 |
| Number of information bits |  | 5bits |

Proposal 6: Consider the Fixed reference channel for LP-WUR requirements in table below:

Table A.3.4.2-1 Fixed reference channel for LP-WUR requirements

| Parameter | Unit | Value |
| --- | --- | --- |
| MR Channel bandwidth | MHz | All CBW |
| L-WUS bandwidth | RB | 11 |
| Subcarrier spacing | kHz | 15/30kHz |
| RM coding | bits | 16bits |
| CRC |  | No CRC |
| Chip rate |  | M=4 (4 chips in an OFDM symbol) |
| overlaid OFDM sequence |  | Length 33: generated by 31-length ZC sequence with extension |
| Number of overlaid OFDM sequence per chip to carry information |  | 4 (up to 2 roots) |
| WUS duration for OOK |  | 8OFDM symbols |
| WUS duration for OFDM |  | 2 OFDM symbols |
| Manchester coding for OOK |  | 1/2 |
| Number of information bits |  | 5bits |

- Conclusion

In this contribution, we share our views on LP-WUR REFSENS, ASCS and ACS:

Proposal 1: Use -4dB or -4.5dB as single SNR for both OOK-based and OFDM-based LP-WUR when discuss REFSENS requirements.

Proposal 2: From LP-WUS signal transmission perspective, RAN4 should use same number of in-band guard RB for both type 1 and type 2 LP-WUR ASCS protection. We sugggest to consider 0RB guard for ASCS and 1dB de-sens, or 1RB guard for ASCS and [0] dB de-sens.

Proposal 3: For ASCS, additional test case that the wanted signal is 14dB higher than LP-WUS REFSENS, i.e., REFSENSLP_WUR + 14 dBm, is not needed.

Proposal 4: Specify the wanted signal level as LR REFSENS+18.5dB as single value for both type 1 and type 2. The guard RB can be ≤1. The ACS requirements can be calculated accordingly.

Observation 1: Given the NF of MR is slightly different each band, rather than listing all bands in the table and recalculate the REFSENS value, a simple way to define LP-WRU REFSENS is to specify the delta REFSENS based on a reference MR REFSENS value of each band, e.g., 10MHz CBW REFSENS value.

Proposal 5: Specify the delta REFSENS based on a MR REFSENS value of each band as reference, e.g., 10MHz CBW REFSENS value.

Proposal 6: Consider the Fixed reference channel for LP-WUR requirements in table below:

Table A.3.4.2-1 Fixed reference channel for LP-WUR requirements

| Parameter | Unit | Value |
| --- | --- | --- |
| MR Channel bandwidth | MHz | All CBW |
| L-WUS bandwidth | RB | 11 |
| Subcarrier spacing | kHz | 15/30kHz |
| RM coding | bits | 16bits |
| CRC |  | No CRC |
| Chip rate |  | M=4 (4 chips in an OFDM symbol) |
| overlaid OFDM sequence |  | Length 33: generated by 31-length ZC sequence with extension |
| Number of overlaid OFDM sequence per chip to carry information |  | 4 (up to 2 roots) |
| WUS duration for OOK |  | 8OFDM symbols |
| WUS duration for OFDM |  | 2 OFDM symbols |
| Manchester coding for OOK |  | 1/2 |
| Number of information bits |  | 5bits |

# References

- R4-2505234, WF for 114bis131 NR_LPWUS_UERF, vivo, RAN4#114bis