3GPP TSG-RAN WG4 Meeting #115	R4-2506886

# Malta, MT, 19th – 23rd May, 2025

Agenda item:	 7.28.3.4

Source:	 vivo

Title:	Discussions on LP-WUS Testability

Document for:	 Approval

- Introduction

Issue 4-1-1: Whether LR can be tested directly

Agreements:

- Down select below two approaches for LP-WUR UE RF requirements:

- Option 1: stick to 1% MDR LP-WUS as performance metric, but with a note in TS that LP-WUR ON is not mandated.

  - The note depends on RAN1 conclusion

- Option 2: update it to 1% MDR of PDCCH triggered by LP-WUR

Issue 4-1-4: UE test mode for LP-WUR

Agreements:

- Test mode design as well as the details of test mode for LP-WUS verification can be left to RAN5 with RAN4 inputs.

In last RAN4 meeting, it was agreed that the test mode design and details of test mode for LP-WUS verification can be left to RAN5 which means the detailed procedure will not be discussed in Rel-19 WI phase. However, one key issue related to testability should be concluded in RAN4 first, which is the test metric of RF requirements.

RAN4 agreed to use 1% MDR of LP-WUS as performance metric and conformance test in the beginning of the WI, and perform the evaluations accordingly, this is also well aligned with RAN1 approach for coverage and other physical layer design discussions. However, there was views in RAN4 that there is no mandatory action to ensure LP-WUR is always on in the real network behavior, then RAN4 should not use LP-WUS detection as metric for LP-WUR.

Companies have different understandings on how to specify RAN4 RF requirements, some companies believe that the RF performance verification does not mean the mandating of LP-WUR on and it is common sense that RF testing may not be well aligned with UE behavior in the real network, and RF testing is mainly to ensure the RF elements performance.

In this contribution, we share our views on LP-WUS RF requirements testability issues.

- LP-WUS UE RF testability

The newly proposed option 2 in WF was from the demodulation test case of power saving:

##### 5.3.2.1.3	Minimum requirements for power saving

During the test the UE shall monitor the DCI format 2_6 PDCCH in DRX off state and decide whether to receive the following PDCCH in DRX on period.

The parameters specified in Table 5.3.2.1.3-1 are valid for FDD test unless otherwise stated.

The argument was that the LP-WUR monitoring and MR monitoring can operate intertwined, and network does not know. This case would happen in a UE in the real network, but this is not the intention of LP-WUR RF performance verification. The MR performance has been well verified based on current core requirements in the spec. The target in this WI is to ensure the RF performance of LP-WUR, mainly the hardware part.

If we go with option 2, then there would be a situation that a UE does not support LP-WUR, all the PDCCH decoding are done by MR, but the UE can pass LP-WUR RF requirements, this is far away from the goal of this Rel-19 LP-WUR WI.

- Specify the necessary RAN4 core requirement(s) to support the feature (RAN4).

  - Specify UE low-power wake-up receiver requirements, at least REFSENS, ACS and ASCS requirements with consideration of possible new methodology to assess the low-power wake-up receiver performance

    - Define guard RBs for ACS and ASCS cases

    - Study testability of above requirements

    - Consider impacts of different architecture and impairments, and set requirements that enable all types of reasonable implementation

  - Study and if necessary specify or support by declaration, the corresponding BS requirements, e.g., dynamic range for LP-WUS/LP-SS.

    - Current NR BS requirements is baseline

  - Specify necessary RRM requirements

In addition, in the WID, the objective of RAN4 core requirements is to specify LP-WUR requirements, rather than wake up performance of a UE combining LP-WUR and MR.

Observation1: In the WID, the objective is clearly focused to specify UE RF requirements for low-power wake-up receiver.

The UE test mode is to ensure that the UE operation condition is well fitted the target of verification purpose but not means the real UE behavior is mandated accordingly. There are a lot of test mode cases not aligned with UE behavior but just for testing purpose. For example, the antenna locking test mode for FR1 radiated performance, the continuously uplink power up test mode for conductive testing, and the FR2 beam locking mode, are all test mode not used in the network.

Observation2: A Test mode is not only to ensure consistency, simplicity and repeatability in Testing, but also to address complex/extreme testing scenarios, e.g., highest/lowest signal case, radiated power of specific direction, turning off unnecessary functions, high-traffic scenarios, and so on.

Overall, we believe RAN4 should go with option 1. It can be clarified that the RF requirement is trying to verify partial operation condition of a UE support LP-WUR in the network, i.e., LP-WUR on condition. With LR off or MR monitor case, there is no need to specify LP-WUR RF requirements accordingly.

Proposal 1: Stick to 1% MDR of LP-WUR as performance metric, also add a note in TS that LP-WUR always ON is not mandated in a UE.

- Conclusion

In this contribution, we share our views on LP-WUR testability issue.

Observation1: In the WID, the objective is clearly focused to specify UE RF requirements for low-power wake-up receiver.

Observation2: A Test mode is not only to ensure consistency, simplicity and repeatability in Testing, but also to address complex/extreme testing scenarios, e.g., highest/lowest signal case, radiated power of specific direction, turning off unnecessary functions, high-traffic scenarios, and so on.

Proposal 1: Stick to 1% MDR of LP-WUR as performance metric, also add a note in TS that LP-WUR always ON is not mandated in a UE.

# References

- R4-2505234, WF for 114bis131 NR_LPWUS_UERF, vivo, RAN4#114bis.

- RP-241824, Revised WID: Low-power wake-up signal and receiver for NR (LP-WUS/WUR), vivo, NTT DOCOMO, RAN#105.