# 3GPP TSG-RAN WG4 Meeting #115	R4-2506900

Malta, MT, 19th – 23rd May, 2025

Agenda Item:	7.28.5.2

Source: 	Ericsson

Title:	Discussion on LP-WUS RRM requirement

Document for:	Discussion

# Introduction

In last RAN4 meeting, RAN4 had discussed the LP-WUS/WUR WI and made the following agreement [1].

Issue 1-1-10: Considerations on higher priority frequency layer Agreement: - Support relaxed higher priority frequency layer search at case 1 Issue 1-1-12: On LR measurement metrics LP-RSRP and LP-RSRQ Agreement: - For frequency range 1, the reference point for the LP-RSRP and LP-RSRQ shall be the antenna connector of the UE Issue 1-2-1: General on LR accuracy requirement Agreement: No dedicated accuracy requirement is defined in the performance section for LP-WUR based RRM measurement in Idle/inactive states, and reflect the accuracy performance as a margin relative to the threshold in the core requirement. Issue 1-2-1-1: Detail on LR accuracy and side conditions requirements Agreement: For FR1: - [3.5] dB can be used for core requirements for LP-RSRQ accuracy and [5.5 or 6] dB can be used for core requirements for LP-RSRP accuracy, under the side conditions Ês/Iot [x, x = -3] dB - [3.5] dB can be used for core requirements of SSB based RSRQ accuracy and [5.5 or 6] dB can be used for core requirements for SSB based RSRP accuracy, under the side conditions Ês/Iot [y, y = -3] dB - Note: [2 or 2.5] dB RF impairment margin is assumed Issue 1-2-2: Periodicity for LP-WUR RRM requirements Agreement: Regardless of whether LP-SS is configured or not, periodicity for SSB based LP-WUR measurement delay requirements is based on LO periodicity. Issue 1-2-4-1: On LR measurement filtering requirement Agreement: - The UE shall filter the [SS-RSRP and SS-RSRQ] measurements of the serving cell using at least 2 measurements. Within the set of measurements used for the filtering, at least two measurements shall be spaced by [LO periodicity /2] for SSB based LR. Issue 1-2-5-2: On MR evaluation requirements Agreement: - Reuse legacy Nserv - Note: the legacy values are Nserv = 4 when DRX = 0.32s and 0.64s, Nserv = 2 when DRX = 1.28 and 2.56s Issue 1-2-6: On MR wake up delay related requirements for LP-WUR and RAN1 LS R1-2501624 Issue 1-2-6-2: On SSBs number for different SSB periodicities Agreement Same SSB number for different SSB periodicity Agreement When SSB periodicity is 5 ms or 10ms, a lower bound 20 ms will be used for MR wake up delay. Note: This above agreement based on the assumption that the wake up duration is defined as the ramping time + SSB number * SSB periodicity Issue 1-2-7: On LP-WUR cell selection evaluation requirements for case 1 Agreement: - No need to define the LR-WUR based cell selection criterion evaluation in case 1.

In this contribution, we continue the discussion on the remaining issues of LP-WUS.

- General aspects

Issue 1-1-1: Cases/states to be considered for RRM relaxation and serving cell measurement offloading

Based on LP-WUS WID, different scenarios to support the RRM relaxation shall be defined as below:

- MR further relaxation with LR

- MR offloading by LR

In last RAN4 meetings, RAN4 agreed the case #1, #3, but the other MR relaxation case is FFS.

RAN4 to further discuss case #2 to #4: RRM measurement case index; MR serving cell measurement; MR neighboring cell measurement; LR measurement. #1 Fully offloading case; Off; Off: FFS the condition and the details; ON. #2 Relaxed case a; On with relaxation measurement; Off; ON. #3 Relaxed case b; On with relaxation measurement; On with relaxation measurement; ON. #4 Relaxed case c; Off; On, FFS the condition and the details; ON.

When both NW and UE support LP-WUS feature, different scenarios are valid by the NW using different thresholds and the UE will need to adapt its measurement behaviour to the respective zones. Adapting herein comprises turning ON/OFF the MR or LR receiver chains.

In legacy release, when neighbour cell measurement relaxation criteria were introduced, UE may perform neighbour cell measurements following normal measurement requirements, relaxed measurement requirements till completely stop the measurement. Now in Rel-19 LP-WUS, different serving cell measurement relaxation scenarios will be introduced. As shown in the figure below, different thresholds for neighbour cell measurement and serving cell measurement can be introduced for different serving cell and neighbour cell combinations.

Figure 1 Serving and neighbour cell measurement scenarios

There are still FFS for some scenarios. When UE enters LP-WUS mode, UE will use LR to monitor the serving cell quality without MR to achieve the power saving gain. Due to UE does not to combine the measurement results across MR and LR, it seems scenario 2 is not useful.

RAN4 #112 Issue 1-1-1: Cases/states to be considered for RRM relaxation and serving cell measurement offloading Agreement: - Support case #3 - For serving cell measurement, further discuss: - Option 1: Further discuss whether to combine the measurements across the two radios, i.e., MR and WUR - Option 2: From RAN4 requirement perspective, not consider combining the measurements across the two radios, i.e., MR and WUR - Option 3: It is up to UE implementation whether to combine the measurement across the two radios. - Other options are not precluded. RRM measurement case index; MR serving cell measurement; MR neighboring cell measurement; LR measurement. #3 Relaxed case b; On with relaxation measurement; On with relaxation measurement; ON. - Note: In RAN4 understanding, the discussion of the related measurement criteria (i.e., whether to use the legacy or new criteria) is out of RAN4 responsibility. Recommendations: Suggest to discuss issues related to higher priority frequency layers under 1-1-11. Continue discuss case 2 and other issues.

Proposal 1: RAN4 not to introduce scenario 2 ‘both MR and LR for serving cell measurement but neighbour cell measurement is OFF’.

Issue 1-1-2: Threshold for switch between different cases

In last meeting, we noticed there is an open issue to discuss how to define a threshold for switching between scenarios.

Issue 1-1-2: Threshold for switch between different cases - Proposals - P1: Up to RAN2 decision (CMCC CT) - P2: New LP-based threshold(s) based on LP-RSRP or LP-RSRQ used for switch into case 1 from other cases or exit from case 1 to other cases are needed (Apple ZTE vivo) - P2-1: LR measurement result shall be comparable to MR measurement result or shall be equivalent to MR measurement result with certain offset/margin (e.g., LR threshold is MR threshold + offset/margin). (Apple ZTE) - P2-2: Details on threshold are up to RAN2 (vivo) - P3: Discuss first what has been agreed in RAN2 (Nokia) - P4: RAN4 to study switching between Case#3 and Case#1 based on a timer. Since UE need to be confident enough to enter Case#1 before deactivating MR, it is better for the UE to enter Case#3 first for a period of time (confidence period) before UE can enter Case#1. (MTK)

In our understanding, whether and how to introduce the new threshold(s) for switching between different cases should be a RAN2 work. From our information, RAN2 is also discussing the thresholds. Therefore, RAN4 should leave such discussion RAN2.

Proposal 2: Whether and how to introduce new thresholds for switching between different scenarios should be up to RAN2 decision.

- RRC-CONNECTED mode

RAN4 shall focus on developing the RRM requirements for IDLE and INACTIVE mode during this meeting since not many agreements have been reached in other WGs for CONNECTED mode. Discussions on CONNECTED mode shall therefore be postponed until any agreements are achived in other WGs regarding, and if so RAN4 can check the RRM requirements for those at later stage.

Proposal 3: Discussions on LP-WUS CONNECTED mode requirements are postoponed until more progress is achieved in other WGs.

- Others

EMR

In last meeting, some companies mentioned to discuss UE’s behaviour in EMR. In legacy IDLE/Inactive mode RRM requirement, the relaxation of neighbour cell measurements is not allowed when EMR is configured on that frequency layer and T331 is running. However, RAN4 never discussed the UE’s behaviour when NW configures EMR in MR relaxation and offloading scenarios. Thus, we support RAN4 to discuss UE’s behaviour of MR relaxation with EMR.

Issue 1-5-1: LR based RRM with EMR in IDLE/Inactive mode - Proposals - P1: For a UE which supports idleInactiveNR-MeasReport-r16 or idleInactiveEUTRA-MeasReport-r16, and serving cell configures carriers for idle mode CA/DC measurement reporting with T331 running, UE shall keep the MR ON for EMR measurement regardless of the MR offloading condition is met or not. (Apple) - P2: The principle of P1 is reasonable. The issue can be discussed later (vivo) - P3: The methodology of EMR requirement definition under each Cases can refer to the methodology of high priority layer measurement requirement definition. (CMCC) - P4: Discuss the UE behaviour while MR relaxation is applied to idle-mode measurements. (Nokia) - If T331 timer is configured UE shall not enter LP-WUS mode (MR relaxation / offloading) (e.g. for the first 300 seconds in idle-mode) - RAN4 to also discuss if the rel-18 idle-mode behavior follows the same principle where UE shall keep the MR ON.

Proposal 4: RAN4 to discuss the UE’s behaviour of MR relaxation with EMR impact.

RAN1 LS

In RAN1 #120 meeting, RAN1 sent a LS to RAN4 to ask about the number of SSBs for wake-up.

In our understanding, when UE wakes up from deep sleep mode, UE can obtain the timing information from LR. In last meeting, UE vendors mentioned that 1-2 SSBs are needed for AGC retuning similar as legacy handover case. On top of that 1 SSB is required for fine timing tracking. We think it looks reasonable to use total 3 samples. When UE wakes up from ultra-deep sleep mode, we understand UE may not have any prior information. Additional SSBs for coarse sync are fine. Therefore, we prefer to send LS to confirm RAN1’s WS based on RAN4 agreement in last meeting.

In RAN1#118bis, it was agreed to support 3 candidate values for the wake-up delay capability a UE reports for LP-WUS operation in IDLE/INACTIVE mode. In addition, the following agreement was made in RAN1#120: Agreement For UE capability report on the wake-up delay: - Alt 1: For the 3 candidate values for the wake-up delay capability report, support {[70ms], [500ms] and [900ms]}. - The reported values assume SSB periodicity of 20ms, where [70ms] assumes [3] SSBs needed for synchronization, [500ms] and [900ms] assume [5] SSBs needed for synchronization. - FFS: translation of wake-up delay for different SSB periodicities - FFS: different sets of 3 candidate value for different SSB periodicities Note that the definition of wake-up delay is the minimum gap time between LP-WUS reception and MR to start PDCCH monitoring. RAN1 respectfully asks RAN4 to check and confirm the number of SSBs currently assumed by RAN1 for MR synchronization for SSB periodicity of 20ms. RAN1 also respectfully asks RAN4 to confirm if it is reasonable to assume the same number of SSBs for MR synchronization for different SSB periodicities.

RAN4 #114bis Issue 1-2-6-2: On SSBs number for different SSB periodicities Agreement Same SSB number for different SSB periodicity Agreement When SSB periodicity is 5 ms or 10ms, a lower bound 20 ms will be used for MR wake up delay. Note: This above agreement based on the assumption that the wake up duration is defined as the ramping time + SSB number * SSB periodicity

Proposal 5: RAN4 to confirm RAN1’s WS based on RAN4’s agreements in last meeting.

RedCap UE capability

In Rel-19 LP-WUS WID, it clearly explained the use case for LP-WUS as below. Energy efficiency is even more critical for UEs without a continuous energy source. Among vertical use cases, sensors and actuators are deployed extensively. Wearables include smart watches, rings, eHealth related devices, and medical monitoring devices. With typical battery capacity, it is challenging to sustain up to 1-2 weeks as required. In our understanding, RedCap UE is a typical use case for LP-WUS.

In last RAN1 meeting, it clearly mentioned RedCap UE can support LP-WUS if reported. Furthermore, LP-WUR is assumed as a new engine. All the related requirements and simulations can be independent with UE type. No additional evaluation efforts is needed. Therefore, when RAN4 defines the LP-WUS related requirement, RAN4 shall consider both normal UE and RedCap UE. RedCap UE shall also meet LP-WUS related requirement.

RAN1 #120bis meeting Agreement For RRC idle/inactive, support Alt 2 for both Redcap and non-redcap UEs: - Alt 2: LP-WUS/LP-SS frequency resource can be out of initial DL BWP where UE receives paging but within the same carrier. - FFS whether to define the maximum frequency gap between LP-WUS/LP-SS and SSB

Proposal 6: RedCap UE shall meet LP-WUS related requirement based on UE’s capability.

- Summary

In this contribution, we have discussed the LP-WUS operation. Based on the discussions, we have made following proposals and observations:

Proposal 1: RAN4 not to introduce scenario 2 ‘both MR and LR for serving cell measurement but neighbour cell measurement is OFF’.

Proposal 2: Whether and how to introduce new thresholds for switching between different scenarios should be up to RAN2 decision.

Proposal 3: Discussions on LP-WUS CONNECTED mode requirements are postoponed until more progress is achieved in other WGs.

Proposal 4: RAN4 to discuss the UE’s behaviour of MR relaxation with EMR impact.

Proposal 5: RAN4 to confirm RAN1’s WS based on RAN4’s agreements in last meeting.

Proposal 6: RedCap UE shall meet LP-WUS related requirement based on UE’s capability.

- References

[1]. R4-2502600, “WF for RRM requirements for LP-WUS/WUR”, vivo

[2]. R1-2501624, “LS on the wake-up delay for LP-WUS operation in IDLE/INACTIVE mode”, Apple