3GPP TSG-RAN WG4 Meeting #115		R4-2506916
Malta, MT, 19th – 23rd May, 2025

Title:	R19 SRS IL indication

Source:	OPPO

Agenda item:	7.1.2.3.4

Document for:	Approval

- Introduction

SRS IL reporting has been discussed since Rel-18, and in last meeting WF [1] is agreed though still doesn’t has conclusion on whether to report or how to report the SRS IL indication. This paper further shares our view on this based on our views shared meetings ago.

- Discussion

2.1 UE compensation

It was agreed in last meeting that in this SRS IL reporting topic, it assumes UE will do compensation up to its Pcmax, meanwhile during the discussion it was clarified that whether UE will do self-compensation is up to UE implementation in reality. This means if UE support this reporting feature it will do the compensation, otherwise, there is no restriction.

Observation 1:   If UE support SRS IL reporting feature it will do the compensation, otherwise, there is no restriction.

2.2 SRS IL changes

SRS IL depends on the RFFE paths that each SRS resource maps to the physical antennas. And usually, UE will implement more than 4 antennas for each band to avoid the problems like hand blocking issues. When UE detects there is hand blocking problems, it will change the antennas used for Tx and Rx. In this case the SRS resource mapping to the physical antennas will be changed and so does the SRS ILs as show in figure 1.

Figure 1 SRS antenna port mapping to Real Antennas may be changed due to for example hand blocking

Observation 2:   The mapping between SRS resource and physical antennas might change due to e.g. hand blocking.

Proposal 1:   	UE compensation issue and SRS IL changes due to hand blocking need to be considered if SRS IL reporting scheme is pursued in RAN4.

2.3 Static or dynamic reporting

For the two aspects discussed above, it can be seen that whether and when UE will do the SRS IL compensation is up to UE implementation. This makes NW have no idea whether UE has already done the compensation if only rely on the static reporting, and so does the SRS IL change situation. Only UE itself knows whether the reported SRS IL is applicable or not.

To make sure the gNB compensation is correct, the dynamic reporting should be considered.

Observation 3:   Dynamic reporting is necessary to make sure gNB knows whether it needs to do compensation or which value it should be used.

When it comes to the details of dynamic reporting, up to now there are different schemes or views.

To cope with the above UE compensation issue:

At least UE needs to indicate whether the SRS IL has been compensated inside and the compensation usually only happens when the Tx power from PA is not enough. That means in most of the time, the compensation is done by UE itself, and only when UE is in cell edge for example, gNB compensation will join.

After UE indicate gNB to do compensation, then some while later, UE Tx power itself is enough to do the compensation, then it needs to re-indicate gNB to not do the compensation.

To further cope with the hand blocking issue:

The hand blocking factor needs to be added, which means when the hand blocking happens in the region where UE self-compensation cannot help then it will report to the gNB about the different SRS ILs and mapping to the SRS antenna ports to be used. The new SRS IL values need to be reported to gNB.

After gNB receives these dynamic indications from UE, it can clearly know when to do the SRS IL compensation in channel estimation. These details need to be considered when discussing static or dynamic reporting.

Proposal 2:   	If SRS IL is to be reported, dynamic reporting shall be considered.

For the reporting schemes, one way might be based on gNB request, for example, when gNB is going to do channel estimation, it asks UE to report which SRS IL is applied.

Proposal 3:   	To simplify dynamic reporting complexity, gNB request based SRS IL reporting might be a way to go. For example, before gNB to do the SRS IL compensation in channel estimation, it asks UE to report which SRS IL is applied.

- Conclusions

In this contribution, SRS IL reporting issue is discussed and got the following observations and proposals.

Observation 1:   If UE support SRS IL reporting feature it will do the compensation, otherwise, there is no restriction.

Observation 2:   The mapping between SRS resource and physical antennas might change due to, e.g. hand blocking.

Proposal 1:   	UE compensation issue and SRS IL changes due to hand blocking need to be considered if SRS IL reporting scheme is pursued in RAN4.

Observation 3:   Dynamic reporting is necessary to make sure gNB knows whether it needs to do compensation or which value it should be used.

Proposal 2:   	If SRS IL is to be reported, dynamic reporting shall be considered.

Proposal 3:   	To simplify dynamic reporting complexity, gNB request based SRS IL reporting might be a way to go. For example, before gNB to do the SRS IL compensation in channel estimation, it asks UE to report which SRS IL is applied.

- References

- R4-2505101, WF on 6Rx RF requirements and SRS IL imbalance, Intel, #114bis, 2025-04