3GPP TSG-RAN WG4 Meeting #115														R4-2506928

Malta, MT, 19th – 23rd May, 2025

Agenda item:	8

Source: 	OPPO

Title: 	Rel-19 RAN4 UE feature list for NR

Document for:	Approval

- Introduction

This contribution includes the RAN4 UE feature list for Rel-19 NR.

- NR_ENDC_RF_Ph4

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (V2X WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 46. NR_ENDC_RF_Ph4 | 46-1 |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 46-2 | 6Rx for handheld and FWA | Indicates whether UE supports 6Layer, and this is only applied to FWA in Rel-19. |  | Yes | N/A | UE doesn’t support 6Layer in this band. | Per FS | No | FR1 only | N/A | RAN4 has sent LS R4-2505225 to RAN2, | Optional with capability signaling |
|  | [46-3] | [SRS IL imbalance reporting] | [Indicates whether UE supports SRS IL imbalance reporting.] |  | Yes | N/A | UE doesn’t support SRS IL imbalance reporting | Per band | No | FR1 only | N/A | RAN4 is still under discussion whether this feature is to be introduced in Rel-19. | Optional with capability signaling. |
|  | [46-4] | [MPR reduction for single CC] | [Indicates whether UE supports R19 MPR reduction .] |  | Yes | N/A | UE doesn’t support R19 MPR reduction | Per band | No | FR1 only | N/A |  | Optional with capability signaling |
|  | [46-5] | [MPR reduction for single CC] | [Indicates whether UE supports additional extended values for R19 MPR reduction .] |  | Yes | N/A | UE supports the default extended value for R19 MPR reduction | Per band | No | FR1 only | N/A | RAN4 is still under discussion whether this feature is to be introduced in Rel-19. | Optional with capability signaling |

- NonCol_intraB_ENDC_NR_CA_Ph2

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (V2X WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 47. NonCol_intraB_ENDC_NR_CA_Ph2 | 47-1 |  |  |  |  |  |  |  |  |  |  |  |  |

- FS_NR_FR1_DL_Frag_Carrier

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (V2X WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 48.FS_NR_FR1_DL_Frag_Carrier | 48-1 | [two fragmented carriers by sharing one RX RF chain] | [Indicates whether UE supports two fragmented carriers by sharing one RX RF chain.] |  | Yes | N/A | UE doesn’t support two fragmented carriers by sharing one RX RF chain | Per band Per BC | No | FR1 only | N/A |  | Optional with capability signaling |

- NR_FR1_DL_Frag_Carrier

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (V2X WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 49. FS_NR_FR1_DL_Frag_Carrier | 49-1 |  |  |  |  |  |  |  |  |  |  |  |  |

- NR_ATG_enh

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (V2X WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 50. NR_ATG_enh | 50-1 |  |  |  |  |  |  |  |  |  |  |  |  |

- NR_RRM_Ph5

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (V2X WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 51. NR_RRM_Ph5 | 51-1 | Rx beam reduction | Indicates the reduced Rx beam number [2,4,6] for beam sweeping factor |  | Yes | N/A | UE doesn’t support fast beam sweeping | Per band | No | FR2 only | N/A |  | Optional with capability signaling |
|  | 51-2 | CSSF enhancement solution 1: measure and report on one CC per band | Indicates whether UE supports to measure and report on one CC per band for CSSF enhancement |  | Yes | N/A | UE doesn’t support to measure and report on one CC per band | Per band | No | [FR2 only] | N/A |  | Optional with capability signaling |
|  | 51-3 | CSSF enhancement Solution 3: 3 searchers | Indicates whether UE supports 3 searchers for reducing CSSF |  | Yes | N/A | UE doesn’t support further CSSF enhancement based on 3 searchers | Per UE | No | [FR2 only] | N/A |  | Optional with capability signaling |

- NR_demod_Ph5-Perf

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (V2X WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 52. NR_demod_Ph5-Perf | 52-1 |  |  |  |  |  |  |  |  |  |  |  |  |

- NR_MIMO_Ph5

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (V2X WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 53. NR_MIMO_Ph5 | 53-1 | MIMO Layer for 3Tx | Indicate whether UE supports 3Layers in one band. |  | Yes | N/A | UE cannot be configured with 3Layers in one band. | Per FS | No | FR1 only | N/A |  | Optional with capability signaling. |

- Netw_Energy_NR_enh

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (V2X WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 54. Netw_Energy_NR_enh | 54-1 |  |  |  |  |  |  |  |  |  |  |  |  |

- NR_LPWUS

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (V2X WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 55.NR_LPWUS | 55-1 |  |  |  |  |  |  |  |  |  |  |  |  |

- NR_Mob_Ph4

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (V2X WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 56. NR_Mob_Ph4 | 56-1 |  |  |  |  |  |  |  |  |  |  |  |  |

- NR_XR_Ph3

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (V2X WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 57.NR_XR_Ph3 | 57-1 |  |  |  |  |  |  |  |  |  |  |  |  |

- NR_NTN_Ph3

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (V2X WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 58.NR_NTN_Ph3 | 58-1 |  |  |  |  |  |  |  |  |  |  |  |  |