3GPP TSG-WG4 Meeting #115	R4-2506933

# Malta, Malta, 19 – 23 May 2025

Source: 	NEC

Title: 	Discussion on OTA test scope reduction

Agenda Item:		7.13.4.2.1

Document for:	approval

1	Introduction

This contribution discusses the BS OTA test scope reduction, focusing on interference frequency offset for Tx intermodulation requirements and interference frequency step size for out-of-band blocking requirements, and provides proposals.

2	Discussion

In the previous meeting RAN4#114bis, BS OTA test scope reduction was discussed and a WF [1] was agreed.

However, in the WF, there are still options for interference frequency offset for Tx intermodulation conformance testing and for interference step size for out-of-band requirement conformance testing. We further discuss these issues.

2.1	Interferer frequency offset for Tx intermodulation

Agreements in the previous meeting [1] are blow.

  - Further discuss how to reduce the test simplification for OBUE limits.

    - Option 1: 1st frequency offset

    - Option 2: 3rd frequency offset to check OBUE limits

  - Further discuss how to reduce the test simplification for spurious limits.

    - Option 1: 3rd frequency offset

    - Option 2: For bands with operating band < 100 MHz to use 1st interferer offset and for bands with operating band >= 100MHz to use 3rd interferer offset.

OBUE

Frequency offset to check OBUE limits should be decided so that 3rd order intermodulation products cover the frequency range beyond 10 MHz from the wanted signal channel edge where the OBUE limit is most stringent. Both frequency offsets in option 1 and 2 meet this condition. Considering the interfering signal position based on 3rd order frequency offset has higher risk that it may be outside of the OBUE range and also considering the possibility of the simultaneous testing together with checking ACLR limits, option 1 looks more suitable to check OBUE limits. We support option 1, that is, to adopt 1st order frequency offset to check OBUE limits.

Proposal 1:

To adopt 1st order frequency offset to check OBUE limits.

Spurious emissions

For spurious emissions requirements are applied to frequency range beyond ΔfOBUE of the operating band edge, interferer frequency offset should be chosen to make the significant intermodulation products fall in this region. It is clear 3rd order frequency offset provides higher probability to make the intermodulation products fall in the spurious region. As we do not see any reason that 1st order frequency offset causes severer intermodulation products in spurious region, we support option 1, that is, to adopt 3rd order frequency offset to check spurious emission limits.

Proposal 2:

To adopt 3rd order frequency offset to check spurious emissions limits.

2.2	Interferer frequency step size for out-of-band blocking

Agreements in the previous meeting are blow [1].

  - Agreement on step size of OOBB requirement for further discussion:

    - Option 1: 1/3 of the minimum supported bandwidth

    - Option 2: Option 2: Use 1% of the momentary frequency as the step size for OTA receiver out-of-band blocking testing for frequencies up to 6000 MHz. Use a fixed step size of 60 MHz for frequencies higher than 6000 MHz

    - Other options are not precluded.

To decide the interferer frequency step size for out-of-band blocking, we should consider the principle design for Rel-15 FR2 BS OOBB CW step size in TR 38.817-02 [2] shown below.

The base station out-of-band blocking requirement is tested with a CW interferer and the wanted uplink signal. A spurious response occurs when the m-th harmonic of the interferer frequency mixes with the n-th harmonic of an internal signal, like a clock or a local oscillator signal, and this mixing product falls within the receive bandwidth of the wanted signal (or near the receive bandwidth, if the receiver selectivity is insufficient to suppress the spurious mixing product). The frequency of the unwanted mixing product equals ±m·fCW±n·finternal, where fCW is the interferer frequency and finternal is the frequency of the internal signal involved in the mixing product. m can assume the values 1, 2, 3, … and n can assume the values 0, 1, 2, …. Spurious responses for which the harmonic number m of the interferer signal equals 4 or more are usually not significant. Hence, the frequency step size should be chosen such that any spurious response due to a maximum third harmonic of the interferer will be captured within the receive bandwidth of the wanted signal.

We have reviewed the options in the WF [1].

Option 2 proposes the frequency step size of up to 60 MHz. It may result in any spurious responses due to a 3rd harmonic of the interferer signal may not be captured within the receive bandwidth of the wanted signal when the frequency step size is large. We do not think we can assume passing the tests with this option guarantees passing the test with other interference frequencies. On the other hand, when the momentary frequency is low, interference step size could be as small as 0.3 MHz. It is smaller than the step size in the current specification. It means it needs more conformance testing time to test certain interferer frequency range. We are not happy with it. We also think using 1% of the momentary frequency as the step size makes the conformance test setting complex.

Option 1 proposes the frequency step size of 1/3 of the minimum supported bandwidth. It secures the spurious response due to a 3rd harmonic of the interferer signal captured within the receive bandwidth. Therefore, we support this option.

Proposal 3:

To adopt 1/3 of the minimum supported bandwidth for out-of-band blocking interferer frequency step size.

Conclusions

We discussed the issues of the NCR ACLR requirement for the simultaneous NCR-Fwd and NCR-MT transmission and provided the following proposal.

We discusses the BS OTA test scope reduction, focusing on interference frequency offset for Tx intermodulation requirements and interference frequency step size for out-of-band blocking requirements, and provide following proposals.

Proposal 1:

To adopt 1st order frequency offset to check OBUE limits.

Proposal 2:

To adopt 3rd order frequency offset to check spurious emissions limits.

Proposal 3:

To adopt 1/3 of the minimum supported bandwidth for out-of-band blocking interferer frequency step size.

Reference

[1] R4-2504740, “Way Forward for [114bis][304] NR_BS_RF_Part3_OTA_TRP”, ZTE

[2] TR 38.817-02