# 3GPP TSG-RAN WG4 Meeting #115                                   R4-2506950

# Malta, MT, 19th – 23rd May, 2025

# Title: 	Discussion on NTN_testing_NGSO_channel_model

# Source: 	ZTE Corporation, Sanechips

# Agenda item:	7.10.5.1

# Document for:	Approval

# 1 Introduction

In RAN #104 meeting, the revised WID of Enhanced requirements and conductive test methodology for NR NTN and IoT NTN was approved in [1], wherein the objectives related to channel model and demodulation requirements are copied below:

NTN testing for NGSO - Specify the TE-emulated channel model with varying Doppler and delay shifts for NR-NTN and IoT-NTN in the FR1-NTN bands and the corresponding LTE bands - Match the satellite motion trajectory based on the ephemeris for NGSO (Non-Geostationary Orbit) scenarios - Inform RAN5 to assist specifying RF frequency error tests, if needed - Checking if the existing frequency error requirement, can be met under the TE-emulated channel model without intention of modifying the existing core requirement - Specify UE RRM performance test of uplink timing for NGSO for NR-NTN and IoT-NTN in the FR1-NTN bands and the corresponding LTE bands based on the above channel model - Checking if the existing RRM uplink timing requirements, can be met under the TE-emulated channel model without intention of modifying the existing core requirement - Specify UE demodulation performance requirement(s) of PDSCH for NGSO for NR-NTN and IoT-NTN in the FR1-NTN bands and the corresponding LTE bands based on the above channel model - Minimize the number of demodulation performance requirements - Specify the applicability of the tests under the TE-emulated channel model Less than 5MHz for NTN - Specify the necessary conformance requirements for SAN for less than 5 MHz for NR-NTN in FR1-NTN bands - Specify the necessary RRM performance requirements and test cases for less than 5 MHz for NR-NTN in FR1-NTN bands - Specify the necessary demodulation performance requirements for less than 5 MHz for NR-NTN in FR1-NTN bands

In this contribution, we provide some of our view point on the NGSO channel model.

- Discussion

Impact on RAN4 demodulation requirements with TE-emulated time vary doppler shift and propagation delay models

- Proposals: - Option 1: - Keep the same NR NTN PDSCH requirements (SNR) even if the time-varying Doppler shift and propagation delay model is applied. - Keep the same IoT NTN UE demodulation requirements (SNR) even if the time-varying Doppler shift and propagation delay model is applied. - Option 2: Considering worst case for frequency error and timing error as 0.1 ppm = 200Hz@2GHz and 29*64*Tc = 9.44e-7s to check whether existing NR NTN and IoT-NTN UE demodulation requirements can apply. Based on simulation result, further discussion whether additional margin to consider the estimation/pre-compensation errors with the TE-emulated varying Doppler shift and propagation delay models - Option 3: - RAN4 shall start to agree parameters for using the NGSO channel model in order to understand whether the margin produced is acceptable to define requirements with. - RAN4 shall use the new NGSO channel model for all performance requirements, in it’s current state. - Option 4: After concluding discussions on the time-varying Doppler shift and propagation delay, initiate simulation campaigns to analyse the impact on demodulation requirements.

In the previous discussion at RAN4, it was agreed that both the TE and UE side would calculate the time delay and Doppler based on the model according to ephemeris information . The TE vendor needs to add the correct time delay and Doppler to the test environment during the test, and the UE vendor needs to estimate the time delay and Doppler for pre-compensation before the baseband. Compared to legacy requirements, RAN4 needs to consider residual Doppler and time delay after UE performs pre-compensation. Therefore, from the perspective of basedband demodulation, RAN4 should re-evaluate the legacy requirements under residual frequency offset and time delay conditions in light of the impact of time delay and Doppler effects.

Observation 1. TE vendor needs to add the correct time delay and Doppler to the test environment during the test, and the UE vendor needs to estimate the time delay and Doppler for pre-compensation before the baseband.

Proposal 1. Propose to consider residual frequency offset and time delay to re-evaluate the legacy requirements.

Test cases for NR NTN with applying time varying Doppler Shift and propagation delay model

- agreement - Apply the time-varying Doppler shift and propagation delay model with LEO-600 30 degrees elevation angle condition to TS 38.101-5 Clause 8.2.1.2.2.1 Test 1-1 and Test 1-2. - FFS on test 1-3 and 1-4 - Note: test 1-3 is related with UE capability to support 32 HARQ process, and test 1-4 is related with UE capability with HARQ disable

In legacy requirements, test 1-1 and test 1-2 are related to supporting 16 HARQ process, a mandatory feature. Test 1-3 is related with U capability to support 32 HARQ process, and test 1-4 is related with UE capability with HARQ disable, both are optional features. Also in WID[1], it is clearly stated that the demodulation performance requirements should be minimized. Therefore, we propose to only consider test cases 1-1 and 1-2 for NR NTN with applying time varying Doppler Shift and propagation delay model.

Proposal 2. Propose to only consider test cases 1-1 and 1-2 for NR NTN with applying time varying Doppler Shift and propagation delay model.

# 3 Conclusion

In this contribution, we give some discussions on NTN_testing_NGSO_channel_model, The conclusions are:

Observation 1. TE vendor needs to add the correct time delay and Doppler to the test environment during the test, and the UE vendor needs to estimate the time delay and Doppler for pre-compensation before the baseband.

Proposal 1. Propose to consider residual frequency offset and time delay to re-evaluate the legacy requirement.

Proposal 2. Propose to only consider test cases 1-1 and 1-2 for NR NTN with applying time varying Doppler Shift and propagation delay model.

# 4	Reference

- RP-241281, Revised WID on Enhanced requirements and conductive test methodology for NR NTN and IoT NTN.