# 3GPP TSG-RAN WG4 Meeting #115                                  R4-2506951

# Malta, MT, 19th – 23rd May, 2025

# Title: 	Discussion on Rel-19 ATG demodulation requirements

# Source: 	ZTE Corporation, Sanechips

# Agenda item:	7.12.7

# Document for:	Approval

# 1 Introduction

In RAN #106 meeting, the revised WID of enhancements for Air-to-ground network for NR was approved in [1], the objectives related to enhanced ATG are listed :

The performance part of the work item includes: - Specify corresponding RRM performance and demodulation requirements for intra-band co-located and inter-band co-located DL CA. - Specify corresponding demodulation requirements for support of UL MIMO with 2TX for single CC for UE with dual-polarization antenna. - Specify demodulation requirements for support of DL MIMO for UE with limited test cases. - Specify conformance testing and conformance requirements for BS.

In Rel-18, RAN4 defined demodulation requirements for PDSCH under AWGN channel model on FR1 single scenario. However, considering that ATG has the advantage of high throughput, low propagation delay, and low cost application, some operators and the aircraft industry have a strong request for the deployment of ATG. In R19, the following two aspects are considered:

- Enhance throughput to meet ATG capacity demand

- Fully utilize operators’ spectrum especially for the fragment spectrum

In this contribution, we give some initial discussion on the enhanced ATG demodulation requirements.

- 2 Discussion

- DL CA requirements

- CA Duplex mode

Agreement: - TDD 30 kHz + TDD 30 kHz - FDD 15 kHz + TDD 30 kHz FFS: - FDD 15 kHz + FDD 15 kHz

During the last meeting, RAN4 discussed the CA duplex mode and reached the above agreement. According to the WID[1], it is clearly stated that the requirements for defining DL CA requirements, including intra-band contiguous CA and inter-band CA, e.g. CA_n79C and CA-n3A-n39A. From RAN4 legacy banseband demodulation requirements, RAN4 shall define TDD 30kHz + TDD 30kHz, FDD 15 kHz + TDD 30 kHz CA duplex models for R-19 enhanced ATG DL CA requirements.

- Proposal 1. Propose not to define FDD 15 kHz + FDD 15 kHz for R-19 enhanced ATG DL CA requirements.

TDD pattern

Agreement: - Use legacy TDD pattern 7DS2U as the staring point FFS: - new TDD pattern 30D4S6U

Currently, legacy CA demodulation requirements also follows single CC requirement. From this point of view, we think ATG feature shall follow this rule.

- Proposal 2. Propose to define new TDD pattern 30D4S6U for R-19 enhanced ATG DL CA requirements.

Rank

- Proposal 1: Rank 2 - Proposal 2: Rank 1

- In legacy CA demodulation requirements, RAN4 only reuses the test parameters from single CC requirements to define CA requirements. Hence, we propose to only consider Rank 1 for DL CA demodulation requirements.

- Proposal 3. Propose to only consider Rank 1 for DL CA demodulation requirements.

DL MIMO requirements

Antenna configuration

Agreement: - 2T2R and 2T4R - FFS the antenna correlation: - Proposal 1: XP-low

From the WID[1], it is clearly pointed that the dual-polarization antenna is used for UL MIMO with 2Tx transmission. Therefore, for DL MIMO transmission, dual-polarization antenna also needs to be considered for receiving antenna.

- Proposal 4. Propose to consider XP-low for DL MIMO transmission.

- MCS

Agreement: - For initial simulation, consider 16QAM Table 1 MCS13, 64QAM Table 1 MCS 22 and 256QAM Table 2 MCS 24

- We evaluated 16QAM Table 1 MCS13, 64QAM Table 1 MCS 22 and 256QAM Table 2 MCS 24 with 2T2R, the SNR point of 70% maximum throughput and curves can be found in the following. From our simulation results, MCS 13 and MCS 22 can be tested in the reasonable SNR point, and for 256QAM with MCS 24, the SNR point is about 22.76dB, considering additional margin, the SNR could be a little higher. Therefore, we prefer to consider MCS 13 and MCS 22 to define requirements.

- Table 1. Simulation results for 2T2R Rank 2

| Propogation condition | Antenna configuration | MCS | Layer | SNR(70% Max TP) |
| --- | --- | --- | --- | --- |
| AWGN+220Hz | 2T2R | 16QAM Table 1 MCS13 | Rank 2 | 7.41 |
|  |  | 64QAM Table 1 MCS 22 | Rank 2 | 14.87 |
|  |  | 256QAM Table 2 MCS 24 | Rank 2 | 22.76 |

- Table 2.  Throughput curves for 2T2R Rank 2

| - Fig 1. FDD, MCS13, Rank 1 and Rank 2 | - Fig 2. FDD, MCS22, Rank 1 and Rank 2 |
| --- | --- |
| - Fig 3. FDD, MCS24, Rank 1 and Rank 2 |  |

- Observation 1. From our simulation results, MCS 13 and MCS 22 can be tested in the reasonable SNR point, and for 256QAM with MCS 24, the SNR point about 22.76dB, considering additional margin, the SNR could be a little higher.

- Proposal 5. Propose to consider MCS 13 and MCS 22 defining demodulation requirements.

UL MIMO requirements

Whether to reuse existing requirements for ATG UL MIMO

Agreement: - The legacy PUSCH test cases in Clause 8.2.1with 2Tx 2 layer will be applied for ATG - FFS: - Whether other legacy test cases are needed.

Regarding UL demodulation requirements, RAN4 has a common understanding that all BS types shall pass the Rel-15 feature demodulation requirements during the conformance testing. Considering the Rel-19 ATG feature, which is only focus on UL MIMO with 2Tx, the legacy requirements with 2Tx shall be selected. And for other legacy test cases, we do not think it is necessary.

Proposal 6. No need to consider other legacy test cases.

Issue 3-1-2: Whether to introduce new test cases for ATG UL MIMO

- Proposals - Proposal 1: - incremental PUSCH 2 layer test cases need to be introduced - Proposal 2: - Do not introduce new 2Tx PUSCH requirements for ATG scenario

As discussed in the above, ATG BS shall pass the legacy test cases at first during the conformance testing. However, due to the frequency offset caused by aircraft movement, the baseband performance needs to be verified even though RAN4 only assumes 0.1ppm frequency offset after pre-compensation. From this point of view, we prefer to introduce incremental PUSCH 2 layers test cases.

Proposal 7. Propose to introduce incremental PUSCH 2 layer test cases.

# 3 Conclusion

In this contribution, we give some discussions on demodulation performance requirements of ATG UE demodulation requirements, The conclusions are:

- Observation 1. From our simulation results, MCS 13 and MCS 22 can be tested in the reasonable SNR point, and for 256QAM with MCS 24, the SNR point about 22.76dB, considering additional margin, the SNR could be a little higher.

- Proposal 1. Propose not to define FDD 15 kHz + FDD 15 kHz for R-19 enhanced ATG DL CA requirements.

- Proposal 2. Propose to define new TDD pattern 30D4S6U for R-19 enhanced ATG DL CA requirements.

- Proposal 3. Propose to only consider Rank 1 for DL CA demodulation requirements.

- Proposal 4. Propose to consider XP-low for DL MIMO transmission.

- Proposal 5. Propose to consider MCS 13 and MCS 22 defining demodulation requirements.

Proposal 6. No need to consider other legacy test cases.

Proposal 7. Propose to introduce incremental PUSCH 2 layer test cases.

# 4	Reference

- RP-243285, Revised WID on Enhancements for Air-to-ground network for NR. CMCC, ZTE Corporation, CATT, Huawei, HiSilicon, Verizon, China Telecom, China Unicom.