# 3GPP TSG-RAN WG4 Meeting # 115                                 R4-2506953

# Malta, MT, 19th – 23rd May, 2025

# Title: 	Discussion on SAN demodulation performance requirements for IoT-NTN

# Source: 	ZTE Corporation, Sanechips

# Agenda item:	7.32.6

# Document for:	Approval

# 1 Introduction

In RAN #105 meeting, the revised WID of Non-Terrestrial Networks (NTN) for Internet of Things (IoT) Phase 3 was approved in [1], wherein the objectives related to IoT-NTN are listed as below:

- Support of Capacity enhancements for uplink - Study then specify, if beneficial, enhancements to enable multiplexing of multiple UEs (e.g. up to the min of 4 and the maximum allowed by the existing UL and DL signalling) in a single 3.75 kHz or 15 kHz subcarrier via orthogonal cover codes (OCC) for NPUSCH format 1 and NPRACH [RAN1, RAN2, RAN4] - Multi-tone support for 15 kHz SCS should also be considered - Specify necessary signalling, if needed - Update RF requirements accordingly, if needed Note: Impact of impairment shall be taken into account - Study and specify, if beneficial the following enhancements to reduce the necessary uplink and downlink signaling to complete an Early Data Transmission (EDT) transaction [RAN2]: - Msg3 transmission without msg1/ Random Access Response (RAR) - Efficient delivery (reduced overhead) of msg4 / RRCEarlyDataComplete - Study and specify RRM requirement, if identified [RAN4]

In the following section, we will provide the detailed discussions for IoT NTN SAN demodulation requirements.

# 2. Proposed work plan

According to the approved WID, the Perf part of this WI targets Sep 2025, which means RAN4 Demod part only has 2 meetings. Considering the above, the following work plan is proposed:

- RAN4#115:

  - Agree on the test scope, test parameters.

- RAN4#116:

  - Collection of simulation results.

  - Draft CR endorsed, and big CR agreed.

Proposal 1: As the WI Perf. Part targets Sep 2025, the following work plan is proposed:

- RAN4#115:

  - Agree on the test scope, test parameters.

- RAN4#116:

  - Collection of simulation results.

  - Draft CR endorsed, and big CR agreed.

# 3 Discussion

IoT-NTN has been specified in 3GPP RAN Rel-17, since then been optimised in Rel-18 and commercial deployment are ongoing at the moment. Based on real deployment or deployment plans further evolution of IoT-NTN is needed.  Currently, NB-IoT NTN is already being deployed live at this very moment. In these early and upcoming deployments, it is clearly emerging that IoT-NTN, in particular NB-IoT, will have to support massive capacity in terms of number and types of UE, some of which with worse characteristics than others (e.g. low cost devices, wearables, etc). Multiplexing of UEs by usage of orthogonal cover codes (OCC) for NPUSCH format 1 and NPRACH should therefore be studied and if beneficial being specified.

NPUSCH format 1 demodulation requirements

Currently, NPUSCH format 1 supports single tone with 3.75kHz and 15kHz SCS, and multiple tones only with 15kHz SCS. In WID[1], it is clearly stated that a single 3.75kHz or 15kHz subcarrier shall be considered. And multi-tone with15kHz should also be considered. However, from RAN1’s conclusion, multi-tone with 15kHz is low priority. Therefore, from our understanding, RAN4 shall discuss whether need to define multi-tone demodulation requirements at first.

Observation 1. Multi-tone with 15kHz is low priority based on RAN1’s conclusion.

Propose 2. Propose to define only single tone with both 3.75kHz and 15kHz SCS demodulation requirements.

As illustrated in WID[1], in order to improve coverage enhancement, OCC scheme is allowed to multiplex multiple UEs. Code division multiplexing assigns unique orthogonal codes to different users or data streams so that they are transmitted on the same frequency band, but the receiver can separate their respective signals through correlation operations, etc. From the perspective of baseband reception, it is necessary to first perform channel estimation for each user through DMRS, and than perform decoding and other steps. Therefore, the DMRS design of division multiplexing is crucial for multi-users multiplexing scenarios. In addition, based on the study of RAN1, currently, two potential options are considered, e.g., TDM of DMRS and CDM of DMRS. For TDM of DMRS, which means that the time domain of locations of DMRS for different UEs are different. No OCC is applied for the DMRS of different UEs. And the CDM of DMRS, the time domain locations of DMRS for different UEs are the same. Different OCCs are applied for the DMRS of different UEs. Currently, RAN1 agreed to use CDM of DMRS to single tone with 15kHz, and TDM of DMRS to single tone with 3.75kHz. Therefore, we propose to reuse RAN1’s conclusion to define demodulation requirements.

Propose 3. Propose to consider CDM of DMRS to single tone with 15kHz and TDM of DMRS to single tone with 3.75kHz if necessary.

Further, the legacy demodulation requirements only defined with limited test cases, e.g., 1R and 2R, NTN TDLA100-1 and NTN TDLC5-1.Therefore, we propose to reuse same same antenna configuration and propagation conditions to define R-19 demodulation requirements.

Propose 4. Propose to reuse legacy antenna configuration (e.g., 1T1R and 1T2R) and propagation conditions (e.g., NTN TDLA100-1 and NTN TDLC5-1).

NPRACH demodulation requirement

Considering that RAN1 has following conclusion in previous meeting, we propose to no need to consider NPRACH with OCC test case in Rel-19.

Propose 5. Propose to do not define NPRACH with OCC demodulation requirements.

# 3 Conclusion

In this contribution, we give some discussions on the MIMO layers related to 6Rx, The conclusions are:

Observation 1. Multi-tone with 15khz is low priority based on RAN1’s conclusion.

Proposal 1: As the WI Perf. Part targets Sep 2025, the following work plan is proposed:

- RAN4#115:

  - Agree on the test scope, test parameters.

- RAN4#116:

  - Collection of simulation results.

  - Draft CR endorsed, and big CR agreed.

Propose 2. Propose to define only single tone with both 3.75kHz and 15kHz SCS demodulation requirements.

Propose 3. Propose to consider CDM of DMRS to single tone with 15kHz and TDM of DMRS to single tone with 3.75kHz if necessary.

Propose 4. Propose to reuse legacy antenna configuration (e.g., 1T1R and 1T2R) and propagation conditions (e.g., NTN TDLA100-1 and NTN TDLC5-1).

Propose 5. Propose to do not define NPRACH with OCC demodulation requirements.

# 4 References

- RP-242397, Revised WID on Non-Terrestrial Networks (NTN) for Internet of Things (IoT) Phase 3.