# 3GPP TSG-RAN WG4 Meeting #115                                   R4-2506954

# Malta, MT, 19th – 23rd May, 2025

# Title: 	Discussion on UE and BS demodulation performance requirements for SBFD

# Source: 	ZTE Corporation, Sanechips

# Agenda item:	7.25.6

# Document for:	Approval

# 1 Introduction

In RAN#102, a new R19 WI on Evolution of NR duplex operation: Sub-band full duplex (SBFD) was approved, and the revised WID at RAN#104 can be found in [1].

The objectives related to RAN4 core and performance parts are listed :

- Specify enhancements for CLI handling [RAN1, RAN2, RAN3, RAN4] - Information exchange among gNBs, including [RAN3] - Semi-static cell-specific SBFD time and frequency location configuration - Measurement resource configuration, i.e., SSB and/or periodic NZP CSI-RS - Strongest DL beam information - CLI-mitigation request - UL resource muting for PUSCH, including [RAN1, RAN2, RAN4] - Indication/determination of UL resource muting for PUSCH based on semi-static configuration, assuming comb-2 for both DFT-S-OFDM and CP-OFDM in each allocated PRB and up to 2 symbols in time domain - PUSCH resource mapping, i.e., rate-matching around the muted REs - UCI resource determination in symbols with muted REs - L1 based UE-to-UE CLI measurement and reporting based on existing CSI framework, including [RAN1, RAN2, RAN4] (Reuse major principles) - Periodic, semi-persistent, or aperiodic measurement resource (set), i.e., SRS-RSRP resource or CLI-RSSI resource, and configuration/determination of ‘typeD’ QCL assumptions for the CLI measurement resource - At least aperiodic reporting - New report quantities, e.g., L1-SRS-RSRP, L1-CLI-RSSI and/or measurement resource indices - UCI bits generation - Priority rules for multiple CSI reporting - CLI measurement accuracy requirement - Specify applicable RRM core requirements for CLI handling mechanisms [RAN4] - Specify other RRM core requirements for SBFD operation, if identified [RAN4] The objectives are as follows [RAN4]: - Specify BS RF conformance requirements to support SBFD operation at gNB - Specify necessary RRM performance requirements to support SBFD operation - Specify necessary BS and/or UE demodulation performance requirements to support SBFD operation

In this contribution, we give some initial discussion on BS and/or UE demodulation performance requirements to support SBFD operation.

- 2 Discussion

- Compared with existing half-duplex system, the base station of the simultaneous co-frequency full-duplex system can simultaneously transmit and receive co-frequency signals, which can maximize the use of time-frequency domain resources and improve the system transmission capacity. In half-duplex, there is a certain protection interval between the uplink and downlink, while in full-duplex, the uplink and downlink overlap in the time-frequency domain. When the signals overlap in the time-frequency domain, the bass station receiving antenna is subject to the huge power self-interference from the base station transmitting antenna. Therefore, the key point of full-duplex technology is to effectively suppress and eliminate strong self-interference.

- As previously studied in Rel-18, the interference that needs to be considered in a full-duplex system are mainly self-interference and cross-link interference (CLI). The gNB in the SBFD is a full-duplex model, and the UE is a half- duplex model. In full-duplex with non-overlapping sub-bands, the non-ideality of the BS’s transmit and receive filters will cause out of band leakage, and the BS will generate self-interference. For a single BS deployment scenario, it is necessary to consider the BS self-interference and  the CLI of the interfering UE to the interfered UE. For a multi-BS networking deployment scenario, considering the BS self-interference, the interference of the neighbouring BS’s downlink to the uplink of this BS(inter-BS interference), the interference between UEs, and the interference of the neighbouring cell BS to the UE. In general, the interference types are illustrated in the following Fig.1.

- UE-gNB co-channel interference

- gNB-UE co-channel interference

- gNB-gNB co-channel CLI

- UE -UE co-channel CLI

- gNB self interference

Fig.1. Interference types for SBFD

BS side interference handling

At present, antenna domain, analog circuit domain and digital domain self-interference elimination are the main self-interference elimination technologies. Antenna partial self-interference elimination can be achieved by isolating the transmit and receiver antennas, antenna cross polarization to increase antenna loss. The circulator can be used in the analog circuit domain to complete partial self-interference elimination through analog circuit design. After the first two steps, the self-interference is reconstructed in the digital domain through parameter estimation, and the reconstructed self-interference is subtracted from the received signal to achieve the purpose of eliminating self-interference as much as possible.

And regarding the CLI, beamforming nulling, advanced receiver and interference management are the main CLI elimination technologies. To suppress the DL signal, DL beamforming considering the interference to other BS could be used, by measuring the channel between the transmitter and the receiver, the DL beamforming weights can be adjusted to avoid transmitting in the direction of the receiver. The transmit beamforming nulling at the aggressor gNB will be directed toward the victim gNB, thereby reducing the power received at the victim gNB. From baseband precessing point of view, a potential solution to suppress the interference is using advanced receiver, e.g., MMSE-IRC and ML receiver. As we all known, the performance of advanced receiver is critically based on the interference covariance matrix or interference channel estimation. How to achieve the accurate channel estimation or covariance matrix requires further consideration.

- UL resource muting for PUSCH, including [RAN1, RAN2, RAN4] - Indication/determination of UL resource muting for PUSCH based on semi-static configuration, assuming comb-2 for both DFT-S-OFDM and CP-OFDM in each allocated PRB and up to 2 symbols in time domain - PUSCH resource mapping, i.e., rate-matching around the muted REs - UCI resource determination in symbols with muted REs

In the WID[1], it is pointed that UL resource muting for PUSCH to support CLI handling, which including the gNB-gNB interference UE-gNB interference suppression. Since the UE-gNB CLI is less than gNB-gNB CLI. Therefore, we can primarily discuss the gNB-gNB CLI.

Fig 1. gNB-gNB CLI

UE side interference handling

The interference on the UE side is mainly CLI, which can be measured by reference signals and reported, e.g., L1 and L3 measurement. Another potential solution to suppress the interference is also based on the advanced receiver, which MMSE-IRC receiver is a mandatory feature in the UE side.

Observation 1. The key point of SBFD technology is to effectively suppress and eliminate strong self-interference.

Observation 2. Antenna domain, analog circuit domain and digital domain self-interference elimination are the main self-interference elimination technologies.

Observation 3. Beamforming nulling, advanced receiver and interference management are the main CLI elimination technologies.

Observation 4. The interference on the UE side is mainly CLI, which can be measured by reference signals and reported, e.g., L1 and L3 measurement.

Observation 5. gNB-gNB CLI can be suppressed by advanced receiver.

Proposal 1. RAN4 needs to discuss whether to consider interference suppression scenarios ( gNB-gNB CLI) for SBFD operation in demodulation perspective.

In the above, we discussed the interference cancellation for SBFD from the perspective of baseband. Next, we will discuss the normal demodulation performance requirements for SBFD. Currently, the gNB in the SBFD is a full-duplex model, and the UE is a half- duplex model. From a deployment perspective, gNB first needs to support SBFD, which is a new type of gNB compared to traditional gNBs and requires good interference cancellation capabilities. Therefore, RAN4 shall define a new demodulation requirements for SBFD, especially on the gNB side.

Proposal 2. Propose to define BS demodulation performance requirements for SBFD operation .

# 3 Conclusion

In this contribution, we give some discussions on demodulation performance requirements of SBFD demodulation requirements, The conclusions are:

Observation 1. The key point of SBFD technology is to effectively suppress and eliminate strong self-interference.

Observation 2. Antenna domain, analog circuit domain and digital domain self-interference elimination are the main self-interference elimination technologies.

Observation 3. Beamforming nulling, advanced receiver and interference management are the main CLI elimination technologies.

Observation 4. The interference on the UE side is mainly CLI, which can be measured by reference signals and reported, e.g., L1 and L3 measurement.

Observation 5. gNB-gNB CLI can be suppressed by advanced receiver.

Proposal 1. RAN4 needs to discuss whether to consider interference suppression scenarios ( gNB-gNB CLI) for SBFD operation in demodulation perspective.

Proposal 2. Propose to define BS demodulation performance requirements for SBFD operation .

# 4	Reference

- RP-241614, Revised WID: Evolution of NR duplex operation: Sub-band full duplex (SBFD), Huawei