# 3GPP TSG-RAN WG4 Meeting # 115                                 R4-2506956

# Malta, MT, 19th – 23rd May, 2025

# Title: 	Discussion on UE demodulation performance requirements for LP-WUS

# Source: 	ZTE Corporation, Sanechips

# Agenda item:	7.28.7

# Document for:	Approval

# 1 Introduction

In RAN #105 meeting, the revised WID of low-power wake-up signal and receiver for NR (LP-WUS/WUR) was approved in [1], wherein the objectives related to LP-WUS are listed as below:

The objectives of the work item are the following: - To specify an LP-WUS design commonly applicable to both IDLE/INACTIVE and CONNECTED modes (RAN1, RAN4) - Specify OOK (OOK-1 and/or OOK-4) based LP-WUS with overlaid OFDM sequence(s) over OOK symbol - The LP-WUS design shall ensure that for IDLE/INACTIVE operation, the same information is delivered irrespective of LP-WUR type. The OFDM sequence can carry information. - At least duty-cycled monitoring of LP-WUS is supported - For IDLE/INACTIVE modes - Specify procedure and configuration of LP-WUS indicating paging monitoring triggered by LP-WUS, including at least configuration, sub-grouping and entry/exit condition for LP-WUS monitoring (RAN2, RAN1, RAN3, RAN4) - Specify LP-SS with periodicity with Yms for LP-WUR, for synchronization and/or RRM for serving cell. (RAN1, RAN4) - LP-SS is based on OOK-1 and/or OOK-4 waveform with or without overlaid OFDM sequences. Further down selection between with and without overlaid OFDM sequences is to be done within WI. - Note: For LP-WUR that can receive existing PSS/SSS, existing PSS/SSS can be used for synchronization and RRM instead of LP-SS. - Y will be decided within WI. 320ms is the start point. - Specify further RRM relaxation of UE MR for both serving and neighbor cell measurements, and UE serving cell RRM measurement offloaded from MR to LP-WUR, including the necessary conditions (RAN4, RAN2)

In the following section, we will provide the detailed discussions for LU-WUS demodulation performance requirements.

# 2 Discussion

LP-WUS (Low Power Wake-up Signal) is an energy-saving technology designed for the Internet of Things (IoT) and Low Power Wide Area Networks (LPWAN, such as NB-IoT). It is used to efficiently wake up terminal devices in idle state (IDLE/RRC_IDLE), thereby significantly reducing standby power consumption and extending battery life (up to several years).

- The principle is as follows: the base station first sends an extremely low-power wake-up signal (LP-WUS), which only carries the terminal ID or group ID information.

- The terminal activates the receiving chain and monitors the subsequent data channel only after detecting the matching LP-WUS.

- If LP-WUS is not detected, the terminal remains in deep sleep to avoid invalid monitoring.

The low-power wake-up signal and receiver were studied in Rel-18 to save UE power, it’s observed that significant UE power saving gain (up to more than 90%) is obtained by using LP-WUS/WUR to trigger UE MR paging monitoring compared with existing I-DRX operation (with and without PEI) in RRC IDLE/INACTIVE modes. Also it’s observed that moderate UE power saving gain (up to more than 10%) is obtained with marginal impact to capacity by using LP-WUS/WUR to trigger UE MR PDCCH monitoring compared with existing UE power saving techniques, across different types of XR traffic and system load scenarios.

In WID[1], it is clearly pointed that specify OOK(OOK-1 and/or OOK-4) based LP-WUS with overlaid OFDM sequence(s) over OOK symbol. From the perspective of signal generation, OOK-1 and OOK-4 are different waveforms, OOK-4 requires DFT compared to OOK-1 after signal generation and modulation.

On the other side, the main principle of OOK-based receiver is to perform envelop detection on the received signal, determine OOK-ON/OFF form the high and low levels of the envelop, and then obtain the bit information of the original transmitted signal.

The OFDM based receiver is based on the sequence correlation detection, which determines OOK-ON/OFF from the high and low correlations on different OOK chips, and then obtains the bit information of the original transmitted signal.

Observation 1. Two different types of waves are considered for LU-WUS, OOK-1 and OOK-4.

Observation 2. Two different types of receives are considered for LU-WUS receiver, OOK based receiver and OFDM based receiver.

Proposal 1. Propose to consider both OOK-1 and OOK-4 waveforms to define demodulation requirements.

Proposal 2. Propose to consider both OOK-based LU-WUS receiver and OFDM-based LU-WUS receiver to define demodulation requirements.

Regarding test metric, considering that the LP-WUS signal is designed to save power and reduce power consumption, the FAR means that the terminal wakes up unnecessarily , increasing power consumption (for example, the receiving chain is triggered by mistake when it should be asleep). MDR means the probability that the terminal fails to correctly detect a valid wake up signal (actually there is a signal), and it is necessary to increase uplink retransmission or delay, e.g., the based station needs to send LP-WUS repeatedly.

Proposal 3.  Propose to consider both 1% FAR and 1% MDR as test metrics.

# 3 Conclusion

In this contribution, we give some discussions on the LP-WUS demodulation requirements, The conclusions are:

Observation 1. Two different types of waves are considered for LU-WUS, OOK-1 and OOK-4.

Observation 2. Two different types of receives are considered for LU-WUS receiver, OOK based receiver and OFDM based receiver.

Proposal 1. Propose to consider both OOK-1 and OOK-4 waveforms to define demodulation requirements.

Proposal 2. Propose to consider both OOK-based LU-WUS receiver and OFDM-based LU-WUS receiver to define demodulation requirements.

Proposal 3.  Propose to consider both 1% FAR and 1% MDR as test metrics.

# 4 References

- RP-250780 New WID: UE RF enhancements for NR FR1/FR2 and EN-DC, Phase 4. Huawei, HiSilicon.

- R4-2503014, WF on 6Rx requirements, AT&T.