3GPP TSG-RAN WG4 Meeting #115	R4-2506968

# Malta, MT, 19th – 23rd May, 2025

Title:	Discussion on FR2 CA MPR related feature list

Source:	Samsung

Agenda item:	7.1.2.2.3

Document for:	Discussion

1.	Introduction

Two new UE capabilities for FR2 intra-band contiguous CA MPR enhancement were agreed to be introduced and the LS [1, R4-2505144] has been sent to RAN2. While the detailed MPR dependency of the new UE capabilities has been shared with RAN2, however, the type of the new UE capabilities was not mentioned in the LS.

The type of the new UE capabilities has been agreed as ‘per UE’ in RAN4#112bis meeting [2, R4-2417204]. It is noticed that FR2-1 and FR2-2 was not discussed separately when this agreement was made. In this contribution, we further examine if ‘per UE’ capability is appropriate for UE supporting both FR2-1 and FR2-2.

2. 	Discussion

It was agreed as ‘per UE’ in RAN4#112bis meeting [2, R4-2417204]:

Issue 3-2-1: Optional UE capability Agreement in main session: - An optional per UE capability for FR2 is introduced - The capability is implementation agnostic - Supporting FR2 MPR enhancement is applicable only from Rel-19

After checking the detailed discussion, it turns out that FR2-1 and FR2-2 was not involved explicitly during the discussion. Companies are focusing on optionality given it is UE implementation specific feature especially involving hardware change compared with legacy UE, as indicated in the topic summary [3, R4-2415218]:

Issue 3-2-1: Optional UE capability - Proposals - Proposal 1: The new capability shall be defined as optional and per UE, and only applicable to FR2. (ZTE) - Consider all possible UE implementations, including UE with fast LO switching, or with dedicated LOs for Tx and Rx paths for the new UE capability - Proposal 2: Introduce an optional UE capability to indicate the support of Rel-19 MPR applicability for FR2 CA. The granularity is per UE. (Huawei) - Proposal 3: Capability should not be dependent on a specific implementation (e.g. MPRimprovementULdependent-R19). (NTT DOCOMO) - Proposal 4: An optional new UE capability is needed, which can be per UE for FR2 and should be implementation agnostic. (Samsung) - The new UE capability of supporting FR2 MPR enhancement is not release independent. - Proposal 5: The modifications should be part of a new UE capability to efficiently separate those from legacy behaviour. As some of the improvements might not be feasible for all UE power classes and form factors the support of the new capabilities shall be optional and per band combination. (Apple) - Recommended WF - An optional [per UE] capability for FR2 is introduced - The capability is implementation agnostic - Supporting FR2 MPR enhancement is not release independent

Given the above discussion context, FR2-1 and FR2-2 was not separately discussed. Considering different characteristics of FR2-1 and FR2-2, it is worth to examine the case for UE supporting both FR2-1 and FR2-2.

Proposal 1:	While respecting previous agreed ‘per UE’ type, it is proposed to further examine if there is overlook for UE supporting both FR2-1 and FR2-2.

For example, if there is possibility that FR2-1 supports enhanced MPR and FR2-2 does not support enhanced MPR for a UE supporting both FR2-1 and FR2-2 bands, ‘per BC’ type would be proper. But if there is no such case in commercial implementation, ‘per UE’ type has the merit of saving signalling.

It is also worth mentioning that there is some restriction for the activation based MPR enhancement, i.e., “all the activated UL CCs forms a contiguous block”, as agreed in [R4-2420527]

Issue 3-1-1: Conditions of CC activation based enhancement Agreement in main session and WF: - For the Rel-19 activation based enhancement, FR2 UE CA MPR is based on the activated subset of the CCs in the CA configuration where all the activated UL CCs forms a contiguous block.

In order to avoid misuse of activation based MPR enhancement capability, it is proposed to mention “all the activated UL CCs forms a contiguous block” explicitly in both RAN2 UE capability description (feature list) and RAN4 feature CR.

Proposal 2:	It is proposed to mention “all the activated UL CCs forms a contiguous block” explicitly in both RAN2 UE capability description (feature list) and RAN4 feature CR.

3. 	Conclusion

Proposal 1:	While respecting previous agreed ‘per UE’ type, it is proposed to further examine if there is overlook for UE supporting both FR2-1 and FR2-2.

Proposal 2:	It is proposed to mention “all the activated UL CCs forms a contiguous block” explicitly in both RAN2 UE capability description (feature list) and RAN4 feature CR.

4. 	References

- R4-2505144     “LS on introduction of new UE capabilities for FR2 MPR improvement”, Samsung, Qualcomm

- R4-2417204    “WF on power domain enhancements”, Huawei, HiSilicon

- R4-2415218    “Topic summary for [112bis][114] NR_ENDC_RF_Ph4_part1”, Moderator (Huawei, HiSilicon)