3GPP TSG-RAN WG4 Meeting #115	R4-2506969

# Malta, MT, 19th – 23rd May, 2025

Title:	Discussion on UE RF requirements for LP-WUR

Source:	Samsung

Agenda item:	7.28.3.2

Document for:	Discussion

1.	Introduction

In last RAN4 meeting, it was agreed that two types of UE will be defined, corresponding UE capability details need to be discussed for R19 feature list. Regarding requirements, companies paid more attention to FR1 but FR2 discussion is not so sufficient. In this contribution, we share our views on UE type and FR2 LP-WUR requirements.

2. 	Discussion

2.1 Type 1 or Type 2 UE capability

RAN4 define two LP-WUR types (Type 1 and Type 2) with agreements as below [1, R4-2505234]

Issue 2-1-5: How to map two LP-WUR types with UE capability Agreement: - RAN4 define two LP-WUR types (Type 1 and Type 2), no need 1-to-1 mapping between waveform and LP-WUR type. Issue 2-3-1: ASCS requirements for OOK-based receiver Agreement: - For Type1 UE requirements, which are derived based on the assumption that demodulation of OOK content of LP-WUS is targeted - 1 RB guard for ASCS and 0.5dB de-sens - For Type2 UE requirements, which are derived based on the assumption that demodulation of OFDM content of LP-WUS is targeted - 0 RB guard for ASCS and 0 dB de-sens - For Type 1 and Type2 UE are not limited to specific LP-WUS waveform. - FFS how to define test case and requirement applicability for Type 1 and Type 2 - FFS how type 1 and type 2 capability related to RAN1 UE feature - A UE can only support either Type 1 or Type 2 but not both for LP-WUR

Based on above agreements, it can be concluded that the RAN4 defined LP-WUR type capability is corresponding to applicable requirements only and agnostic to LP-WUS waveform (OOK, OFDM) and power consumption target (ultra-low, moderate).

Proposal 1:	The RAN4 defined LP-WUR type capability is corresponding to applicable requirements only, and should be agnostic to LP-WUS waveform (OOK, OFDM) and power consumption target (ultra-low, moderate).

Regarding the capability type, two options may be discussed, i.e., per UE or per band. It is related with how to interpret the following agreement:

A UE can only support either Type 1 or Type 2 but not both for LP-WUR

The context of this agreement was to emphasize that the UE could not report both Type 1 and Type 2 ‘simultaneously’ for a specific case, the intention was not to define a ‘per UE’ capability in our understanding.

According to the UE feature list [2] attached in the RAN1 LS [3], most RAN1 UE feature list for LP-WUS is ‘[per band]’ type, in theory a UE could support OOK based LP-WUS for one band but support OFDM based LP-WUS for another band. It would be more likely for a UE supporting both FR1 LP-WUS and FR2 LP-WUS that the UE support OOK in FR1 LP-WUR but support OFDM in FR2 LP-WUR. Accordingly, it seems more appropriate to set ‘[per band]’ capability type for the RAN4 defined LP-WUR type, to be aligned with RAN1.

Proposal 2:	set ‘[per band]’ capability type for the RAN4 defined LP-WUR type, to be aligned with RAN1.

2.2 REFSENS for FR2 LP-WUR

In RAN4#113 meeting, it was agreed that OFDM-based FR2 LPWUR architecture comprises 2 Rx chains, and the antennas attached to the respective chains are mutually orthogonally polarized [4, R4-2420528]

Issue 2-2-6: Baseline OFDM-based architecture for FR2 LP-WUR Agreements: - FR2 LPWUR adopts zero-IF topology baseline assumption. FFS details - Multiple frequency conversion in RF domain is not precluded - For deriving a sensitivity requirement, the baseline OFDM-based FR2 LPWUR architecture is assumed to have the following features: - It comprises 2 Rx chains, and the antennas attached to the respective chains are mutually orthogonally polarized - FFS on whether and how to combine two Rx chains - Antenna gain is that of a single element - Coverage target should be considered

And in RAN4#114 meeting it was further confirmed that 1Rx at based band for FR2 LP-WUR [5, R4-2503028].

Issue 2-2-1: Combining strategy for FR2 LP-WUR Agreements: - For Rel-19, confirm no combination gain across two Rx chains is considered for FR2 LP-WUR. FFS whether and how to capture the impacts of Rx selection in REFSENS evaluation. - Confirm 1Rx at baseband for FR2 LP-WUR.

Then conclusion can be drawn that there will be no MRC combining for FR2 LP-WUR. Thus, the best combining strategy of FR2 LP-WUR would be selection-based diversity for the two Rx chains with orthogonally polarized antennas. Previous RAN4 discussion shows that this is common understanding but no agreement captured yet given this is an FFS item. For TR drafting purpose, it is good to achieve an agreement for this issue.

Proposal 3:	For TR drafting purpose, RAN4 to make such agreement that the combining strategy of FR2 LP-WUR should be selection-based diversity for the two Rx chains with orthogonally polarized antennas

Regarding detailed impacts to REFSENS evaluation, the direct impact is the diversity gain in the REFSENS derivation equation.

For FR2 MR, 0dB diversity gain is assumed which was also reflected in the EIS metric which is an average of the two EIS components, as shown in Annex K 1.2 of TS 38.521-2

…… 9) Calculate the resulting “averaged EIS” as: averaged EIS = 2*[1/EIS(PolMeas= PolLink= +1/EIS(PolMeas= PolLink=]-1 The RX beam peak direction is where the minimum “averaged EIS” is found.

However, for LP-WUR, the metric for REFSENS is changed from EIS to MDR. If the MDR based REFSENS requirement applies to both  polarization testing and  polarization testing respectively in the peak direction, it is more stringent compared with the “averaged EIS” metric, because the legacy EIS metric only requires the averaged performance between different link polarization testing but the new MDR metric requires the worst case between the two polarizations testing has to pass, i.e., for FR2 REFSENS, the final MDR should be:

MDR = Min { MDR(PolMeas= PolLink= , MDR(PolMeas= PolLink=) }

Observation 1:	If the MDR based REFSENS requirement applies to both  polarization testing and  polarization testing respectively in the peak direction, it is more stringent compared with the “averaged EIS” metric.

Corresponding to the more stringent MDR metric than averaged EIS, the diversity gain for REFSENS derivation has to be assumed as -3dB so that the worst link polarization testing can pass the requirement. In last meeting, 3dB diversity gain was agreed with square brackets [1, R4-2505234]:

Issue 2-2-2: Diversity gain value for FR2 LP-WUR Agreement: - [-3]dB diversity gain is assumed for FR2 LP-WUR REFSENS derivation

To be more precise, the -3dB diversity gain is bonded with the MDR metric replacing the “averaged EIS” metric. Different from FR1 which is conductive testing, FR2 REFSENS requires two orthogonal link polarization testing. To agree on the -3dB diversity gain assumption, an agreement on the criteria of the two orthogonal link polarizations is also needed as a package, i.e., the 1% MDR REFSENS requirement should be satisfied for each orthogonal link polarization respectively.

Proposal 4:	the 1% MDR REFSENS requirement should be satisfied for each orthogonal link polarization respectively, and -3dB diversity gain is assumed for FR2 LP-WUR REFSENS derivation.

Another issue is about the test direction of REFSENS. For MR, the test metric for peak EIS is EIS (Link=RX beam peak direction, Meas=Link Angle) as indicated in TS38.101-2. Accordingly, the test metric for LP-WUR could be MDR (Link=RX beam peak direction, Meas=Link Angle), but there is a question how to treat the case when there are many directions showing MDR=0%. One simple way could be reusing the RX beam peak direction of MR, however, in case the MR is equipped with two panels while only one of the two panels supports LP-WUR, the MR’s RX beam peak direction may not be appropriate for LR. For now, it is better to trigger the discussion with some options. The following options can be discussed for LP-WUR REFSENS peak direction:

- Option 1: reusing the RX beam peak direction of MR

- Option 2: the direction showing lowest MDR. If more than one direction showing MDR=0, any direction with MDR=0 can be used.

Proposal 5:	The following options can be discussed for LP-WUR REFSENS peak direction:

      - Option 1: reusing the RX beam peak direction of MR

      - Option 2: the direction showing lowest MDR. If more than one direction showing MDR=0, any direction with MDR=0 can be used.

Besides peak performance, spherical coverage was also discussed, and one FFS item is whether to skip the test of spherical coverage for LP-WUR [1, R4-2505234].

Issue 2-2-4: Spherical coverage for FR2 LP-WUR Agreement: - The coverage matching requires the same spherical coverage percentile of WUR and MR for different FR2 power classes. - FFS on whether to skip the test of FR2 LP-WUR for spherical coverage.

Given the FR2 LP-WUR is assumed with single element, i.e., there is no beamforming actually. One of the main purposes of spherical coverage is to guarantee the beamforming performance. For LP-WUR, it is valid not to test the spherical coverage.

Proposal 6:	It is okay to skip the test of FR2 LP-WUR for spherical coverage given no beamforming is assumed.

2.3 blocking and spurious response

In RAN4#114, whether to specify spurious response requirements is still an open issue [5, R4-2503028]

Issue 3-2-4: spurious response requirements for FR2 Way Forward - Further check next meeting on whether need to specify spurious response requirements for FR2 LP-WUR.

It is common understanding that out-of-band blocking and spurious response are two correlated requirements. Given there is no out-of-band blocking for MR, so no spurious response requirements are specified accordingly. The same principle can be used for LR.

Proposal 7:	No need to specify out-of-band blocking and spurious response requirements for FR2 LP-WUR.

3. 	Conclusion

Proposal 1:	The RAN4 defined LP-WUR type capability is corresponding to applicable requirements only, and should be agnostic to LP-WUS waveform (OOK, OFDM) and power consumption target (ultra-low, moderate).

Proposal 2:	set ‘[per band]’ capability type for the RAN4 defined LP-WUR type, to be aligned with RAN1.

Proposal 3:	For TR drafting purpose, RAN4 to make such agreement that the combining strategy of FR2 LP-WUR should be selection-based diversity for the two Rx chains with orthogonally polarized antennas

Observation 1:	If the MDR based REFSENS requirement applies to both  polarization testing and  polarization testing respectively in the peak direction, it is more stringent compared with the “averaged EIS” metric.

Proposal 4:	the 1% MDR REFSENS requirement should be satisfied for each orthogonal link polarization respectively, and -3dB diversity gain is assumed for FR2 LP-WUR REFSENS derivation.

Proposal 5:	The following options can be discussed for LP-WUR REFSENS peak direction:

      - Option 1: reusing the RX beam peak direction of MR

      - Option 2: the direction showing lowest MDR. If more than one direction showing MDR=0, any direction with MDR=0 can be used.

Proposal 6:	It is okay to skip the test of FR2 LP-WUR for spherical coverage given no beamforming is assumed.

Proposal 7:	No need to specify out-of-band blocking and spurious response requirements for FR2 LP-WUR.

4. 	References

- R4-2505234    “WF for [114bis][131] NR_LPWUS_UERF”, vivo

- R1-2501397    “Initial RAN1 UE features list for Rel-19 NR after RAN1 #120”, Moderators (AT&T, NTT DOCOMO, INC.)

- R4-2503104_R1-2501399    “LS on Rel-19 RAN1 UE features list for NR after RAN1#120”, RAN1

- R4-2420528    “WF on UE RF requirements for LP-WUS”, vivo

- R4-2503028    “WF on UE RF requirements for LP-WUS”, vivo