3GPP TSG-RAN WG4 Meeting #115	R4-2506991

# Malta, MT, 19th  - 23rd May 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.307 | CR | 0186 | rev | - | Current version: | 18.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | (LTE_NR_Simult_RxTx-Core) CR on release independence for simultaneous Rx-Tx capability for inter-band CA/DC in TS 38.307 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | MediaTek |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | LTE_NR_Simult_RxTx-Core |  |  |  |  |  | Date: |  |  | 2025-04-28 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Simultaneous Rx-Tx capability for inter-band CA/DC UE was supported from Rel-15 in TS38.331. However, this feature is not captured how to apply the capability as release independent manner in TS38.307. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Added the release independence manner for simultaneous Rx/Tx capability for inter-band CA/DC. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | It is unclear that the simultaneous Rx/Tx capability for inter-band CA/DC will be supported from which release. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.2.2, 8.1.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR … CR ... |  |  |
| affected: |  | x |  | Test specifications |  |  |  | TS 38.521-2, TS 38.533 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

********************************* Start on changes ************************************

### 5.2.2	Inter-band CA

Requirements for additional NR inter-band CA configurations within FR1 of TS 38.101-1 in Rel-P [2] are introduced via this clause.

Requirements for additional NR inter-band CA configurations with UL MIMO within FR1 of TS 38.101-1 in Rel-P [2] are introduced via this clause.

Requirements for additional NR inter-band CA configurations with Tx Diversity within FR1 of TS 38.101-1 in Rel-P [2] are introduced via this clause.

Table 5.2.2-0: NR inter-band CA UE power class within FR1

| Feature | Duplex-mode | Release independent from | Requirements to be fulfilled (see TS 38.307 of the release in which the power class was introduced) |
| --- | --- | --- | --- |
| Inter-band CA Power Class 2 | TDD, FDD and TDD | Rel-15 | Table B.4.2-1 |
| Inter-band CA Power Class 3 | FDD, TDD, SDL and TDD, FDD and TDD | Rel-15 |  |
| Inter-band CA Power Class 1.5 | TDD, FDD and TDD | Rel-17 | Table B.4.2-3, Table B.4.2-4 |

Table 5.2.2-1: NR inter-band CA within FR1

| Feature | DL/UL | Maximum number of bands | Maximum number of CCs | CA BW Classes | Duplex-mode | Release independent from | requirements to be fulfilled (see 38.307 of the REL in which the CA configuration was introduced) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Inter-band CA configurations within NR FR1 | DL | 6 | 9 | A, B, C, D, E | TDD, FDD, SDL and FDD, SDL and TDD, FDD and TDD, FDD and SDL and TDD | Rel-15 | Table B.4.2-1 |
|  | UL | 2 | 2 | A | TDD, FDD, FDD and TDD | Rel-15 |  |
|  |  | 1 | 2 | B, C, 2A |  |  |  |
|  |  | 2 | 3 | A, B, C |  |  |  |
| Inter-band CA configurations with UL MIMO within FR1 | DL | 2 | 2 | A | TDD, FDD and TDD | Rel-171 | Table B.4.2-3 |
|  | UL | 2 | 2 | A | TDD, FDD and TDD | Rel-171 |  |
| Inter-band CA configurations with Tx Diversity within FR1 | DL | 2 | 2 | A | FDD and TDD | Rel-171 | Table B.4.2-4 |
|  | UL | 2 | 2 | A | FDD and TDD | Rel-171 |  |
| Inter-band CA configurations with 1Tx-2Tx switching within NR FR1 | UL | 2 | 2 | A | TDD, FDD, FDD and TDD | Rel-16 | Table B.4.14-1 |
| Inter-band CA configurations with 1Tx-2Tx switching within NR FR1 | UL | 2 | 3 | A, B, C | TDD, FDD, FDD and TDD | Rel-17 | Table B.4.14-1 |
| Inter-band CA configurations with 2Tx-2Tx switching within NR FR1 | UL | 2 | 2 | A | TDD, FDD, FDD and TDD | Rel-17 | Table B.4.14-1 |
|  |  | 2 | 3 | A, B, C |  |  |  |
| Inter-band CA Simultaneous RxTx capability | DL | 3 | 3 | A | TDD-TDD, TDD-FDD, FDD-FDD, TDD-SDL, TDD-SUL | Rel-15 | Table B.4.19 |
|  | UL | 2 | 2 | A |  |  |  |
| NOTE 1: This is applied to FWA UE only. |  |  |  |  |  |  |  |

***************************** Unchanged sections are omitted *******************************

### 8.1.2	Inter-band EN-DC

#### 8.1.2.1	Inter-band EN-DC within frequency range 1

Requirements for additional EN-DC inter-band configurations within FR1 of TS 38.101-3 in Rel-P [4] are introduced via this clause.

Requirements for additional EN-DC inter-band configurations with UL MIMO within FR1 of TS 38.101-3 in Rel-P [4] are introduced via this clause.

Requirements for additional NR inter-band CA configurations with Tx Diversity within FR1 of TS 38.101-3 in Rel-P [2] are introduced via this clause.

Table 8.1.2.1-0: EN-DC inter-band UE power class

| Feature | Duplex-mode | Release independent from | Requirements to be fulfilled (see TS 38.307 of the release in which the band was introduced) |
| --- | --- | --- | --- |
| Inter-band EN-DC Power Class 2 | TDD, FDD and TDD | Rel-15 | Table B.4.6-1 |
| Inter-band EN-DC Power Class 3 | FDD, TDD, FDD and TDD | Rel-15 |  |

Table 8.1.2.1-1: EN-DC inter-band configurations without SUL within FR1

| Feature | DL/UL | maximum number of E-UTRA bands | maximum number of E-UTRA CCs | maximum number of NR bands | maximum number of NR CCs | Duplex-mode | Release independent from | requirements to be fulfilled (see 38.307 of the REL in which the CA configuration was introduced) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Inter-band EN-DC | DL | 5 | 6 | 4 | 4 | FDD, TDD, FDD and TDD, FDD and SDL, FDD and SDL and TDD | Rel-15 | Table B.4.6-1 |
|  | UL | 1 | 2 contiguous | 1 | 2 contiguous | FDD, TDD, FDD and TDD | Rel-15 |  |
| Inter-band EN-DC with UL MIMO within FR1 | DL | 1 | 1 | 1 | 1 | TDD, FDD and TDD | Rel-171 | Table B.4.6-2 |
|  | UL | 1 | 1 | 1 | 1 | TDD, FDD and TDD | Rel-171 |  |
| Inter-band EN-DC with Tx Diversity within FR1 | DL | 1 | 1 | 1 | 1 | FDD and TDD | Rel-171 | Table B.4.6-3 |
|  | UL | 1 | 1 | 1 | 1 | FDD and TDD | Rel-171 |  |
| Inter-band EN-DC with 1Tx-2Tx switching | UL | 1 | 1 | 1 | 1 | TDD, FDD and TDD | Rel-16 | Table B.4.16-1 |
| Inter-band EN-DC Simultaneous RxTx capability | DL | 1 | 1 | 2 | 2 | TDD-TDD, TDD-FDD, FDD-FDD, TDD-SDL, TDD-SUL | Rel-15 | Table B.4.20 |
|  | UL | 1 | 1 | 1 | 1 |  |  |  |
| NOTE 1: This is applied to FWA UE only. |  |  |  |  |  |  |  |  |

***************************** Unchanged sections are omitted *******************************

## B.4.19	Common UE RF requirements for Inter-band CA configurations with Simultaneous RxTx capability

The requirements and test cases listed in Table B.4.19-1 are specified in Rel-P version of TS 38.101-1 [2].

Table B.4.19-1: Common UE RF requirements for Inter-band CA configurations with simultaneous RxTx capability for FR1

| Clause | Description |
| --- | --- |
| 5.2A.2 | Operating bands for inter-band CA |
| 5.5A.3 | Configurations for inter-band CA |
| 6.2A.4.2.3 & 6.2A.4.2.4 | ΔTIB,c for Inter-band CA (two bands) & ΔTIB,c for Inter-band CA (three bands) |
| 7.3A.3.2.1 & 7.3A.3.2.3 | ΔRIB,c for two bands & ΔRIB,c for three bands |
| 7.3A.5 | Reference sensitivity exceptions due to intermodulation interference due to 2UL CA |
| 7.3A.6 | Reference sensitivity exceptions due to cross band isolation for CA |

Table B.4.19-2: Common UE RF requirements for Inter-band CA configurations with simultaneous RxTx capability between FR1 and FR2

| Clause | Description |
| --- | --- |
| 5.2A.1 | Inter-band CA between FR1 and FR2 |
| 5.3A.1 | Inter-band CA UE channel bandwidth between FR1 and FR2 |
| 5.5A.1 | Inter-band CA configurations between FR1 and FR2 |

## B.4.20	Common UE RF requirements for Inter-band EN-DC configurations with Simultaneous RxTx capability

The requirements and test cases listed in Table B.4.20-1 are specified in Rel-P version of TS 38.101-3 [4].

Table B.4.20-1: Common UE RF requirements for Inter-band EN-DC configurations with simultaneous RxTx capability

| Clause | Description |
| --- | --- |
| 5.5B.4 | Inter-band EN-DC configuration within FR1 |
| 5.5B.5 | Inter-band EN-DC configurations including FR2 |
| 5.5B.6 | Inter-band EN-DC configurations including FR1 and FR2 |
| 6.2B.4.2.3 | ΔTIB,c for Inter-band EN-DC |
| 7.3B.2.3 | Reference sensitivity exceptions |
| 7.3B.3.3 | ΔRIB,c for Inter-band EN-DC within FR1 |

********************************* End of changes ****************************************