3GPP TSG RAN WG4 Meeting #115		 R4-2506994

St Julian’s, Malta  19th – 23th May, 2025

Agenda Item:	7.5.3

Source: 	MediaTek Inc.

Title: 	Discussion on signalling aspects for DL fragmented carriers study

Document for:	Approval

1. Introduction

UE architecture assumption and initial evaluation based on existing test configurations were agreed and captured in the WF [12]. In the previous meeting, RAN4 continue discussion on signalling aspects and general aspect were agreed and captured in WF[22]. In this contribution, we continue discussion and provide our proposals on the open issues.

2. Discussion

2.1 Continue discussion on signalling aspects

In RAN4114bis, WF[22] on signalling aspects were agreed and copied below:

Issue 2-3: Triggering condition to enable ”One Rx RF chain”

RAN4 agree there are two aspects regarding the triggering condition enable/disable the one Rx RF chain mode:

- Option 1: Rely on existing channel quality reporting and procedures. No new indication from UE is needed

- Option 2: The UE need the measurement to determine and indicate to the network whether it can support non-contiguous CCs with one RX RF chain or not, assuming the associated degradation is no greater than allowed degradation that is specified

- Note: detailed conditions to enable/disable can be further discussed and decided separately

There are mainly two different aspects. The benefit of option 1 can be easier from UE implementation. The existing channel quality reporting procedure can be re-used as stated in[23]. But new UE measurement type when UE is configured as “one Rx RF chain” would be needed for UE or network to differentiate the channel quality indicator between different architecture configurations.

Observation 1: New UE measurement type may still be needed when the existing channel quality reporting for one Rx RF chain is used

Option 2: UE measurement to determine and indicate to the network

When the UE enables one Rx RF chain to receive two non-contiguous carriers under the network, it was assumed another adjacent channel downlink carrier, from collocated second operator, in the gap may also exists at the same time. From companies simulation, the in-gap blocker level may possible be 10 or 20 dB higher than the wanted carriers in some cell environments that may seriously degrade the receiving signal quality of wanted carriers. A new measurement mechanism can be needed to judge the in-gap blocker power level and compare with the wanted signal power levels. We agree the view point that the states of UE is better to be defined then we discuss procedure of changing states. Referring [23] we propose four states UE shall consider as following:

State 1: UE is configured for CC1 on the band and meets the requirements accordingly

State 2: UE is configured for non-contiguous CC1 and CC2 on the band and uses two Rx chains to receive them and meets existing DL NCCA requirements accordingly

State 3: UE is configured for non-contiguous CC1 and CC2 on the band and uses one Rx RF chain to receive them and meets possible new requirements accordingly

Stete 4: UE is configured same as state 1 (CC1, Pcell) and has been checked no more available Rx RF chain to receive intra-band non-contiguous CC2 using second Rx RF chain

Figure 1. Description of different UE states for fragmented carriers

Figure 2. State diagram for switching between the states

When UE has more available Rx RF chain, UE can just apply legacy DL NCCA architecture to maintain best performance on receiving each CC as illustrated in Figure 2. UE can also be configured for measurement by network to check whether the large interference exists and whether UE is capable to receive one more CC when needed while UE is in state 2.

When UE don’t have more available Rx RF chain (state 4), UE can still be configured for measurement by network doing same check. UE can just report to network capable or not-capable for the one Rx RF chain as UE assisted information.

When UE is configured by network to do measurement on both wanted CCs and in-gap interference, UE can simply using existing RSSI measurement approach for in-gap interference measurement. Network can configure the measurement bandwidth and the virtual RB location to the UE, wherein the RB location to be measured in the in-gap region can be referring to PCC or one of the CCs. The details of measurement flow can be further discussed during WI phase if there is work item in the future.

The benefit if UE can measure in-gap interference can be observed from the state chart is ping-pong between state 2 and state 3 or between state 4 and state 3 can be reduced if large in-gap interference can be discovered.

Figure 3. Spectrum plot of DL fragmented carriers and in-gap interference

Observation 2: The nominal NR UE can support Channel BW up to 100MHz so it would be possible to measure noise level of both wanted CCs and in-gap range if total BW is within the 100MHz under one Rx RF chain mode.

Observation 3: One possible method to measure in-gap interference level is to referring existing RSSI measurement procedure with virtual RB ranking in the in-gap region or refer to measuring neighbour cell procedure. The measurement would be applicable in both one Rx RF chain mode and separated Rx RF chain mode.

Observation 4: The ping-pong between state 2/4 and state 3 can be reduced if UE can measure in-gap interference.

After UE measures all the power levels, there are different signalling methods to proceed the measurement results:

- UE judge whether one Rx RF chain is applicable: Consider different UE may have different interference rejection capability and to save dynamic signalling overhead, UE can self-judge whether the one Rx RF chain can be enabled and only report capable/not-capable and necessary CCs information to network. The signalling overhead can be minimized with this approach.

- Network judge whether one Rx RF chain is applicable: An alternative is that UE reports all the measured information, including in-gap interference level, wanted signal power levels and UE capable ISR (interference-to-signal ratio), to network for the network to judge whether one Rx RF chain can be enabled.

Observation 5: To minimize signalling overhead, UE can judge whether the one Rx RF chain can be enabled and just send one-bit (True/False) assistant information to the network that network can decide how to schedule CA configurations to UE.

In-gap inter-operator interference may not be the only factor that has impact on DL receiving performance when one Rx RF chain mode is configured. During Rx performance evaluation and discussion between companies, it was found self-band Tx leakage also plays an important role. It would be more clear to understand these impacts via evaluating the two factors with the help from system level simulation. The simulation assumptions are listed in the Table 1 below.

Table 1: Assumptions for system level simulation

| Parameter | NR |
| --- | --- |
| Carrier | 2G Hz |
| DL BW | 20MHz |
| UL BW | 20MHz |
| Antenna pattern | TR 38.901 Table 7.3-1 |
| Network layout | 19-sites (57 sectors) with wrap-around |
| Inter-site distance in meter | 500 for 2GHz band for UMA |
| System loading and activity | Full buffer 100% |
| Network layout | 19-sites [57 sectors] with wrap-around |
| DL power control | No |
| UL power control | TR 36.942[8] section 5.1.1.6 (set 1) by bandwidth scale, target SNR at BS is 15 dB |
| NR UE dropping | NR UE: 100% outdoor UE number: DL active UE: 10 UE per cell |
| Pathloss model | TR 38.901 UMA |
| O2I penetration loss | High penetration loss as in TR 38.901 |

It is assumed co-located inter-operators, taking 19-sites with 3 sectors in each site. 10 UEs in every sector per operator. These are illustrated below:

Figure 1. Network layout model for system level simulation

For the UE parameter assumptions, they are listed in the Table 2. Per agreed WF, one Rx RF chain ACS is assumed around 15 to 26 dB so the two values are simulated. 33dB is assumed as performance reference of partial separated Rx RF chain configuration.

Table 2: UE assumptions for system level simulation

|  |  | NR |
| --- | --- | --- |
| UE | Tx ACLR | 30dB (ACLR1) 43dB (ACLR2) |
|  | Duplexer isolation between T/R | 50dB |
|  | Rx baseband analog filter Tx leakage rejection | 5dB (one Rx RF chain) 15dB (partial separated Rx RF chain) |
|  | Rx ACS | 15, 26dB (one Rx RF chain) 33dB (partial separated Rx RF chain) |

Simulation result is illustrated in below figures.

Figure 2. CDF of DL signal and interference distribution w w/o ACI of Tx leakage

From the simulation results, some important aspects can be observed.

- In-gap interference and Tx leakage both have impact on DL receiving SINR performance. Which one occupies larger portion may depend on UE implementation and the distance/channel condition between UE and gNB.

- UE self-judgement mechanism would be needed with considering both in-gap interference and Tx leakage. As long as UE can guarantee pass the adjusted minimum requirements, if specified. How to judge whether UE is capable for one Rx RF chain mode can be left to UE implementation.

- Another phenomenon can be observed, even though there are some degradations on the DL receiving SINR, there are certain percentage of UEs can still benefit from one Rx RF chain, when higher order CA combo is configured, with acceptable degradation.

Observation 6: In-gap interference and Tx leakage both have impact on DL receiving SINR performance when one Rx RF chain is configured.

Proposal 1: The existing RSSI measurement method can be used for the in-gap interference and Tx leakage measurement

Proposal 2: UE self-judgement mechanism considering both in-gap interference and Tx leakage is proposed when one Rx RF chain mode is enabled.

Proposal 3: We propose to capture above signalling approach in the last part of the contribution into TR38.755

3. Conclusion

Observation 1: New UE measurement type may still be needed when the existing channel quality reporting for one Rx RF chain is used

Observation 2: The nominal NR UE can support Channel BW up to 100MHz so it would be possible to measure noise level of both wanted CCs and in-gap range if total BW is within the 100MHz under one Rx RF chain mode.

Observation 3: One possible method to measure in-gap interference level is to referring existing RSSI measurement procedure with virtual RB ranking in the in-gap region or refer to measuring neighbour cell procedure. The measurement would be applicable in both one Rx RF chain mode and separated Rx RF chain mode.

Observation 4: The ping-pong between state 2/4 and state 3 can be reduced if UE can measure in-gap interference.

Observation 5: To minimize signalling overhead, UE can judge whether the one Rx RF chain can be enabled and just send one-bit (True/False) assistant information to the network that network can decide how to schedule CA configurations to UE.

Observation 6: In-gap interference and Tx leakage both have impact on DL receiving SINR performance when one Rx RF chain is configured.

Proposal 1: The existing RSSI measurement method can be used for the in-gap interference and Tx leakage measurement

Proposal 2: UE self-judgement mechanism considering both in-gap interference and Tx leakage is proposed when one Rx RF chain mode is enabled.

Proposal 3: We propose to capture above signalling approach in the last part of the contribution into TR38.755

TP for TR38.755

8	Study of means for a UE to inform the network of appropriate CA configuration it can support

<Start of TP>

It has been agreed to consider the following aspects for how to indicate UE capability for supporting ”one Rx RF chain” for DL fragmented carriers[22]:

  - Alternative 1: use the legacy band combination reporting scheme + Rx sharing capability indication

    - Alternative 1 has no modification of CA configurations in RAN4

  - Alternative 1a: use the legacy band combination reporting scheme + Rx sharing capability and other related information including the frequency separation between two carriers.

    - Alternative 1a has no modification of CA configurations in RAN4

  - Alternative 2: use the new notation for CA configurations with single Rx RF Chain

  - Alternative 2a: use the new bandwidth class conditioned on the frequency separation between two carriers

RAN4 also agreed to clarify CA fallback and architecture switching as following aspects:

  - CA fallback with/without UE architecture switching

    - Consider the fall back

      - from high order CA to lower order CA

      - from CA to single carrier mode

    - Decided by network

  - UE architecture switching:

    - Switching between 2 Rx RF chains and 1 Rx RF chain on CCs of intra-band NC CA

When discussing UE behaviours, it is also agreed to consider triggering condition to enable the “one Rx RF chain” mode as following aspects:

- Option 1: Rely on existing channel quality reporting and procedures. No new indication from UE is needed

- Option 2: The UE need the measurement to determine and indicate to the network whether it can support non-contiguous CCs with one RX RF chain or not, assuming the associated degradation is no greater than allowed degradation that is specified

- Note: detailed conditions to enable/disable can be further discussed and decided separately

8.1	Indication of one Rx RF chain mode for DL fragmented CA

8.1.x	Indication of fragmented CA alternative 1: Legacy band combination reporting scheme + Rx sharing capability indication

< to be updated>

8.1.x	Indication of fragmented CA alternative 1a: Legacy band combination reporting scheme + Rx sharing capability and other related information

< to be updated>

8.1.x	Indication of fragmented CA alternative 2: New notation for CA configurations with single Rx RF Chain

< to be updated>

8.1.x	Indication of fragmented CA alternative 2a: use the new bandwidth class conditioned on the frequency separation between two carriers

To allow both the UE and the network separating fragmented carrier operation from other types of CA configurations a different indication is suggested during the study phase.

A proposal is to use the currently undefined NR CA bandwidth class (BCS) ‘F’ should be introduced for indicating a CA configuration with fragmented carrier operation.

TS 38.101-1: Table 5.3A.5-1: NR CA bandwidth classes

| NR CA bandwidth class | Aggregated channel bandwidth | Number of contiguous CC | Fallback group |
| --- | --- | --- | --- |
| A | BWChannel ≤ BWChannel,max | 1 | 1, 2, 34, 4 |
| B | 20 MHz ≤ BWChannel_CA ≤ 100 MHz | 2 | 2, 34 |
| C | 100 MHz < BWChannel_CA ≤ 2 x BWChannel,max | 2 | 1, 34 |  |
| D | 200 MHz < BWChannel_CA ≤ 3 x BWChannel,max | 3 |  |  |
| E | 300 MHz < BWChannel_CA ≤ 4 x BWChannel,max | 4 |  |  |
| F | BWChannel ≤ 100 MHz | 1, NOTE 5 | 4 |  |
| G | 100 MHz < BWChannel_CA ≤ 150 MHz | 3 | 2 |  |
| H | 150 MHz < BWChannel_CA ≤ 200 MHz | 4 |  |  |
| I | 200 MHz < BWChannel_CA ≤ 250 MHz | 5 |  |  |
| J | 250 MHz < BWChannel_CA ≤ 300 MHz | 6 |  |  |
| K | 300 MHz < BWChannel_CA ≤ 350 MHz | 7 |  |  |
| L | 350 MHz < BWChannel_CA ≤ 400 MHz | 8 |  |  |
| M3 | 50 MHz ≤ BWChannel_CA ≤ 200 MHz | 3 | 34 |  |
| N3 | 80 MHz ≤ BWChannel_CA ≤ 300 MHz | 4 |  |  |
| O3 | 100 MHz ≤ BWChannel_CA ≤ 400 MHz | 5 |  |  |
| NOTE 1: BWChannel, max is maximum channel bandwidth supported among all bands in a release NOTE 2: It is mandatory for a UE to be able to fallback to lower order NR CA bandwidth class configuration within a fallback group. It is not mandatory for a UE to be able to fallback to lower order NR CA bandwidth class configuration that belong to a different fallback group. NOTE 3: This bandwidth class is only applicable to bands identified for use with shared spectrum channel access in Table 5.2-1. NOTE 4: Fallback group 3 is only applicable to bands identified for use with shared spectrum channel access in Table 5.2-1. NOTE 5: For fragmented carrier operation with a single contiguous frequency range configured for the UE with multiple fragmented carriers as indicated by IE [FragmentedCarrier]. |  |  |  |

In the proposal above, where the newly added parts are highlighted with yellow, F is inserted with a corresponding number of contiguous CCs as ‘1’, as the intention is to treat the multiple fragmented carriers as a single CC regarding the number of occupied Rx RF chains. Adding Note 5 clarifies that this single contiguous CC can consist of multiple fragmented carriers assumed occupying 1 Rx chain. The new BCS, F, is then added with fallback group 4, which is added correspondingly for the single CC BCS, A, as the fallback from fragmented carrier operation would be single CC operation. However, if the UE can fallback to 2 Rx RF chains from single Rx RF chain (due to high in-gap interference) this should also be possible.

By adding the new NR CA bandwidth class, this can be used to identify fragmented carrier operation and the number of fragments. Examples using band n25 with two fragments are shown in Fig 8.1.x-1.

Fig.  8.1.x-1: Example of CA_n25(2F)

8.2	CA fallback and architecture switching

< to be updated>

8.3	Triggering condition to enable ”One Rx RF chain”

8.3.x Triggering condition proposal for option 1: Rely on existing channel quality reporting and procedures

< to be updated>

8.3.x Triggering condition proposal for option 2: UE measurement to determine and indicate to the network

Triggering condition to enable ”One Rx RF chain” with UE assisted in-gap interference measurement and indication

When the UE enables one Rx RF chain to receive two non-contiguous carriers under the network, it was assumed another adjacent channel downlink carrier, from collocated second operator, in the gap may also exists at the same time. From companies simulation, the in-gap blocker level may possible be 10 or 20 dB higher than the wanted carriers in some cell environments that may seriously degrade the receiving signal quality of wanted carriers. A new measurement mechanism can be needed to judge the in-gap blocker power level and compare with the wanted signal power levels. We agree the view point that the states of UE is better to be defined then we discuss procedure of changing states. Referring [23] we propose four states UE shall consider as following:

State 1: UE is configured for CC1 on the band and meets the requirements accordingly

State 2: UE is configured for non-contiguous CC1 and CC2 on the band and uses two Rx chains to receive them and meets existing DL NCCA requirements accordingly

State 3: UE is configured for non-contiguous CC1 and CC2 on the band and uses one Rx RF chain to receive them and meets possible new requirements accordingly

Stete 4: UE is configured same as state 1 (CC1, Pcell) and has been checked no more available Rx RF chain to receive intra-band non-contiguous CC2 using second Rx RF chain

Figure 8.3.x-1: Description of different UE states for fragmented carriers

Figure 8.3.x-2: State diagram for switching between the states

When UE has more available Rx RF chain, UE can just apply legacy DL NCCA architecture to maintain best performance on receiving each CC as illustrated in Figure 2. UE can also be configured for measurement by network to check whether the large interference exists and whether UE is capable to receive one more CC when needed while UE is in state 2.

When UE don’t have more available Rx RF chain (state 4), UE can still be configured for measurement by network doing same check. UE can just report to network capable or not-capable for the one Rx RF chain as UE assisted information.

When UE is configured by network to do measurement on both wanted CCs and in-gap interference, UE can simply using existing RSSI measurement approach for in-gap interference measurement. Network can configure the measurement bandwidth and the virtual RB location to the UE, wherein the RB location to be measured in the in-gap region can be referring to PCC or one of the CCs. The details of measurement flow can be further discussed during WI phase if there is work item in the future.

The benefit if UE can measure in-gap interference can be observed from the state chart is ping-pong between state 2 and state 3 or between state 4 and state 3 can be reduced if large in-gap interference can be discovered.

Figure 8.3.x-3: Spectrum plot of DL fragmented carriers and in-gap interference

Observation 1: It would be possible to measure noise level of both wanted CCs and in-gap range if total BW is within the 100MHz under one Rx RF chain mode. One possible method to measure in-gap interference level is to referring existing RSSI measurement procedure with virtual RB ranking in the in-gap region or refer to measuring neighbour cell procedure. The measurement would be applicable in both one Rx RF chain mode and legacy (partial-separated Rx RF chain) mode.

Observation 2: The ping-pong between state 2/4 and state 3 can be reduced if UE can measure in-gap interference.

After UE measures all the power levels, there are different signalling methods to proceed the measurement results:

- UE judge whether one Rx RF chain is applicable: Consider different UE may have different interference rejection capability and to save dynamic signalling overhead, UE can self-judge whether the one Rx RF chain can be enabled and only report capable/not-capable and necessary CCs information to network. The signalling overhead can be minimized with this approach.

- Network judge whether one Rx RF chain is applicable: An alternative is that UE reports all the measured information, including in-gap interference level, wanted signal power levels and UE capable ISR (interference-to-signal ratio), to network for the network to judge whether one Rx RF chain can be enabled.

Observation 3: To minimize signalling overhead, UE can judge whether the one Rx RF chain can be enabled and just send one-bit (True/False) assistant information to the network that network can decide how to schedule CA configurations to UE.

In-gap inter-operator interference may not be the only factor that has impact on DL receiving performance when one Rx RF chain mode is configured. During Rx performance evaluation and discussion between companies, it was found self-band Tx leakage also plays an important role. It would be more clear to understand these impacts via evaluating the two factors with the help from system level simulation. The simulation assumptions are listed in the Table 1 below.

Table 8.3.x-1: Assumptions for system level simulation

| Parameter | NR |
| --- | --- |
| Carrier | 2G Hz |
| DL BW | 20MHz |
| UL BW | 20MHz |
| Antenna pattern | TR 38.901 Table 7.3-1 |
| Network layout | 19-sites (57 sectors) with wrap-around |
| Inter-site distance in meter | 500 for 2GHz band for UMA |
| System loading and activity | Full buffer 100% |
| Network layout | 19-sites [57 sectors] with wrap-around |
| DL power control | No |
| UL power control | TR 36.942[8] section 5.1.1.6 (set 1) by bandwidth scale, target SNR at BS is 15 dB |
| NR UE dropping | NR UE: 100% outdoor UE number: DL active UE: 10 UE per cell |
| Pathloss model | TR 38.901 UMA |
| O2I penetration loss | High penetration loss as in TR 38.901 |

It is assumed co-located inter-operators, taking 19-sites with 3 sectors in each site. 10 UEs in every sector per operator. These are illustrated below:

Figure 8.3.x-4: Network layout model for system level simulation

For the UE parameter assumptions, they are listed in the Table 2. Per agreed WF, one Rx RF chain ACS is assumed around 15 to 26 dB so the two values are simulated. 33dB is assumed as performance reference of partial separated Rx RF chain configuration.

Table 8.3.x-2: UE assumptions for system level simulation

|  |  | NR |
| --- | --- | --- |
| UE | Tx ACLR | 30dB (ACLR1) 43dB (ACLR2) |
|  | Duplexer isolation between T/R | 50dB |
|  | Rx baseband analog filter Tx leakage rejection | 5dB (one Rx RF chain) 15dB (partial separated Rx RF chain) |
|  | Rx ACS | 15, 26dB (one Rx RF chain) 33dB (partial separated Rx RF chain) |

Simulation result is illustrated in below figures.

Figure 8.3.x-5: CDF of DL signal and interference distribution w w/o ACI of Tx leakage

From the simulation results, some important aspects can be observed.

- In-gap interference and Tx leakage both have impact on DL receiving SINR performance. Which one occupies larger portion may depend on UE implementation and the distance/channel condition between UE and gNB.

- UE self-judgement mechanism would be needed with considering both in-gap interference and Tx leakage. As long as UE can guarantee pass the adjusted minimum requirements, if specified. How to judge whether UE is capable for one Rx RF chain mode can be left to UE implementation.

- Another phenomenon can be observed, even though there are some degradations on the DL receiving SINR, there are certain percentage of UEs can still benefit from one Rx RF chain, when higher order CA combo is configured, with acceptable degradation.

<End of TP>

4. Reference for the contribution:

To be consistent with TR38.755, the reference contributions are based on TR38.755 v0.2.0

[1]	3GPP TR 21.905: “Vocabulary for 3GPP Specifications”.

[2]	RP-241669, “New SID: Study on NR FR1 DL Fragmented Carriers”.

[3] 	3GPP TS 38.101-1: “NR; User Equipment (UE) radio transmission and reception; Part 1: Range 1 Standalone”.

[4] 	3GPP TS 38.101-2: “NR; User Equipment (UE) radio transmission and reception; Part 2: Range 2 Standalone”.

[5] 	3GPP TS 38.101-3: “NR; User Equipment (UE) radio transmission and reception; Part 3: Range 1 and Range 2 Interworking operation with other radios”.

[6] 	R4-2413339 Discussion on methods for reducing the number of UE Rx chains for Fragmented Carriers, RAN4#112, Nokia

[7] 	R4-2415390 Discussion on methods for reducing the number of UE Rx RF chains for Fragmented Carriers, RAN4#112bis, Nokia

[8] 	RP-233374, Fragmented carriers in the DL, TELUS, Bell Mobility, T-Mobile USA, Telstra Limited, US Cellular Corporation, Nokia, Nokia Shanghai Bell

[9] 	R4-2411691 On general aspects of fragmented carriers, RAN4#112, Huawei

[10] 	RP-241269, On Fragmented Carriers in Rel-19, Apple

[11] 	RP-241361, Recommended WF on Fragmented Carriers, MediaTek

[12] 	R4-2417201 WF on NR FR1 DL fragmented carriers study, RAN4#112bis

[13] 	R4-2413031 On architecture options for fragmented spectrum reception, RAN4#112, Skyworks

[14] 	R4-2416138 Discussion on UE Rx chain architecture and network assumption, RAN4#112bis, MediaTek

[15] 	R4-2420519 WF on NR FR1 DL fragmented carriers study, RAN4#113

[16] 	R4-2415804 On methods for reducing the number of UE Rx chain, RAN4#112bis, Huawei

[17]	R4-2503479 TP to TR 38.755 on Clause 5, RAN4#114bis, CATT

[18]	R4-2503558 TP for TR38.755: Company C evaluation results, RAN4#114bis, Qualcomm

[19]	R4-2504452 TP on UE RF requirements impact for fragmented carriers, RAN4#114bis, Apple

[20]	R4-2504214 Further discussion of impacts on UE RF requirements and DL performance impacts, RAN4#114bis, MediaTek

[21]	R4-2504129 General aspects of Fragmented CA, RAN4#114bis, Nokia

[22]	R4-2505109 WF on NR FR1 DL fragmented carriers study, RAN4#114bis, MediaTek, Apple, Qualcomm

[23]	R4-2503556 Procedures for fragmented carriers, RAN4#114bis, Qualcomm