3GPP TSG-RAN WG4 Meeting #115	R4-2506995

Malta, 19 May – 23 May, 2025

Agenda item:	9.3

Source: 	vivo

Title: 	Discussion on reply LS R2-2503163

Document for:	Discussion

- Introduction

This contribution discuses issues in RAN2 LS R2-2503163.

- Discussion

On the collision between the MUSIM Gap and Measurement Gap (for the UE supporting musim-GapPriorityPreference capability) in R18, RAN2 identified that the network can configure the MUSIM gap without priority and the measurement Gap with priority together and vice versa. In addition, it is also supported from RAN2 specification that the network can configure the MUSIM gap without priority and the measurement Gap without priority together.

RAN2 respectfully ask RAN4 on whether UE requirement is needed for the above collision cases.

In LS R2-2503163, the following issue has been raised:

In current RAN4 specs, it indicates “Priorities are applied for each periodic MUSIM gap. A UE shall request priorities for all requested periodic MUSIM gaps when the UE requests MUSIM gaps [2].”

Since a UE shall request priorities for all requested periodic MUSIM gaps when the UE requests MUSIM gaps, and the network will keep the relative priority between periodic MUSIM gaps. This means normally priorities will be allocated to periodic MUSIM gaps.

In addition RAN2#123 has the following agreement:

2.	When receiving priorities for periodic MUSIM gap(s), the UE may receive changed priority values. If network doesn’t retain the relative priorities among MUSIM gaps, UE behaviour is not specified.

And also in RAN4 specs:

The requirements in clause 9.1.10 apply provided different priorities are allocated to each periodic MUSIM gap and each measurement gap which is configured via GapConfig-r17 without preConfigInd-r17 or ncsgInd-r17.

Hence if priority is not allocated for MUSIM gaps, neither UE behaviour nor requirements is specified and there is no any further RAN4 action on this issue.

Proposal 1: If priority is not allocated for MUSIM gaps, UE behaviour is not specified and there is no any further RAN4 action on this issue.

- Conclusions

In this contribution we provide our analysis on the RAN2 LS R2-2503163 and has the following proposal:

Proposal 1: If priority is not allocated for MUSIM gaps, UE behaviour is not specified and there is no any further RAN4 action on this issue.

References

[1] R2-2503163, LS on Collision between MUSIM Gap and Measurement Gap RAN2, RAN2 129bis