3GPP TSG-RAN WG4 Meeting #115	R4-2507007

Malta, 19th – 23rd May 2025

Agenda item:	7.26.3.1

Source: 		NTT DOCOMO, INC.

Title: 	Discussions on A-IoT BS RF requirements

Document for:	Discussion

# Introduction

From the meeting RAN4#114, we started to discuss A-IoT BS RF requirements in D1T1-B scenario on the WID [1]. Some RF issues are still open, we contribute the general view.

# Discussion

According to the latest WF [2], the open issues of A-IoT BS RF are described as follows.

### Issue 1-5-2: ACLR

- Agreement:

  - Consider following options for ACLR requirements

    - Option 1: ACLR1 is 40dB and ACLR2 is 50dB for A-IOT BS 1PRB transmission BW with 100kHz offset

    - Option 2: ACLR 1 is 30dB and ACLR2 is 40dB for A-IoT BS 1PRB transmission BW with Foffset=100kHz.

  - FFS on whether frequency offset is needed or not for other PRB numbers

### Issue 2-1-1: Reference sensitivity level

- Agreement:

  - Consider reference sensitivity requirements for the reader [without frequency shift (R=1)]

  - FFS on side condition of CW for sensitivity requirement

### Issue 2-3: ACS

- Agreement:

  - For ACS requirements, use legacy NB-IoT requirements as starting point

  - FFS on the frequency offset for ACS requirement

### Issue 2-4-1: general blocking

- Agreement:

  - FFS whether or not to reuse the in-band blocking requirement from NR Micro BS.

### Issue 2-7-2: Narrowband intermodulation

- Agreement:

  - FFS whether or not to reuse FR1 NB-IoT medium range BS Rx narrowband intermodulation requirement in the licensed band for A-IoT BS Rx narrowband intermodulation requirement.

## Adjacent Channel Leakage Power Ratio

Regarding the ACLR limits, there is no compelling reason to relax them based on the differences between NB-IoT and A-IoT.

Proposal 1: The ACLR limits can be the same as those for NB-IoT as Table 1. (ACLR1: 40 dB, ACLR2: 50 dB)

In the previous discussion on the ACLR requirements, it was noted that when number of PRBs is 1, two options remain, both of which include 100 kHz offset. This offset is considered necessary due to the difficulties associated with designing a narrowband filter for the BWChannel. It is fine to reduce 100 kHz offset depending on the filter performance from 2 PRBs to 4 PRBs.

Proposal 2: The 100 kHz offset for ACLR can be applied for PRBs 2 to 4 as Table 1.

Table 1: A-IoT BS ACLR limit

| BS channel bandwidth of lowest/highest carrier transmitted BWChannel (kHz) | BS adjacent channel centre frequency offset below the lowest or above the highest carrier centre frequency transmitted | Assumed adjacent channel carrier (informative) | Filter on the adjacent channel frequency and corresponding filter bandwidth | ACLR limit |
| --- | --- | --- | --- | --- |
| 200/400/600/800kHz | BWChannel + 100 kHz | A-IoT of same BW | Square (BWConfig) | 40 dB |
|  | 2 x BWChannel + 100 kHz | A-IoT of same BW | Square (BWConfig) | 50 dB |

## Adjacent Channel Selectivity

ACS is not important in the absence of indoor UE, but there is no reason for defining a additionally relaxed requirement for this case.

Observation 1: No need to relax the A-IoT ACS requirements for 1 PRB.

Proposal 3: For the ACS requirements, when the number of PRBs is 1, the requirements of the NB-IoT standalone Medium Range Base Station can be reused as Table 2.

Proposal 4: For PRBs 2 to 4, it is acceptable to use the same types of interfering signal and offsets as those defined for the case with 1 PRB as Table 2.

Table 2: Adjacent channel selectivity for A-IoT

| A-IoT channel bandwidth of the lowest/highest carrier received [kHz] | Wanted signal mean power [dBm] | Interfering signal mean power [dBm] | Interfering signal centre frequency offset to the lower/upper Base Station RF Bandwidth edge or sub-block edge inside a sub-block gap [kHz] | Type of interfering signal |
| --- | --- | --- | --- | --- |
| 200 | PREFSENS + 19.5dB | -44 | ±100 | 180 kHz A-IoT signal |
| 400 | PREFSENS + TBD | -44 | ±100 | 180 kHz A-IoT signal |
| 600 | PREFSENS + TBD | -44 | ±100 | 180 kHz A-IoT signal |
| 800 | PREFSENS + TBD | -44 | ±100 | 180 kHz A-IoT signal |

# Conclusion

In this contribution, we propose A-IoT BS RF requirements for this WI as follows.

Proposal 1: The ACLR limits can be the same as those for NB-IoT as Table 1. (ACLR1: 40 dB, ACLR2: 50 dB)

Proposal 2: The 100 kHz offset for ACLR can be applied for PRBs 2 to 4 as Table 1.

Table 1: A-IoT BS ACLR limit

| BS channel bandwidth of lowest/highest carrier transmitted BWChannel (kHz) | BS adjacent channel centre frequency offset below the lowest or above the highest carrier centre frequency transmitted | Assumed adjacent channel carrier (informative) | Filter on the adjacent channel frequency and corresponding filter bandwidth | ACLR limit |
| --- | --- | --- | --- | --- |
| 200/400/600/800kHz | BWChannel + 100 kHz | A-IoT of same BW | Square (BWConfig) | 40 dB |
|  | 2 x BWChannel + 100 kHz | A-IoT of same BW | Square (BWConfig) | 50 dB |

Observation 1: No need to relax the A-IoT ACS requirements for 1 PRB.

Proposal 3: For the ACS requirements, when the number of PRBs is 1, the requirements of the NB-IoT standalone Medium Range Base Station can be reused as Table 2.

Proposal 4: For PRBs 2 to 4, it is acceptable to use the same types of interfering signal and offsets as those defined for the case with 1 PRB as Table 2.

Table 2: Adjacent channel selectivity for A-IoT

| A-IoT channel bandwidth of the lowest/highest carrier received [kHz] | Wanted signal mean power [dBm] | Interfering signal mean power [dBm] | Interfering signal centre frequency offset to the lower/upper Base Station RF Bandwidth edge or sub-block edge inside a sub-block gap [kHz] | Type of interfering signal |
| --- | --- | --- | --- | --- |
| 200 | PREFSENS + 19.5dB | -44 | ±100 | 180 kHz A-IoT signal |
| 400 | PREFSENS + TBD | -44 | ±100 | 180 kHz A-IoT signal |
| 600 | PREFSENS + TBD | -44 | ±100 | 180 kHz A-IoT signal |
| 800 | PREFSENS + TBD | -44 | ±100 | 180 kHz A-IoT signal |

# References

[1] 3GPP, RP-250796, “New Work Item: Solutions for Ambient IoT (Internet of Things) in NR”, CMCC, Huawei, T-Mobile USA Inc.

[2] 3GPP, R4-2505097, “WF on requirements for A-IoT BS and CW”, Huawei.