3GPP TSG RAN WG4 Meeting #115	R4-2507016

Saint Julian's, MT, 19 May 2025 – 23 May 2025

Agenda Item:	7.29.3.1

Source: 	Ericsson

Title:	Discussion on measurements enhancements for purpose of supporting LTM

Document for:	Discussion

# Introduction

In this contribution we discuss Event triggered measurement requirements based L1 measurement requirements.

# Discussion

## Event triggered measurement requirements

### Requirements for reporting criteria per measurement category

In previous meetings, there was a proposal to either share the capability of L3 events with LTM events or introduce a new capability. However, we believe that coordination between the CU and the DU presents challenges, as L3 measurements are typically configured by the CU, while L1 measurements are configured by the DU. This separation makes it difficult to manage the Ecat capability for LTM L1 events effectively.

Additionally, after consulting with our product implementation team, it appears that the existing Ecat capability is already fully utilized. Different applications may require configuration with distinct events, further complicating the potential for sharing capabilities between L3 and LTM events. Given these considerations, introducing a new capability might be necessary to accommodate the diverse requirements and ensure efficient operation.

- The existing Ecat capability is already fully utilized in product.

- RAN4 to agree that existing Ecat capability is not shared between L3 events and LTM L1 events.

We think RAN4 first needs to discuss and decide on the granularity of the LTM L1 events capability. Legacy Ecat was defined for frequency layer and different Ecat is defined for intra-frequency, inter-frequency and so on. Since, LTM CSI config may contain multiple cells consisting of both intra-frequency and inter-frequency measurements, we think sharing a combined Ecat between intra and inter-frequency allows the NW flexibility to configure different cells as per the channel or load conditions.

- RAN4 to support a common LTM Ecat capability to be shared between intra- and inter-frequency candidate cells.

RAN2 agreed on introducing the following events for the purpose of event triggering measurement reporting.

| Event | Description |
| --- | --- |
| Event LTM2 | Beam of serving cell becomes worse than absolute threshold; |
| Event LTM3 | Beam of candidate cell becomes amount of offset better than beam of serving cell; |
| Event LTM4 | Beam of candidate cell becomes better than absolute threshold; |
| Event LTM5 | Beam of serving cell becomes worse than absolute threshold1 AND Beam of candidate cell becomes better than another absolute threshold2. |

The events are designed to support the event evaluation on the PCell/serving cells and candidate cells. We think RAN4 should consider this when discussing the Ecat capability. Since agreeing on actual number requires further analysis, we suggest the actual number to be FFS.

- RAN4 to agree that Ecat capability for LTM L1 measurements includes both PCell event, and candidate cells’ events. The detail number of Ecat is FFS.

### Impact of event triggered to reporting to LTM procedures

Our understanding of the use case of the RAN2 introduced events is as follows:

- Event LTM2: As the name suggests this event is similar to event A2 for the event triggered L3 measurements. When the beam of the serving cell becomes worse than absolute threshold, NW may request an aperiodic report to schedule early TCI state activation or PDCCH order based RACH.

- Event LTM3: As the name suggests this event is similar to event A3 for the event triggered L3 measurements. When beam of candidate cell becomes amount of offset better than beam of serving cell, NW can trigger the cell switch based on the event triggered report from the UE.

- Event LTM4: As the name suggests this event is similar to event A4 for the event triggered L3 measurements. When beam of candidate cell becomes better than absolute threshold, based on the UE report, NW may activate the TCI states of the candidate cell or update the active TCI state list.

- Event LTM5: As the name suggests this event is similar to event A5 for the event triggered L3 measurements. When Beam of serving cell becomes worse than absolute threshold1 AND beam of candidate cell becomes better than another absolute threshold2, based on the UE report, NW may activate the TCI states of the candidate cell or update the active TCI state list or perform LTM cell switch.

Based on the above analysis we can observe that RAN2 agreed events are designed to request a candidate cell L1 measurement report to perform early DL/UL synchronization on candidate cells or trigger an LTM cell switch.

- RAN2 agreed events are designed to request a candidate cell L1 measurement report to perform early DL/UL synchronization on candidate cells or trigger an LTM cell switch.

TCI state known condition

The network may configure the UE with both L3-based events and L1-based events simultaneously. For example, the network may trigger an LTM cell switch based on Event LTM3 and schedule early TCI state activation and PDCCH order-based RACH based on L3-based events. In this case, the current TCI state known conditions need to be updated if the early TCI state activation is based on an L3 measurement report and the cell switch is based on an event-triggered L1 report. This is because the existing known conditions (at least for TCI state activation) are only based on the periodic L1-RSRP report.

Rel-18 TCI state known conditions and the conditions (clause 8.25 of TS 38.133) under which TCI state activation requirements are applicable (clause 6.3.1 of TS 38.133) are mentioned in the below table.

Known conditions of the TCI state: The candidate TCI state in the TCI state activation list is known if the following conditions are met: - During the period from the last transmission of the RS resource used for the L1-RSRP measurement reporting for the target downlink TCI state to the completion of active downlink TCI state activation, where the RS resource for L1-RSRP measurement is the RS in target downlink TCI state or QCLed to the target downlink TCI state - The MAC CE command indicating the activation of downlink TCI state is received within 1280 ms upon the last transmission of the RS resource for beam reporting or measurement - The UE has sent at least 1 valid L1-RSRP report for the target downlink TCI state before the MAC CE command is received. - The target downlink TCI state remains detectable during the downlink TCI state activation period - The SSB associated with the downlink TCI state remain detectable during the downlink TCI activation period - SNR of the downlink TCI state ≥ -3dB Otherwise, the candidate TCI state is unknown. TCI state activation requirements are applicable for: Known TCI state in FR1 and FR2 specifed in clause 8.25.2, and - Unknown TCI state in FR1 specifed in clause 8.25.2, provided the following conditions are met: - UE has reported L3 measurement result with the associated SSB index of the TCI state within 5 seconds before the LTM TCI state activation command. - SNR of the associated SSB is above -3dB.

We may need to consider a scenario where event triggered reporting is configured but the event is not triggered. In this scenario, though UE was measuring the RS, since the report was not sent, the TCI state is still considered as unknown based on the existing known condition definition. The cell switch delay requirements are not applicable in this case, as per the Release 18 agreements. To avoid no requirement in this scenario, TCI state known condition defintion needs to be modified. Potential modification is add a sentence saying TCI state is known if the “UE has sent at least one L3-RSRP report with beam index corresponding to the candidate cell on which the L1 event is configured”.

- RAN4 to revisit the definition of TCI state known condition to accommodate event triggered LTM reporting. Potential spec change is highlighted in the following:

  - The UE has sent at least 1 valid L1-RSRP report for the target downlink TCI state before the MAC CE command is received. Or UE has sent at least one L3-RSRP report with beam index corresponding to the candidate cell on which the L1 event is configured.

Another open issue discussed was listed below.

- For FR1:

  - UE has sent an event-triggered L1 report, and the reporting condition continues to be met at the time of receiving the cell switch command or TCI state activation command,

    - Case 1: leaving condition of the event is configured and corresponding condition is not met,

      - the related cell switch delay and TCI state activation requirements are applicable

    - Case 2: leaving condition of the event is NOT configured, FFS

- For FR2: FFS

For the case 2, since NW do not know if the event entering condition is still be met at the time of sending the TCI state switching or cell switch command, existing cell known, and TCI state known conditions may not be able to be enhanced.

# Summary and Conclusion

In this contribution we analyzed potential RAN4 RRM requirements impact and made following observations and proposals.

- RAN4 to agree that existing Ecat capability is not shared between L3 events and LTM L1 events.

- RAN4 to support a common LTM Ecat capability to be shared between intra- and inter-frequency candidate cells.

- RAN4 to agree that Ecat capability for LTM L1 measurements includes both PCell event, and candidate cells’ events. The detail number of Ecat is FFS.

- RAN4 to revisit the definition of TCI state known condition to accommodate event triggered LTM reporting. Potential spec change is highlighted in the following:

  - “The UE has sent at least 1 valid L1-RSRP report for the target downlink TCI state before the MAC CE command is received. Or UE has sent at least one L3-RSRP report with beam index corresponding to the candidate cell on which the L1 event is configured”.

# References

[1] 	RP-241515, New WID: NR mobility enhancements Phase 4

[2] 	R4-2420101, WF on NR mobility enhancements Phase 4