# 3GPP TSG-RAN WG4 Meeting #115	R4-2507017

# Saint Julian's, MT, 19th – 23rd May, 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.133 | CR | - | rev | - | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Draft CR on event-Triggered Reporting for Intra-Frequency L1 Measurements on Neighboring Cells |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_Mob_Ph4-Core |  |  |  |  |  | Date: |  |  | 2025-05-09 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Disussion on event triggered measurement reporting for the L1 measurements on LTM candidate cells is concluded and the agreements shall be captured in the specification. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Event-Triggered and event triggered periodic Reporting for Intra-Frequency L1 Measurements on Neighboring Cells is introduced. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The spec. is missing. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 9.14.3.4, 9.14.3.5 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  |  | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

---------------------- Start of changes ---------------------------

#### 9.14.3.4	Event Triggered Reporting

Reported L1-RSRP measurements contained in an event triggered L1-RSRP measurement report shall meet the requirements in clauses 10.1.19D for FR1 and 10.1.20A for FR2, respectively. This requirement applies for event triggered L1-RSRP reports send on PUSCH using the MAC CE.

The UE shall not send any event triggered measurement reports if no reporting criteria is fulfilled.

The measurement reporting delay is defined as the time between an event that will trigger an event triggered measurement report and the point when the UE starts to transmit the measurement report over the air interface. This measurement reporting delay excludes a delay which caused by no UL resources being available for UE to send the measurement report on.

If the event is triggered by a neighboring cell measurement, the event-triggered measurement delay shall be less than TL1-RSRP_Measurement_Period_SSB_intra as specified in Clause 9.14.5, assuming TReport = 0. Here, TSSB_NBC refers to the periodicity of the neighbor cell SSB-Index configured for intra-frequency event-triggered L1-RSRP measurement report.

If the event is triggered by the serving cell measurement, the event triggered measurement delay shall be less than TL1-RSRP_Measurement_Period_SSB as specified in 9.5.4, assuming TReport = 0. Here, TSSB refers to the periodicity of the serving cell SSB-Index configured for event triggered L1-RSRP measurement report, which is indicated by the ssb-periodicityServingCell.

#### 9.14.3.5	Event-triggered Periodic Reporting

Reported L1-RSRP measurements contained in an event triggered L1-RSRP measurement report shall meet the requirements in clauses 10.1.19D for FR1 and 10.1.20A for FR2, respectively. This requirement applies for event triggered L1-RSRP reports send on PUSCH using the MAC CE.

The first report in event triggered periodic measurement reporting shall meet the requirements specified in clause 9.14.3.4. For the subsequent reports in the event triggered periodic report, UE follows the [reportInterval and ReportAmount] as specified in TS 38.331.

--------------------End of changes--------------------------